AI Robot Control

Requirements Specification — What it must do, requirement by requirement

| Item | Value |
| --- | --- |
| Document | Document 2: Requirements Specification |
| Product | AI Robot Control (package com.falcon.robot, build falcon.ai.robot.apk) |
| Version | 1.0 |
| Date | 28 September 2026 |
| Platform | Android 8.0 (API 26) and newer; landscape tablet |
| Status | Issued for review |

# Document control

## Revision history

| Revision | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 28 September 2026 | Development | First issue, covering the application as built: sixteen spoken commands, the Bluetooth Low Energy link, on-device recognition, camera recording and the four interface languages. |

## The specification set

| # | Document | Answers |
| --- | --- | --- |
| 1 | Technical Task Specification | What is to be built, on what, and what counts as finished |
| 2 | Requirements Specification (this document) | What it must do, requirement by requirement |
| 3 | System Design Specification | How it is put together |
| 4 | Screen Design Specification | What each screen contains and where |
| 5 | Test Case Specification | How each requirement is proved |
| 6 | User Manual | How to work the application |

The six documents are separate files that share one numbering. A reference such as “Document 3, section 3.8” means that section of the System Design Specification. Identifiers are stable across the set: FR-nnn and NFR-nn for requirements, TC-nnn for test cases, and S, O, D, C, R, X for the scope, objective, deliverable, constraint, risk and outstanding item tables of Document 1.

Where the application's present behaviour differs from what a reader might assume — telemetry that is simulated, a transport that is not yet written — it is stated where it matters rather than left to be discovered.

## Contents

Right-click here and choose “Update Field” to build the table of contents.

# Document 2  Requirements Specification

Each requirement carries an identifier, the area it belongs to, a priority and the test case that verifies it. Priorities are M (mandatory: acceptance depends on it), S (should: expected, and deferrable only by agreement) and C (could: included where it costs little).

## 2.1  Definitions

| Item | Detail |
| --- | --- |
| Service | The background component that owns the camera, the microphone and the models, and outlives the screens. |
| Link | The Bluetooth Low Energy connection to the robot, held by the application. |
| Demo link | The address-and-port connection offered by the connection dialog, which has no transport behind it and reaches no robot. |
| Command | One line of text sent to the robot, such as MOVE FORWARD. |
| Action | One of the sixteen things the operator can ask for, by button or by voice. |
| Latch | A direction button that stays on, with its command repeated every second. |
| Panel | A bordered section of a screen with a title row. |

## 2.2  Functional requirements

### 2.2.1  Shell

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-001 | The application shall present eight screens — Home, Robot Control, Face Recognition, Voice Recognition, Object Detection, LiDAR SLAM, Remote Control and Settings — reachable from a sidebar present on every screen. | M | TC-001 |
| FR-002 | The sidebar shall mark the screen that is open. | M | TC-002 |
| FR-003 | The application shall run full screen, with the system bars hidden, and shall not rotate away from the layout the device reports. | S | TC-003 |
| FR-004 | A splash screen shall be shown while the first screen is prepared. | S | TC-004 |
| FR-005 | Every screen shall show the robot's connection state and shall offer the connection dialog from that indicator. | M | TC-005 |

### 2.2.2  Link

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-010 | The application shall scan for Bluetooth Low Energy devices that advertise a name, and shall list them strongest signal first. | M | TC-010 |
| FR-011 | A scan shall stop by itself after twelve seconds. | S | TC-011 |
| FR-012 | The application shall connect to a device chosen from the list, discover the serial service, enable notifications and only then report the link as connected. | M | TC-012 |
| FR-013 | The application shall request the permissions the platform version needs before scanning, and shall say so when they are refused. | M | TC-013 |
| FR-014 | Commands shall be sent as one line of UTF-8 text terminated by a newline, split to the negotiated maximum transmission unit. | M | TC-014 |
| FR-015 | The application shall write using whichever write type the robot's characteristic offers. | M | TC-015 |
| FR-016 | A write the platform refuses shall be retried when the link is next free, and shall not block the commands behind it. | M | TC-016 |
| FR-017 | Text the robot sends back shall be shown in the Remote Control log. | S | TC-017 |
| FR-018 | A device that does not offer the serial service shall be reported and disconnected. | M | TC-018 |
| FR-019 | When the link drops, every screen shall show the robot as offline, and commands shall fail rather than appear to succeed. | M | TC-019 |
| FR-020 | The link shall be held by the application, not by a screen: leaving the Remote Control screen shall not disconnect the robot. | M | TC-020 |
| FR-021 | The connection dialog's address link shall be labelled in the interface as a demonstration link that reaches no robot. | M | TC-021 |

### 2.2.3  Robot Control

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-030 | The screen shall show robot status: mode, speed, battery, temperature and estimated position. | M | TC-030 |
| FR-031 | The screen shall show a 3D figure of the robot that acts out each command as it is given. | S | TC-031 |
| FR-032 | The screen shall show the live camera with face and object results drawn over it. | M | TC-032 |
| FR-033 | The screen shall offer a choice of camera source from the camera view. | M | TC-033 |
| FR-034 | The screen shall offer camera settings — brightness, contrast, saturation, resolution and frame rate — in their own panel, scrollable by dragging. | M | TC-034 |
| FR-035 | The screen shall show the LiDAR map, at the same width as the 3D view above it, with its own full-screen control. | S | TC-035 |
| FR-036 | The LiDAR panel shall offer Load Map from its title row. | S | TC-036 |
| FR-037 | The screen shall offer a log that, when started, records face, object and speech results and the commands sent, each with the time. | M | TC-037 |
| FR-038 | A status line along the foot shall always show the most recent activity. | M | TC-038 |
| FR-039 | Robot Status, Camera and LiDAR panels shall each be able to fill the screen and be restored. | S | TC-039 |
| FR-040 | The sidebar shall carry a button for every spoken command. | M | TC-040 |
| FR-041 | Buttons shall switch face recognition, object detection and listening on and off. | M | TC-041 |
| FR-042 | A direction button shall latch on, and the command shall be repeated to the robot every second until it is stopped. | M | TC-042 |
| FR-043 | Pressing the latched direction again, pressing another direction, pressing Stop, or leaving the screen shall end the movement. | M | TC-043 |
| FR-044 | Every command sent from this screen shall be reported as delivered to the robot, sent to the demonstration link only, or not sent at all. | M | TC-044 |
| FR-045 | A spoken command shall light the same button, and drive the same latch, as the button for it. | M | TC-045 |

### 2.2.4  Camera

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-050 | The camera shall be opened once by the background service and shared by every screen that shows it. | M | TC-050 |
| FR-051 | The camera permission shall be asked for once; a refusal shall not be asked again on every screen change. | M | TC-051 |
| FR-052 | A red record button in the top right of the camera view shall start and stop recording. | M | TC-052 |
| FR-053 | Recordings shall be written to the application's own folder as MP4, named with the date and time. | M | TC-053 |
| FR-054 | Recording shall carry no sound, so that listening continues while recording. | M | TC-054 |
| FR-055 | An Open button beside the record button shall list the recordings, newest first, and hand a chosen one to a video player. | M | TC-055 |
| FR-056 | Recording shall continue while the operator is on another screen. | S | TC-056 |
| FR-057 | Where the camera cannot bind the video use case, the picture shall be kept and the recording refused with a message. | M | TC-057 |

### 2.2.5  Face

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-060 | The application shall detect faces in the camera frame and draw a box for each. | M | TC-060 |
| FR-061 | The application shall identify a detected face against the register held on the tablet and show name, identifier, similarity, department, position and time. | M | TC-061 |
| FR-062 | A face that matches nothing shall be reported as unrecognised. | M | TC-062 |
| FR-063 | The operator shall be able to enrol a face with name, identifier, department and position. | M | TC-063 |
| FR-064 | The register shall be searchable, and a record shall be removable. | S | TC-064 |
| FR-065 | The register shall be held on the device and shall survive restart. | M | TC-065 |
| FR-066 | Recognition events shall be listed with time and result. | S | TC-066 |

### 2.2.6  Objects

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-070 | The application shall detect the eighty COCO classes in the camera frame. | M | TC-070 |
| FR-071 | Boxes, labels and segmentation masks shall each be switchable. | S | TC-071 |
| FR-072 | Objects shall be tracked between frames and keep their identity. | S | TC-072 |
| FR-073 | The confidence threshold shall be adjustable. | S | TC-073 |
| FR-074 | The screen shall list what is in frame with counts, and report the inference time. | S | TC-074 |

### 2.2.7  Speech

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-080 | The application shall recognise speech on the device, with no network connection. | M | TC-080 |
| FR-081 | English and Korean shall be offered; the English model shall be used for English and the tiny-kp model for Korean. | M | TC-081 |
| FR-082 | The language chosen shall be remembered across screen changes and restarts. | M | TC-082 |
| FR-083 | The application shall recognise the sixteen commands of section 2.7 and carry each one out. | M | TC-083 |
| FR-084 | A phrase that matches no command shall be reported as Unknown Command, without showing what was heard, and nothing shall be sent. | M | TC-084 |
| FR-085 | A recognised command shall be acknowledged audibly. | M | TC-085 |
| FR-086 | Face Recognize, Object Detect and Recording shall act on the tablet rather than being sent to the robot. | M | TC-086 |
| FR-087 | Spoken switches and the buttons for them shall never disagree about which way they are set. | M | TC-087 |
| FR-088 | Listening shall continue while the operator is on another screen. | S | TC-088 |
| FR-089 | The Voice screen shall show the transcript, the matched command, the confidence and a history of what was said. | S | TC-089 |
| FR-090 | The Voice screen shall list every command with an example phrase, and running one from the list shall behave as though it had been spoken. | S | TC-090 |
| FR-091 | The operator shall be able to switch command control, continuous listening and noise suppression. | S | TC-091 |
| FR-092 | A live waveform shall be shown while listening, at ninety per cent of the panel width. | C | TC-092 |

### 2.2.8  LiDAR

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-100 | The LiDAR screen shall offer a 3D point cloud, a top view and a 2D map. | S | TC-100 |
| FR-101 | Mapping shall be startable and stoppable, and the state shown. | S | TC-101 |
| FR-102 | A map shall be savable, and a saved map loadable, from the LiDAR screen and from the Robot Control screen. | S | TC-102 |
| FR-103 | Range, resolution, mapping mode and update rate shall be settable and applied to the robot. | C | TC-103 |
| FR-104 | Pose and scan quality shall be shown. | C | TC-104 |

### 2.2.9  Remote

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-110 | The Remote Control screen shall show the robot camera, robot status with position, and the map. | S | TC-110 |
| FR-111 | The screen shall hold the Bluetooth Low Energy panel: state, scan, device list and disconnect. | M | TC-111 |
| FR-112 | Photographs, video and audio recording on the robot shall be commandable, and what has been captured listed. | C | TC-112 |
| FR-113 | An operation log shall record what happened, and be clearable. | S | TC-113 |

### 2.2.10  Settings

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-120 | Settings shall offer the connection, the interface language, the camera and an about box. | S | TC-120 |
| FR-121 | The interface language shall be selectable, and shall take effect at once. | M | TC-121 |
| FR-122 | Camera lens and analysis frame size shall be selectable and remembered. | S | TC-122 |

### 2.2.11  Service

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-130 | Face, object and speech recognition shall run in a foreground service that outlives the screens. | M | TC-130 |
| FR-131 | The service shall show a notification while it holds the camera or the microphone, and offer to stop from it. | M | TC-131 |
| FR-132 | Switch states shall be remembered across restarts. | S | TC-132 |
| FR-133 | The service shall stop by itself when nothing is switched on. | S | TC-133 |

## 2.3  Non-functional requirements

| ID | Class | Requirement | Test |
| --- | --- | --- | --- |
| NFR-01 | Performance | Object detection shall report its inference time; on the target tablet a 640 × 480 frame shall be processed in under 150 ms. | TC-140 |
| NFR-02 | Performance | The camera preview shall stay fluid while both models run. | TC-141 |
| NFR-03 | Performance | A spoken command shall be acted on within two seconds of the speaker finishing. | TC-142 |
| NFR-04 | Performance | A held direction shall be repeated to the robot every second, within ±200 ms. | TC-143 |
| NFR-05 | Availability | Recognition and recording shall survive screen changes and the screen going off. | TC-144 |
| NFR-06 | Robustness | No single refused Bluetooth write shall stop later commands. | TC-016 |
| NFR-07 | Robustness | A missing or unreadable model shall be reported and shall not stop the rest of the application. | TC-145 |
| NFR-08 | Privacy | No image, recording, transcript or face record shall leave the device. | TC-146 |
| NFR-09 | Privacy | Recordings shall be written where no other application can read them, and shared only when the operator opens one. | TC-147 |
| NFR-10 | Security | The application shall hold no credentials and shall open no listening socket. | TC-148 |
| NFR-11 | Usability | Every control shall be operable by finger or mouse; panels that hold more than fits shall scroll by dragging. | TC-149 |
| NFR-12 | Usability | The operator shall always be able to tell whether a command reached the robot. | TC-044 |
| NFR-13 | Localisation | Every visible string shall exist in English, Korean, Japanese and Chinese, with the same placeholders in each. | TC-150 |
| NFR-14 | Portability | The layout shall adapt to screen width, using stacked panels below 600 dp and side-by-side panels above 840 dp. | TC-151 |
| NFR-15 | Maintainability | Every command the application can send shall be defined in one place in the source. | TC-152 |
| NFR-16 | Maintainability | Resource, translation and reference checks shall pass before a build is offered for test. | TC-153 |
| NFR-17 | Offline | The application shall be fully functional in flight mode. | TC-146 |
| NFR-18 | Storage | Recordings shall be kept until the operator removes them; the application shall not delete them by itself. | TC-154 |

## 2.4  Interface to the robot

The robot link is a serial line over Bluetooth Low Energy. The application writes one command per line in UTF-8, terminated by a newline, and reads whatever the robot sends back as text. The service and characteristic identifiers are in Document 3, section 3.8; they are constants in one source file and are changed there if the firmware uses its own.

### 2.4.1  The sixteen spoken commands

These are what the speech recognition puts out. Each has a button in the Robot Control sidebar, and the name below is exactly the label the recogniser uses, so a keyword model's output finds its action without translation.

| Command | Sent to the robot | Effect |
| --- | --- | --- |
| Forward | MOVE FORWARD | Move ahead; repeated every second while latched |
| Backward | MOVE BACKWARD | Move back; repeated every second while latched |
| Turn Right | TURN RIGHT | Turn right; repeated every second while latched |
| Turn Left | TURN LEFT | Turn left; repeated every second while latched |
| Follow me | FOLLOW START / FOLLOW STOP | Switch: alternates on each order |
| Face Recognize | — (tablet) | Switches face recognition on the tablet |
| Object Detect | — (tablet) | Switches object detection on the tablet |
| Dance | POSE DANCE | One-off pose |
| Stop | STOP | Stops movement and releases the latch |
| Arm Left | ARM SELECT LEFT_ARM | Selects the left arm |
| Arm Right | ARM SELECT RIGHT_ARM | Selects the right arm |
| Wave | POSE WAVE | One-off pose |
| Hello | GREET | Greeting |
| T-Pose | POSE T_POSE | One-off pose |
| No | CANCEL | Cancels what was asked for; releases the latch |
| Recording | — (tablet) | Switches the tablet's camera recording |

### 2.4.2  Commands from other controls

| Command | Raised by | Meaning |
| --- | --- | --- |
| GO_HOME | Go Home button | Return to the docking position |
| SHUTDOWN | Shutdown button, after confirmation | Power the robot down |
| VOICE LISTEN ON / OFF | Listening switched on or off | Tells the robot the tablet is listening |
| CAMERA SELECT <source> | Camera chooser | Choose the robot's camera |
| CAMERA B<n> C<n> S<n> | Camera settings panel | Brightness, contrast, saturation |
| CAMERA <resolution> | Camera settings panel | Frame size |
| CAMERA PHOTO | Remote Control | Take a photograph |
| CAMERA VIDEO START / STOP | Remote Control | Robot-side video recording |
| AUDIO RECORD START / STOP | Remote Control | Robot-side audio recording |
| SLAM MAPPING START / SLAM STOP | LiDAR screen | Mapping |
| SLAM SAVE_MAP | LiDAR screen | Save the current map |
| SLAM LOAD_MAP <name> | LiDAR screen, Robot Control LiDAR panel | Load a saved map |
| SLAM AUTO_NAV ON / OFF | LiDAR screen | Autonomous navigation |
| FACE REGISTER <id> | Face screen | A face was enrolled |
| MOVE <x> <y> | Remote Control joystick | Analogue drive, one decimal each |
| MOVE STOP | Remote Control joystick released | Stop analogue drive |
| SPEED <percent> | Remote Control | Drive speed |
| MODE <name> | Remote Control | Control mode |
| ODOMETRY RESET | Remote Control | Zero the reported position |

## 2.5  Permissions

| Permission | When it is asked for | If it is refused |
| --- | --- | --- |
| CAMERA | The first time a screen shows the picture, or a camera feature is switched on | The design artwork stays in the panel; the prompt is not repeated on every screen change |
| RECORD_AUDIO | Listening is switched on | Listening stays off and says why |
| POST_NOTIFICATIONS | A background feature is switched on (Android 13+) | Recognition still runs; the notification is hidden |
| BLUETOOTH_SCAN, BLUETOOTH_CONNECT | Scan is pressed (Android 12+) | The scan does not start and the reason is logged |
| ACCESS_FINE_LOCATION | Scan is pressed (Android 11 and older) | As above; the platform requires it for scanning on those versions |
| FOREGROUND_SERVICE and its camera and microphone types | Declared, not asked | Not applicable |
| ACCESS_NETWORK_STATE | Declared, not asked | Used only to draw the Wi-Fi indicator in the status row |

## 2.6  Data held on the device

| Data | Where | Lifetime |
| --- | --- | --- |
| Face register | files/faces/faces.json (records and embeddings) | Until the operator removes a record or clears the application's data |
| Face photographs | files/faces/ | As above |
| Recordings | Android/data/com.falcon.robot/files/Movies/ | Until the operator removes them |
| Switch states, language, camera choices | SharedPreferences file “settings” | Until changed or the data is cleared |
| Custom spoken phrases | Application files | Until removed |
| Speech and vision models | Assets inside the package, unpacked on first use | For the life of the installation |

## 2.7  Languages

The interface exists in English (default), Korean, Japanese and Chinese. The Korean interface uses North Korean forms. Speech recognition offers English and Korean; the interface language and the recognition language are separate settings, because the operator may prefer to read one and speak another.

## 2.8  Traceability

Every requirement above names its test case, and Document 5 section 5.9 carries the reverse index. A requirement with no passing test case is not accepted, whatever the state of the code.

## 2.9  Use cases

| ID | Use case | Actors | Pre-condition | Main flow | Outcome | Requirements |
| --- | --- | --- | --- | --- | --- | --- |
| UC-01 | Take the robot out and drive it | Operator, robot | The tablet is on and the robot is powered | 1. Open Remote Control and press Scan. 2. Tap the robot. 3. Open Robot Control. 4. Press Forward; the robot moves and keeps moving. 5. Press Stop. | The robot stands still where it was driven to, and each command was reported as delivered | FR-010, FR-012, FR-042, FR-044 |
| UC-02 | Drive by voice | Operator, robot | The robot is connected | 1. Press Voice in the sidebar. 2. Say “forward”. 3. Hear the acknowledgement and watch the Forward button light. 4. Say “stop”. | The robot moved on the spoken order and stopped on the spoken order | FR-083, FR-045, FR-085 |
| UC-03 | Enrol a visitor and recognise them | Operator, visitor | The camera permission is granted | 1. Open Face Recognition. 2. Have the visitor face the camera. 3. Press add and fill in the details. 4. Have them step away and return. | The visitor is named, with a similarity figure, and the event is in the history | FR-060, FR-061, FR-063 |
| UC-04 | Record an inspection | Operator | The camera permission is granted | 1. Open Robot Control. 2. Press the red button. 3. Drive the robot along the route. 4. Press the red button again. 5. Press Open and play the file. | An MP4 of the route is on the tablet and plays | FR-052, FR-053, FR-055 |
| UC-05 | Watch for objects while driving | Operator | The robot is connected | 1. Press Objects in the sidebar. 2. Press Start on the Log panel. 3. Drive the robot. | Boxes follow what the camera sees and the log lists what was found, with times | FR-070, FR-037 |
| UC-06 | Load a map before a patrol | Operator, robot | The robot is connected and maps have been saved | 1. Press Load Map in the LiDAR title row. 2. Choose the map. | The robot is told to load that map and the status line says so | FR-036, FR-102 |
| UC-07 | Work in Korean | Operator | None | 1. Settings → Interface Language → Korean. 2. Voice Recognition → Advanced Settings → Language → Korean → Apply. 3. Speak a command in Korean. | The interface is in Korean and Korean speech is recognised by the tiny-kp model | FR-081, FR-121 |
| UC-08 | Find out why the robot is not moving | Operator | Commands appear to do nothing | 1. Look at the line along the foot of Robot Control. 2. Follow what it says. | The operator learns whether the command was delivered, went to the demonstration link or was not sent, and what to do about it | FR-044, FR-021 |
| UC-09 | Hand the tablet to a colleague mid-task | Two operators | Recognition is running | 1. Leave the application with the home button. 2. The colleague returns to it. | Recognition and any recording have continued; the robot is still connected | FR-130, FR-056, FR-020 |
| UC-10 | Stop everything quickly | Operator | Several features are on | 1. Open the notification shade. 2. Press Stop on the notification. | The camera and the microphone are released and the service stops | FR-131, FR-133 |

## 2.10  The command dictionary

A requirement that says “the application shall recognise Forward” is not testable until it says what counts as Forward. This section is that statement, and it is the same data the application holds: one rule per action, in the recogniser's own order.

### 2.10.1  English

English is matched by keyword, not by whole phrase, so that small recognition errors still find the action: “move forward please” and “go forwards” both reach Forward. Where two words are listed as both, the phrase has to contain one from each group — that is what separates Arm Left from Turn Left. Excluded words rule a rule out even when its keyword is present.

| Action | Any of these words | Ruled out by | Sent |
| --- | --- | --- | --- |
| Forward | forward, forwards, ahead, straight | — | MOVE FORWARD |
| Backward | back, backward, backwards, reverse | — | MOVE BACKWARD |
| Turn Right | right | arm | TURN RIGHT |
| Turn Left | left | arm | TURN LEFT |
| Follow me | follow | — | FOLLOW START / FOLLOW STOP |
| Face Recognize | face | — | carried out on the tablet |
| Object Detect | object, objects, target, detect, detection | — | carried out on the tablet |
| Dance | dance, dancing | — | POSE DANCE |
| Stop | stop, halt, freeze | record, recording | STOP |
| Arm Left | arm and left, both | — | ARM SELECT LEFT_ARM |
| Arm Right | arm and right, both | — | ARM SELECT RIGHT_ARM |
| Wave | wave, waving | — | POSE WAVE |
| Hello | hello, hi, hey | — | GREET |
| T-Pose | t, tee or tpose, with pose or tpose | — | POSE T_POSE |
| No | no, nope, cancel | — | CANCEL |
| Recording | recording, record | — | carried out on the tablet |

### 2.10.2  Chinese, Japanese and Korean

These three are matched as substrings instead. Chinese and Japanese put no spaces between words, and Korean glues its endings onto the verb, so 왼쪽으로 and 왼쪽 돌아 share 왼쪽 and little else. Every rule holds all of them at once, whatever the interface is set to, because what matters is the language the operator speaks.

| Action | Chinese | Japanese | Korean |
| --- | --- | --- | --- |
| Forward | 前进, 向前, 往前 | 前進, 前へ, 進め | 전진, 앞으로, 직진 |
| Backward | 后退, 向后, 往后 | 後退, 下がれ, バック | 후진, 뒤로, 물러 |
| Turn Right | 右转, 向右 | 右折, 右へ, 右に | 오른쪽, 우회전, 우측 |
| Turn Left | 左转, 向左 | 左折, 左へ, 左に | 왼쪽, 좌회전, 좌측 |
| Follow me | 跟我走, 跟随, 跟着我 | ついてきて, 追従, フォロー | 따라와, 날따라, 따르시오, 추종 |
| Face Recognize | 人脸识别, 识别人脸 | 顔認識, 顔の認識 | 얼굴 |
| Object Detect | 目标检测, 物体识别, 追踪目标 | 物体検出, 対象追跡 | 대상, 물체, 객체 |
| Dance | 跳舞, 跳个舞 | 踊って, ダンス | 춤, 댄스 |
| Stop | 停止, 停下, 别动 | 停止, 止まれ, ストップ | 정지, 멈춰, 스톱, 그만 |
| Arm Left | 左臂, 左手 | 左腕, 左手 | 왼팔, 왼쪽 팔, 왼손 |
| Arm Right | 右臂, 右手 | 右腕, 右手 | 오른팔, 오른쪽 팔, 오른손 |
| Wave | 挥手, 打招呼 | 手を振, 挨拶 | 손 흔들, 손흔들, 인사 |
| Hello | 你好, 您好 | こんにちは, やあ | 안녕, 반갑 |
| T-Pose | t姿势, 标准姿势 | tポーズ, ティーポーズ | 티 자세, 티자세, t 자세 |
| No | 取消, 不要 | いいえ, キャンセル, やめ | 아니, 취소, 그만두 |
| Recording | 录像, 录制 | 録画, レコーディング | 록화, 녹화, 촬영 |

The Korean phrases include the ones the tiny-kp model in the assets was trained to produce, which is what the Voice screen hears when Korean is the recognition language. Korean here is in the North Korean forms the interface uses; 록화 and 녹화 are both accepted for the same reason.

### 2.10.3  Rules the matcher follows

| No. | Rule |
| --- | --- |
| 1 | The transcript is lowercased and matched against every rule in the order above; the first rule that matches wins |
| 2 | An English rule matches when the transcript contains at least one word from every one of its groups |
| 3 | A Chinese, Japanese or Korean rule matches when the transcript contains one of its phrases anywhere |
| 4 | A rule does not match if the transcript contains one of its excluded words, in any language |
| 5 | Nothing matching leaves the result as Unknown Command, and nothing is sent |
| 6 | A match is acknowledged with the prepared sound before anything is sent |

## 2.11  Actions that are switches

Four of the sixteen are switches rather than one-off orders: saying one again undoes what it started. The switch is held on the tablet, so a button and a spoken order move the same switch, and the button shows the state whichever of them changed it.

| Action | On | Off | Where the state shows |
| --- | --- | --- | --- |
| Follow me | FOLLOW START | FOLLOW STOP | The Quick Actions button and the status line |
| Face Recognize | Face analysis is switched on in the service | Switched off | The AI group button, the Face screen switch, and the overlay |
| Object Detect | Object analysis is switched on | Switched off | The AI group button, the Object screen switch, and the overlay |
| Recording | Video recording starts on the tablet | Recording stops and the file is closed | The record button turns into a red square, and the status line says Recording |

The other twelve are momentary: each says one line and is finished. Forward, Backward, Turn Left and Turn Right are momentary as spoken orders but are latched when they are pressed as buttons, which is described in Document 3, section 3.9.

## 2.12  States the application must be able to show

Most of the ways this application can mislead an operator are states it fails to show. Each state below is required to be visible, and to be distinguishable from the ones next to it.

### 2.12.1  The link

| State | How it is shown | What a command does in it |
| --- | --- | --- |
| Not connected | Offline, grey dot, on every screen | Nothing is sent, and the screen says so |
| Scanning | Scanning, amber dot; the button reads Stop Scan | As above |
| Connecting | Connecting to <name>, amber dot | As above |
| Connected | Connected · <name>, teal dot, on every screen | The line is written to the robot and the screen says it was delivered |
| Bluetooth off | Bluetooth is off, grey dot | Nothing is sent; the scan refuses to start and says why |
| Demonstration link | Connected, from the address dialog | Nothing reaches a robot; the screen says the command went to the demonstration link only |

### 2.12.2  The camera

| State | How it is shown |
| --- | --- |
| Permission never asked | The prompt appears once, at the moment a picture is wanted |
| Permission refused | The design artwork stays in the panel, with one line saying the permission is needed; the prompt does not come back on every screen change |
| Running | The picture, with the resolution in the corner |
| Recording | The record control is a red square and the status line says Recording |
| In use by another screen | The picture follows the screen that is open; the service rebinds rather than opening a second camera |

### 2.12.3  Recognition

| State | How it is shown |
| --- | --- |
| Model loading | The AI line names the model that is loading |
| Model missing or failed | The line says which model, and what is wrong; the rest of the screen keeps working |
| Listening | The listening state and the live waveform on the Voice screen; the Voice button in the AI group is on |
| Heard and matched | The transcript, the confidence, the matched action, and the acknowledgement sound |
| Heard and not matched | Unknown Command; what was heard is not offered as a command |
| Speech engine not built | Said plainly: the optional native engine is absent from the build, and the other models are used |

## 2.13  What must happen when something is missing

These are required behaviours, not accidents of implementation. Each of them exists because the opposite behaviour — a silent failure, or a success that did not happen — has been seen in testing.

| Missing | Required behaviour | Related requirement |
| --- | --- | --- |
| The robot | Commands fail rather than appear to succeed; the screen names the command and says it was not sent | FR-019 |
| A transport behind the demonstration link | The command is logged, the screen says it reached the demonstration link only, and no claim is made about the robot | FR-019 |
| Camera permission | The screen keeps its layout and says what it needs; no repeated prompting | FR-051 |
| Microphone permission | Listening stays off and says why | Section 2.5 |
| Bluetooth permissions | The scan does not start, and the reason is in the log | FR-013 |
| Bluetooth turned off | The state says so, rather than reporting an empty scan | FR-013 |
| A speech model | The screen names the model and carries on with what it has | FR-081 |
| The optional native engine | The application says the engine was not built; nothing else is affected | FR-080 |
| A face in frame | The result panel keeps its last result and says nothing was seen | FR-060 |
| A recognised face | Unrecognised, rather than the nearest name | FR-062 |
| Storage for a recording | The recording does not start, and says why | FR-057 |
| A recording to play | The list says there is nothing yet | FR-055 |
| A map to load | Load Map says there is no map rather than opening an empty view | FR-102 |

## 2.14  Timing

These are the targets the application is designed to; they are measured in Document 5, section 5.28, on the target tablet. Where a figure depends on the device they are written as what the operator should experience rather than as a promise about any particular chip.

| What | Target | Why this figure |
| --- | --- | --- |
| A button press to a line leaving the tablet | Under 100 ms | It has to feel like the button did something |
| A held direction repeating | Every 1000 ms | Often enough for the robot to treat it as continuous, seldom enough not to flood the link |
| Stop reaching the robot | Under 200 ms from the press | Stop is the safety control; it goes to the front of the queue |
| A scan finding a device in range | Within a few seconds; the scan itself lasts 12 s | Long enough for a device advertising slowly, short enough not to drain the tablet |
| Connect to usable | Under 5 s, including service discovery and notifications | Any longer and the operator presses it again |
| Camera picture after opening a screen | Under 1 s once permission is granted | The service keeps the camera open, so this is a rebind, not a cold start |
| Face inference | Under 200 ms per frame | So the boxes track the person |
| Object inference | Under 200 ms per frame | As above |
| Speech: end of an utterance to a result | Under 2 s | Longer than this and the operator repeats themselves |
| The acknowledgement sound | Immediately on a match, before the command is sent | It is the operator's evidence that they were heard |
| Screen change | Under 500 ms | The camera and the models are not restarted |

## 2.15  Languages and text

The interface exists in four languages and the application is built so that adding a fifth is one folder of strings. What follows is what “complete” means for a language.

| Requirement | Detail |
| --- | --- |
| Every visible string is a resource | No text that an operator can read is written in code |
| The four sets are level | English, Korean, Japanese and Chinese hold the same string names; at this revision 443 strings and 22 arrays in each, plus 4 language names that are deliberately shown in their own language everywhere |
| Format specifiers match | A string with %1$s in one language has %1$s in all of them; a mismatch is a crash, not a cosmetic fault |
| Arrays match in length | A dropdown that offers five choices in English offers five in Korean, in the same order, because the code selects by position |
| Korean is North Korean in form | Throughout the interface, and in the spoken phrases |
| Nothing is left unused | A string that nothing shows is removed rather than left to be translated again |
| Interface and speech are separate | The language the operator reads and the language they speak are two settings, because they are often not the same |
| Layouts survive translation | A label that is one word in English may be three in another language; the panels are measured so that this does not push a control off the screen |

## 2.16  What is kept, and what is not

The application keeps very little, and none of it leaves the tablet.

| Kept | Not kept |
| --- | --- |
| Face records: name, identifier, department, position, a photograph and the numeric embedding | Any recording of a voice; audio is used and discarded |
| Recognition history for the current session | Any copy of the face register outside the tablet |
| Video files the operator recorded, in the application's own media folder | Any upload: there is no network transport in the application at all |
| Switch states, the two languages, camera choices | Any analytics or usage reporting |
| Custom spoken phrases the operator added | Any crash reporting to a service |

Clearing the application's data removes all of it, including the face register. There is no backup: a tablet that is replaced starts with an empty register.

## 2.17  Counts

| Group | Mandatory | Should | Could | Total |
| --- | --- | --- | --- | --- |
| Functional requirements | 50 | 28 | 4 | 82 |
| Non-functional requirements | — | — | — | 18 |
| Test cases | — | — | — | 97 |
| Spoken actions | — | — | — | 16 |
| Commands written to the robot for those actions | — | — | — | 13 |
| Actions carried out on the tablet | — | — | — | 3 |
| Commands raised by other controls | — | — | — | 19 |
| Screens | — | — | — | 8 |
| Interface languages | — | — | — | 4 |
| Speech languages | — | — | — | 2 |

The priority split is counted from the priority recorded against each functional requirement in section 2.2; the non-functional requirements carry a class rather than a priority and are all expected. Acceptance turns on the mandatory ones; the others are deferred only by agreement, and only in writing.

## 2.18  Where each action can be raised

An action is not a button and not a phrase: it is the thing both of them ask for. Each of the sixteen is reachable from at least two places, and every place must produce the same command.

| Action | Sidebar | Spoken | Voice screen list | Gamepad |
| --- | --- | --- | --- | --- |
| Forward | Yes, latched | Yes | Yes | D-pad up |
| Backward | Yes, latched | Yes | Yes | D-pad down |
| Turn Left | Yes, latched | Yes | Yes | D-pad left |
| Turn Right | Yes, latched | Yes | Yes | D-pad right |
| Stop | Yes | Yes | Yes | B |
| Arm Left | Yes | Yes | Yes | — |
| Arm Right | Yes | Yes | Yes | — |
| Wave | Yes | Yes | Yes | X |
| Hello | Yes | Yes | Yes | — |
| T-Pose | Yes | Yes | Yes | — |
| Dance | Yes | Yes | Yes | Y |
| No | Yes | Yes | Yes | — |
| Follow me | Yes, a switch | Yes | Yes | — |
| Face Recognize | Yes, a switch | Yes | Yes | — |
| Object Detect | Yes, a switch | Yes | Yes | — |
| Recording | Yes, a switch | Yes | Yes | — |

The gamepad is a convenience rather than a requirement: a pad, where one is attached, drives the robot with the same commands as the on-screen stick, and its four face buttons stand for Stop, Go Home, Wave and Dance. Go Home has a button in Quick Actions but is not one of the sixteen spoken commands.

## 2.19  What the non-functional classes mean

Section 2.3 gives each non-functional requirement a class. This is what each class is judged by, so that “the application shall be responsive” is never the whole of a requirement.

| Class | Judged by |
| --- | --- |
| Responsiveness | The figures of section 2.14, measured rather than felt |
| Robustness | The application survives the cases of Document 5, section 5.30, without crashing and without leaving the robot moving |
| Honesty | Every state the application reports can be checked against something outside the application: the robot console, the file system, the platform settings |
| Availability | Recognition and the link survive screen changes, leaving the application, and an hour of continuous use |
| Portability | The layouts hold from a phone to the target tablet, and the platform differences are handled rather than assumed away |
| Localisation | The four languages are level, by the check rather than by inspection |
| Maintainability | A command, a language or a screen can each be added by touching the files listed in Document 3, section 3.24, and no others |
| Privacy | Nothing leaves the tablet, because there is no transport by which it could |

## 2.20  Requirements the tablet cannot close on its own

These are written down separately because a test report that marked them failed against the application would be pointing at the wrong side of the link.

| Requirement | What the tablet does | What is still needed |
| --- | --- | --- |
| A command makes the robot move | Writes the agreed line and reports that it did | Firmware that answers to that wording |
| Battery, temperature, mode and speed are real | Displays whatever the robot sends, and simulates it otherwise | Telemetry from the robot |
| The position is real | Displays what it is given | Odometry from the robot |
| Following works | Sends FOLLOW START and FOLLOW STOP and holds the switch | A following behaviour on the robot |
| The map is the room | Draws what it is given, and loads what was saved | A LiDAR and a mapper on the robot |
| The robot cameras | Offers the controls and the tabs | A video transport, which this link does not have |
| The robot stops when the link drops | Stops repeating, and reports the link as down | A timeout on the robot |

# Appendix A  Glossary

| Term | Meaning |
| --- | --- |
| BLE | Bluetooth Low Energy: the radio link to the robot |
| GATT | The protocol BLE uses for services and characteristics |
| NUS | Nordic UART Service: a serial line over BLE, the shape the robot's service takes |
| MTU | Maximum transmission unit: how many bytes one BLE write carries |
| CameraX | The Android library that opens the camera and delivers frames |
| Use case | One job the camera does at a time: preview, analysis or video |
| dp | Density-independent pixel: the unit layouts are measured in |
| COCO | The eighty everyday classes the object model was trained on |
| Embedding | The vector that stands for a face, compared to recognise it |
| Keyword spotting | Recognising a fixed set of words without transcribing speech |
| Foreground service | A background component Android keeps alive, with a notification |
| Latch | A direction button that stays on, its command repeated every second |
| Demonstration link | The address connection that exercises the screens and reaches no robot |