AI Robot Control

Screen Design Specification — What each screen contains and where

| Item | Value |
| --- | --- |
| Document | Document 4: Screen Design Specification |
| Product | AI Robot Control (package com.falcon.robot, build falcon.ai.robot.apk) |
| Version | 1.0 |
| Date | 28 September 2026 |
| Platform | Android 8.0 (API 26) and newer; landscape tablet |
| Status | Issued for review |

# Document control

## Revision history

| Revision | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 28 September 2026 | Development | First issue, covering the application as built: sixteen spoken commands, the Bluetooth Low Energy link, on-device recognition, camera recording and the four interface languages. |

## The specification set

| # | Document | Answers |
| --- | --- | --- |
| 1 | Technical Task Specification | What is to be built, on what, and what counts as finished |
| 2 | Requirements Specification | What it must do, requirement by requirement |
| 3 | System Design Specification | How it is put together |
| 4 | Screen Design Specification (this document) | What each screen contains and where |
| 5 | Test Case Specification | How each requirement is proved |
| 6 | User Manual | How to work the application |

The six documents are separate files that share one numbering. A reference such as “Document 3, section 3.8” means that section of the System Design Specification. Identifiers are stable across the set: FR-nnn and NFR-nn for requirements, TC-nnn for test cases, and S, O, D, C, R, X for the scope, objective, deliverable, constraint, risk and outstanding item tables of Document 1.

Where the application's present behaviour differs from what a reader might assume — telemetry that is simulated, a transport that is not yet written — it is stated where it matters rather than left to be discovered.

## Contents

Right-click here and choose “Update Field” to build the table of contents.

# Document 4  Screen Design Specification

This document fixes what each screen contains, where it sits and how it behaves when the screen is a different size. Measurements are in density-independent pixels (dp) and are the values in the layout and dimension resources, not sketches.

## 4.1  Common rules

### 4.1.1  Frame

| Element | Rule |
| --- | --- |
| Window | Full screen; the system bars are hidden and return on a swipe |
| Navigation sidebar | Always on the left: Home, Robot Control, Face Recognition, Voice Recognition, Object Detection, LiDAR SLAM, Remote Control, Settings. 100 dp wide on a tablet, 78 dp below that; the open screen is marked |
| Page area | To the right of the sidebar, padded by 18 dp on a tablet and 12 dp otherwise |
| Panel | Rounded dark surface, 12 dp of padding, a title row and a divider |
| Panel title | 26 dp tall, 14 sp semibold, a 16 dp icon, 8 dp between icon and text |
| Gap between panels | 14 dp on a tablet, 10 dp otherwise |
| Rows of panels | Weighted by each panel's tag attribute; side by side above 840 dp, stacked below 600 dp |

### 4.1.2  Palette and type

| Role | Colour | Used for |
| --- | --- | --- |
| Background | #060A14 | The window behind everything |
| Surface | #0D172B | Panels, status bar |
| Divider | #17233A | The line under a title |
| Primary text | #E8EEF8 | Values, button labels |
| Secondary text | #8A97AE | Field names, hints |
| Muted text | #5D6A82 | Empty states |
| Blue / light blue | #2F80FF / #55A6FF | Selection, links, log titles |
| Teal | #1ED9A4 | Connected, running, good |
| Cyan | #22D3EE | Camera and AI accents |
| Amber | #FFB547 | Working, warning |
| Red | #FF4D5E | Recording, failure, shutdown |

| Text | Size |
| --- | --- |
| Panel title | 14 sp semibold |
| Field label | 12 sp secondary |
| Value | 13 sp primary |
| Sidebar button | 11 sp primary, 32 dp tall |
| Log line | 11 sp |
| Status bar | 12 sp light blue, 40 dp tall |

## 4.2  Robot Control (the main screen)

The working screen. It has no page header: the space goes to the panels. Two rows of panels, a sidebar of actions down the right, and a status line along the foot; the sidebar and the status line are outside the scrolling area, so they stay in reach wherever the page is scrolled and even with a panel full screen.

+--------------------------------------------------------------+--------+

| Robot Status            | Camera & AI                        | Actions|

| 3D figure, mode, speed  | live picture, overlays             |  AI    |

| battery, temperature    | record / open  camera chooser      |  Move  |

| position X Y Z          | one line: model, time, listening   |  Arm   |

+-------------------------+----------------+-------------------+  Pose  |

| LiDAR                   | Camera Settings| Log               |  Quick |

| map, load map, full     | brightness ... | start/stop, lines |        |

+--------------------------------------------------------------+--------+

| status line: the most recent activity, with the time                   |

+------------------------------------------------------------------------+

### 4.2.1  Panels

| Panel | Weight | Width on a 1280 × 800 dp tablet | Contents |
| --- | --- | --- | --- |
| Robot Status | 1 | 498 dp | 3D figure; mode, speed, battery, temperature; estimated position X, Y, Z; full-screen and collapse controls |
| Camera & AI | 1 | 498 dp | Live picture with face and object overlays; record and Open buttons top right; camera chooser bottom right; resolution bottom left; one status line underneath |
| LiDAR | matched | 498 dp, pinned to the width of Robot Status | Map view; Load Map and full screen in the title row; the title opens the LiDAR screen |
| Camera Settings | 1.15 | 248 dp | Brightness, contrast, saturation, resolution and frame rate; scrolls by dragging |
| Log | 1.1 | 237 dp | Start/stop button in the title row; timestamped lines |
| Actions sidebar | fixed | 115 dp (84 dp below 600 dp) | Nineteen buttons in five groups |

### 4.2.2  The actions sidebar

One button per row, 32 dp tall, icon at the left with 8 dp before it and 6 dp after, label at 11 sp. The list scrolls within the panel. Every spoken command has a button here.

| Group | Buttons | What each one does |
| --- | --- | --- |
| AI | Face, Objects, Voice | Switch face recognition, object detection and listening |
| Move Control | Forward, Backward, Turn Left, Turn Right, Stop | Latch a direction (repeated every second) or stop |
| Arm Control | Arm Left, Arm Right | Select an arm |
| Preset Pose | Wave, Hello, T-Pose, Dance | One-off poses |
| Quick Actions | Follow me, Recording, No, Go Home, Shutdown | Following switch, tablet recording, cancel, return to base, power down (confirmed) |

### 4.2.3  Camera view controls

| Control | Position | Behaviour |
| --- | --- | --- |
| Record | Top right, red disc in a white ring, 34 dp | Starts recording; becomes a red square while recording |
| Open | Top right, left of Record, 34 dp | Lists the recordings, newest first; one tap plays |
| Camera chooser | Bottom right | Front, rear or a robot camera |
| Resolution | Bottom left | The frame size in use |
| Full screen | Title row | The panel fills the page; pressed again, it returns |

### 4.2.4  Status line

Always the last thing that happened, with the time: a recognition result, or the fate of a command — delivered to the robot, sent to the demonstration link only, or not sent at all. With the log running, the same lines accumulate in the Log panel.

## 4.3  Remote Control

The screen where the robot link is made, and where the robot's own camera and recording are worked.

| Row | Panels |
| --- | --- |
| Header | Page title and subtitle, with the connection indicator |
| Top | Camera View (robot camera, source tabs, photo, switch, full screen) ¦ Robot Status (mode, speed, battery, temperature, connection, estimated position, compass) ¦ Map View (2D and 3D tabs, zoom, legend) |
| Bottom | BLE Connection ¦ a column of Camera & Recording above the Operation Log |

### 4.3.1  The BLE Connection panel

| Control | Behaviour |
| --- | --- |
| State line | Not connected / Scanning / Connecting to <name> / Connected · <name> / Bluetooth is off, with a coloured dot |
| Scan | Starts a twelve-second scan; becomes Stop Scan while it runs |
| Disconnect | Drops the link; greyed out when there is none |
| Devices Found | A row per device: name, then address and signal strength. The row for the connected device is marked. A tap connects |

## 4.4  Face Recognition

| Panel | Contents |
| --- | --- |
| Live camera | Source and resolution in the title row; picture with boxes; capture, rescan, source and full-screen tools |
| Recognition result | Photograph, name, identifier, similarity with a bar, department, position, access time; Open Door and Add to Log |
| Face database | Search, add, and a row per record with photograph, name, identifier, department and an active mark; long press removes |
| History | Recognitions with time and result |
| Statistics | Counts for the day |
| Settings | Detection switch, boxes, labels, threshold, camera |

## 4.5  Voice Recognition

| Panel | Contents |
| --- | --- |
| Listening | Microphone button, live waveform at ninety per cent of the panel width, Speak Now prompt, listening state |
| Recognition result | Transcript, language, time, confidence; the matched command with its icon and description, or Unknown Command |
| Voice commands | Every command with an example phrase and a built-in mark; a tap runs it as though it had been spoken; search |
| Voice control | Command control, continuous listening and noise suppression |
| History | What was said, when, and what came of it |
| Advanced settings | Language tab (recognition language, auto detect, punctuation), recognition tab (sensitivity, timeout, confirmation sound), audio tab (noise suppression, echo cancellation, gain, output volume), and Apply |

## 4.6  Object Detection

| Panel | Contents |
| --- | --- |
| Live camera | Picture with boxes, labels and masks; source chooser; full screen |
| Detection result | What is in frame with counts, and the inference time |
| Detection settings | Detection switch, boxes, labels, masks, tracking, confidence threshold |
| Detected objects | A row per tracked object with its class and confidence |

## 4.7  LiDAR SLAM

| Panel | Contents |
| --- | --- |
| Point cloud | 3D, top and map tabs; layers, centre, snapshot and full-screen tools |
| 2D map | Zoom in, out and fit; a legend for robot, path, target and obstacle |
| SLAM status | Mapping, localisation, navigation and map quality |
| Robot pose | X, Y, yaw and a compass |
| LiDAR | Range, points per scan and a sweep indicator |
| Control | Start and stop mapping, Save Map, Load Map, automatic navigation, follow, navigation target |
| Settings | Range, resolution, mapping mode, update rate and Apply |
| Log | Timestamped lines, clearable |

## 4.8  Home and Settings

| Screen | Contents |
| --- | --- |
| Home | A card per screen with its name and one line about it; the system status row: connection, Wi-Fi, battery and time |
| Settings | Connection (address and state, opens the connection dialog), Interface language, Camera (lens and analysis frame size), System information |

## 4.9  Behaviour when the screen is a different size

| Width | Layout |
| --- | --- |
| 840 dp and wider | Panels side by side in weighted rows; the tablet dimension set: 18 dp padding, 14 dp gaps, 100 dp navigation, 115 dp action sidebar |
| 600 dp to 840 dp | Tablet dimensions, but rows stack |
| Below 600 dp | Compact dimensions: 12 dp padding, 10 dp gaps, 78 dp navigation, 84 dp action sidebar; rows stack and the page scrolls |

## 4.10  States a screen must be able to show

| State | How it appears |
| --- | --- |
| No robot | Offline in the status row; values show a dash; a command reports that it was not sent |
| Demonstration link | Connected, with the command line saying plainly that nothing reached the robot |
| No camera permission | The design artwork stays in the panel, with a line saying why |
| Model loading | The AI line says which model is loading |
| Recording | The record button is a red square; the status line says Recording |
| Nothing recognised | Unknown Command; what was heard is not shown |
| Panel full screen | One panel fills the page; the sidebar and status line remain |

## 4.11  Control inventory: Robot Control

Every control on the main screen, by the identifier it carries in the layout. This is the list a tester or a maintainer works from.

| Identifier | Type | Behaviour |
| --- | --- | --- |
| robot_fullscreen / robot_collapse | Icon buttons, 26 dp | Fill the page with Robot Status; fold the panel to its title |
| camera_fullscreen | Icon button, 26 dp | Fill the page with the camera panel |
| camera_record | Icon button, 34 dp | Start and stop recording; red disc, red square while recording |
| camera_open | Icon button, 34 dp | List the recordings and play one |
| camera_source | Chip on the picture | Choose the camera |
| camera_info | Label on the picture | The frame size in use |
| ai_status | Line under the picture | Model, inference time, listening state |
| lidar_load_map / lidar_fullscreen | Icon buttons, 26 dp | Load a saved map; fill the page with the LiDAR panel |
| lidar_title | Panel title | Opens the LiDAR SLAM screen |
| brightness / contrast / saturation | Sliders | Sent as one camera command |
| camera_resolution / camera_fps | Dropdowns | Frame size and frame rate |
| btn_log | Button in the log title row | Start and stop the log |
| toggle_face / toggle_object / toggle_voice | Sidebar buttons | Switch the two models and listening |
| move_forward / move_backward / move_left / move_right | Sidebar buttons | Latch a direction, repeated every second |
| move_stop | Sidebar button | Stop and release the latch |
| arm_left / arm_right | Sidebar buttons | Select an arm |
| pose_wave / pose_hello / pose_tpose / pose_dance | Sidebar buttons | One-off poses |
| qa_follow | Sidebar button | Following switch, shared with the spoken order |
| qa_recording | Sidebar button | The tablet's recording, same as the red button |
| qa_no | Sidebar button | Cancel; releases the latch |
| qa_home / qa_shutdown | Sidebar buttons | Return to base; power down after confirmation |
| robot_tip | Status line | The last activity, with the time |

## 4.12  Navigation

Splash  ->  Home

|

+--> Robot Control  --(LiDAR panel title)-->  LiDAR SLAM

+--> Face Recognition

+--> Voice Recognition

+--> Object Detection

+--> LiDAR SLAM

+--> Remote Control   (the Bluetooth link is made here)

+--> Settings

The sidebar is on every screen, so any screen reaches any other in one press.

The connection dialog opens from the status indicator on every screen.

## 4.13  Panel states

| State | Entered by | Left by |
| --- | --- | --- |
| Normal | Default | — |
| Full screen | The panel's full-screen button | The same button; the sidebar and status line stay in reach |
| Collapsed | The Robot Status collapse button | The same button |
| Scrolled | Dragging inside a panel that holds more than fits | Dragging back; the page underneath never steals the drag |

## 4.14  How the inventories are to be read

Section 4.11 lists the controls of the Robot Control screen. The sections that follow do the same for the other screens. Each row is one control that has an identifier in the layout: the identifier is what the screen looks up, and the static check of Document 3, section 3.17 refuses a build in which a screen looks up something its own layout does not have. A control with no identifier — a label that never changes, a divider, a spacer — is not listed.

## 4.15  Face Recognition

Six panels: the camera down the left, the result beside it, the register below, and a row of history, statistics and settings along the foot.

### 4.15.1  Live camera

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_camera | Panel | The whole camera panel |
| feed_dot | Indicator | Whether the picture is live |
| feed_source | Label | Which camera is in use |
| feed_resolution | Label | The frame size |
| feed_fullscreen | Button | Fill the page with this panel, and return |
| feed_frame | Frame | Holds the picture and everything drawn over it |
| camera_preview | Preview | The picture itself |
| face_overlay | Overlay | Boxes and names over the picture |
| camera_message | Label | Why there is no picture, when there is none |
| tool_capture | Button | Take the frame for an enrolment |
| tool_rescan | Button | Look again |
| tool_source | Button | Change camera |
| tool_fullscreen | Button | As feed_fullscreen, from the tool strip |

### 4.15.2  Recognition result

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_result | Panel | The result panel |
| result_title | Title | Its title row |
| result_photo | Image | The photograph held for the person recognised |
| result_name | Value | Their name |
| result_id | Value | Their identifier |
| result_similarity | Value | How close the match was, as a percentage |
| result_similarity_bar | Bar | The same figure as a bar |
| result_department | Value | Their department |
| result_position | Value | Their position |
| result_access_time | Value | When they were recognised |
| btn_open_door | Button | The action a recognition is allowed to take |
| btn_add_log | Button | Put this recognition in the history |

### 4.15.3  Register, history and statistics

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_database | Panel | The face register |
| db_title | Title | Its title row |
| db_add | Button | Enrol a new face |
| db_search | Field | Filter the register |
| db_list | List | One row per record: photograph, name, identifier, department, and whether it is active; a long press removes it |
| history_title | Title | The history panel |
| btn_view_all | Button | The whole history rather than the recent part |
| history_list | List | Recognitions with time and result |
| stats_title | Title | The statistics panel |
| stats_donut | Chart | Recognised against unrecognised |
| stats_total | Value | How many faces were seen |
| stats_recognized, stats_recognized_pct | Values | How many were recognised, and the share |
| stats_unrecognized, stats_unrecognized_pct | Values | The other side of the same figure |
| today_title | Title | Today's row |
| today_recognized_icon, today_recognized | Value | Recognised today |
| today_unrecognized | Value | Unrecognised today |

### 4.15.4  Settings

| Identifier | Kind | What it is |
| --- | --- | --- |
| settings_title | Title | The settings panel |
| icon_detection, sw_face_detection | Switch | Detect faces at all |
| icon_recognition, sw_face_recognition | Switch | Identify the faces that are detected |
| spinner_mode | Dropdown | Recognition mode |
| seek_threshold, value_threshold | Slider | How close a match has to be; the readout moves with the slider |
| spinner_database | Dropdown | Which register to use |
| spinner_camera | Dropdown | Which camera to use |

## 4.16  Voice Recognition

### 4.16.1  Listening

| Identifier | Kind | What it is |
| --- | --- | --- |
| listen_panel | Panel | The listening panel |
| voice_visual | Frame | Everything that reacts to sound |
| mic_hit | Button | The microphone: press to listen, press again to stop |
| speak_hint | Label | Speak Now, while it is listening |
| live_wave | Waveform | The live level, at ninety per cent of the panel width |
| listen_badge, listen_dot, listen_state | Indicator | Listening or not, with its dot |
| mic_selector, mic_selector_icon, mic_device | Chooser | Which microphone |

### 4.16.2  Result and action

| Identifier | Kind | What it is |
| --- | --- | --- |
| result_title | Title | The result panel |
| result_quote | Frame | The transcript, set apart |
| btn_speak | Button | Listen again |
| result_text | Value | What was heard |
| result_confidence, result_confidence_bar | Value and bar | How sure the engine was |
| result_language | Value | Which language it was recognised in |
| result_time | Value | When |
| action_title | Title | The matched action |
| action_icon, action_name, action_desc | Value | Which of the sixteen it was, and what it does; Unknown Command when nothing matched |
| btn_execute | Button | Carry it out |

### 4.16.3  Commands, control and history

| Identifier | Kind | What it is |
| --- | --- | --- |
| voice_db_title | Title | The command list |
| voice_search | Field | Filter it |
| voice_db_list | List | Every command with an example phrase and a built-in mark; a tap runs it as though it had been spoken |
| control_title, voice_toggles | Switches | Command control, continuous listening, noise suppression |
| history_title | Title | The history panel |
| btn_clear_history | Button | Empty it |
| voice_history_list | List | What was said, when, and what came of it |

### 4.16.4  Advanced settings

| Identifier | Kind | What it is |
| --- | --- | --- |
| advanced_title | Title | The advanced panel |
| adv_tab_language | Tab | Recognition language, automatic detection, punctuation |
| adv_tab_hotword | Tab | Sensitivity, timeout, confirmation sound |
| adv_tab_audio | Tab | Noise suppression, echo cancellation, gain, output volume |
| adv_rows | Rows | Whichever tab's settings are showing |
| btn_apply_settings | Button | Apply them |

## 4.17  Object Detection

| Identifier | Kind | What it is |
| --- | --- | --- |
| feed_panel | Panel | The camera panel |
| camera_preview | Preview | The picture |
| tracking_overlay | Overlay | Boxes, labels and masks |
| camera_message | Label | Why there is no picture, when there is none |
| feed_fullscreen | Button | Fill the page, and return |
| feed_live | Indicator | Whether the picture is live |
| feed_source | Chooser | Which camera |
| feed_info | Label | Frame size and inference time |
| panel_result | Panel | What is in frame |
| result_list | List | One row per class with its count |
| settings_title | Title | The settings panel |
| toggle_list | Switches | Detection, boxes, labels, masks, tracking |
| spinner_threshold | Dropdown | The confidence below which a detection is ignored |
| total_count, total_trend | Values | How many objects, and which way that is going |
| count_list | List | One row per tracked object with its class and confidence |

## 4.18  LiDAR SLAM

The widest screen in the application: a column of views on the left, a column of status and control on the right.

### 4.18.1  Point cloud and map

| Identifier | Kind | What it is |
| --- | --- | --- |
| view_frame, view_3d, view_alt, view_top | Views | The 3D cloud, the alternative rendering and the top view |
| view_live | Indicator | Whether scanning is live |
| tab_3d, tab_top, tab_map | Tabs | Which of the three is showing |
| tool_layers | Button | Which layers are drawn |
| tool_center | Button | Centre on the robot |
| tool_snapshot | Button | Keep a picture of the view |
| tool_fullscreen | Button | Fill the page, and return |
| map_title | Title | The 2D map panel |
| map_zoom_in, map_zoom_out, map_fit | Buttons | Zoom, and fit the map to the panel |
| map_frame, map_2d | View | The map itself |
| map_legend | Legend | Robot, path, target and obstacle |

### 4.18.2  Status

| Identifier | Kind | What it is |
| --- | --- | --- |
| slam_title | Title | The status panel |
| status_mapping | Value | Whether a map is being built |
| status_localization | Value | Whether the robot knows where it is |
| status_navigation | Value | Whether it is navigating |
| status_quality | Value | How good the map is |
| pose_title | Title | The pose panel |
| pose_x, pose_y, pose_yaw | Values | Where it is and which way it faces |
| pose_compass | Compass | The same heading, drawn |
| lidar_title | Title | The sensor panel |
| lidar_range | Value | How far it can see |
| radar_image, radar_sweep | View | The sweep indicator |
| lidar_points | Value | Points per scan |

### 4.18.3  Control, settings and log

| Identifier | Kind | What it is |
| --- | --- | --- |
| control_title | Title | The control panel |
| btn_start_mapping, btn_stop | Buttons | Begin and end mapping |
| btn_save_map, btn_load_map | Buttons | Keep a map, and open one |
| sw_auto_nav | Switch | Automatic navigation |
| sw_follow | Switch | Following, the same switch the spoken Follow me moves |
| target_title, btn_set_target | Button | Choose a navigation target |
| slam_dpad | Direction pad | Drive from this screen |
| drive_forward, drive_backward, drive_rotate_left, drive_rotate_right | Buttons | The four directions, sending the same commands as the Robot Control sidebar |
| settings_title | Title | The settings panel |
| seek_range, value_range | Slider | Sensor range |
| spinner_resolution | Dropdown | Map resolution |
| spinner_mapping_mode | Dropdown | Mapping mode |
| spinner_update_rate | Dropdown | How often the map is updated |
| btn_apply | Button | Send the settings to the robot |
| log_title, btn_clear_log, log_scroll, log_list | Log | Timestamped lines, and a way to empty them |

## 4.19  Remote Control

### 4.19.1  Camera and status

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_camera, camera_title | Panel | The robot's camera |
| cam_front, cam_rear, cam_left, cam_right | Tabs | Which of the robot's cameras |
| feed_frame, camera_feed | View | The picture |
| tool_photo | Button | Take a photograph |
| tool_switch | Button | Change camera |
| tool_fullscreen | Button | Fill the page, and return |
| camera_info | Label | What the picture is |
| status_title, status_online | Panel | Robot status, and whether it is there |
| value_mode, value_speed, value_battery, bar_battery, value_temperature | Values | Mode, speed, battery with its bar, temperature |
| value_connection | Value | How it is connected |
| pos_x, pos_y, pos_z | Values | Where it thinks it is |
| compass | Compass | Which way it faces |

### 4.19.2  Map, connection, recording and log

| Identifier | Kind | What it is |
| --- | --- | --- |
| map_title, map_2d_tab, map_3d_tab | Tabs | The map panel and its two views |
| map_frame, map_image, map_live | View | The map, and whether it is live |
| map_zoom_in, map_zoom_out, map_locate | Buttons | Zoom, and find the robot |
| legend_robot | Legend | What the marks mean |
| ble_title | Title | The Bluetooth Connection panel |
| ble_status | Value | Not connected, Scanning, Connecting, Connected with the device name, or Bluetooth is off, each with its dot |
| ble_scan | Button | Start a twelve-second scan; it reads Stop Scan while one runs |
| ble_disconnect | Button | Drop the link; disabled when there is none |
| ble_devices_label, ble_list | List | A row per device: name, then address and signal strength; the connected one is marked; a tap connects |
| recording_title | Title | The camera and recording panel |
| rec_photo, rec_video, rec_audio | Buttons | Ask the robot for a photograph, a video or an audio recording |
| rec_gallery | Button | What has been captured |
| log_title, btn_clear_log, log_scroll, log_list | Log | The operation log: what was sent, and whatever the robot sent back |

## 4.20  Home, Splash and Settings

| Screen | Identifier | What it is |
| --- | --- | --- |
| Splash | splash_image | The opening artwork |
| Splash | splash_status | What is being prepared |
| Splash | splash_progress, splash_percent | How far along |
| Splash | splash_version | Which build this is |
| Home | hero, hero_image | The header artwork |
| Home | home_system_status | The tablet's own status row |
| Home | status_wifi, home_network | Whether the tablet has a network |
| Home | status_battery, home_battery_bar | The tablet's battery |
| Home | quick_status | The robot's state at a glance |
| Home | home_grid | A card per screen, with one line about it |
| Home | home_scroll, columns | The page, and its rows of panels |
| Settings | settings_list | The four rows: connection, interface language, camera, system information |

Settings is one list rather than a set of panels because each of its rows opens a dialog: the connection dialog, the language chooser, the camera dialog and the about box. The rows show their current value as a subtitle, so the screen answers what it is set to without being opened further.

## 4.21  Style reference

Every repeated piece of the interface is a style, so that a change to how a button looks is one edit rather than two hundred. These are the ones a layout uses by name.

| Style | Height | Text | Used for |
| --- | --- | --- | --- |
| NavRailItem | wrap | 11 sp | An entry in the navigation sidebar, icon above label |
| Panel | wrap | — | A panel on the older screens, 14 dp of padding |
| RobotPanel | wrap | — | A panel on the reworked screens, 12 dp of padding |
| RobotPanelTitle | 26 dp | 14 sp | A panel title row, icon and text, 8 dp between them |
| SectionTitle | wrap | 15 sp | A heading inside a panel |
| PanelDivider | 1 dp | — | The line under a title |
| FieldLabel | wrap | 12 sp | The name of a value |
| StatusText | wrap | 13 sp | A value with a coloured dot before it |
| Input | 44 dp | 14 sp | A single-line field |
| Dropdown | 44 dp | — | A chooser, with its own popup background |
| Segment | 40 dp | 14 sp | One of a row of tabs |
| CameraTab | 34 dp | 13 sp | A camera chooser tab |
| ActionButton | 68 dp | 12 sp | A large button with the icon above the label |
| ControlButton | 52 dp minimum | 13 sp | A button in a row of equal buttons |
| ListControlButton | 40 dp minimum | 13 sp | A button in a column, icon at the left |
| QuickButton | 46 dp minimum | 14 sp | A quick action, icon at the left |
| SidebarButton | 32 dp | 11 sp | One of the nineteen action buttons: icon at the left with 8 dp before it and 6 dp after, label clipped rather than wrapped |
| ListAction | 48 dp | 14 sp | A row that does something when tapped |
| PrimaryButton | 46 dp | 14 sp | The main button of a panel |
| SettingLabel | wrap | — | A row in Settings |

## 4.22  Dimension reference

Two complete sets. The compact set is the default, so a device that reports nothing unusual gets the tighter layout; the tablet set replaces it from 600 dp of width upwards. Nothing in the code carries a size that is in this table.

| Dimension | Compact | Tablet (600 dp and wider) | What it sets |
| --- | --- | --- | --- |
| screen_padding | 12 dp | 18 dp | Around the page |
| gap | 10 dp | 14 dp | Between panels |
| sidebar_width | 78 dp | 100 dp | The navigation sidebar |
| action_sidebar_width | 84 dp | 115 dp | The actions sidebar on Robot Control |
| title_text | 18 sp | 20 sp | A page title |
| hero_min_height | 240 dp | 300 dp | The artwork on Home |
| dpad_size | 180 dp | 200 dp | The direction pad |
| joystick_size | 150 dp | 200 dp | The analogue stick |
| mic_ring_size | 190 dp | 230 dp | The microphone ring on the Voice screen |
| preview_height | 220 dp | 340 dp | A camera panel |
| robot_preview_height | 170 dp | 230 dp | The robot's camera on Remote Control |

One more resource decides the shape of a page: the boolean two_columns, which is true from 840 dp of width. Below that, rows of panels stack instead of sitting side by side, whatever weights they carry.

## 4.23  Colour reference

Twenty-four colours, and no others. A screen that wants a colour takes one of these; a screen that needs a new one adds it here, so that the palette stays countable.

| Name | Value | Used for |
| --- | --- | --- |
| bg | #060A14 | The window behind everything |
| bg_elevated | #0B1324 | A surface that sits above the background |
| surface | #0D172B | Panels |
| surface_stroke | #1C2A45 | The edge of a panel |
| divider | #17233A | The line under a title |
| blue | #2F80FF | Selection and the main accent |
| blue_light | #55A6FF | Links, log titles |
| blue_dim | 20% blue | A blue wash behind something |
| teal | #1ED9A4 | Connected, running, good |
| teal_dim | 20% teal | The same, as a wash |
| purple | #7B5CFF | A second accent |
| purple_light | #9A82FF | Its lighter form |
| purple_dim | 20% purple | Its wash |
| amber | #FFB547 | Working, warning |
| orange | #F59E0B | A stronger warning |
| red | #FF4D5E | Recording, failure, shutdown |
| cyan | #22D3EE | Camera and AI accents |
| white | #FFFFFF | Text on a coloured button |
| white_80 | 80% white | The same, softened |
| nav_label | #B4C0D6 | A sidebar label |
| text_primary | #E8EEF8 | Values, button labels |
| text_secondary | #8A97AE | Field names, hints |
| text_muted | #5D6A82 | Empty states |
| card_subtitle | #BFDDF7 | The second line of a Home card |

## 4.24  Icons

A hundred and twenty-two drawables, nearly all vector, so that they are sharp at any density and can be tinted by state rather than duplicated. They divide as follows.

| Group | How many | Examples |
| --- | --- | --- |
| Backgrounds and shapes | 34 | bg_panel_robot, bg_control_button, bg_feed_button, bg_sidebar, bg_tab, bg_input, bg_popup, bg_splash |
| Status dots | 4 | dot_teal, dot_amber, dot_red, dot_gray |
| Icons | 74 | Everything named ic_, from ic_home to ic_record_stop; the groups below are drawn from these |
| Navigation and screens | — | ic_home, ic_robot, ic_face_id, ic_mic, ic_cube, ic_lidar, ic_gamepad, ic_settings |
| Movement and pose | — | ic_arrow_up, ic_arrow_down, ic_rotate_left, ic_rotate_right, ic_square, ic_arm, ic_pose_wave, ic_tpose, ic_run, ic_follow |
| Camera and recording | — | ic_camera, ic_videocam, ic_photo, ic_record_dot, ic_record_stop, ic_folder_open, ic_play_white |
| Map and sensing | — | ic_map, ic_place, ic_navigation, ic_layers, ic_crosshair, ic_top_view, ic_scan |
| System and status | — | ic_bluetooth, ic_wifi, ic_battery_h, ic_thermometer, ic_chip, ic_bolt, ic_power, ic_shield, ic_warning |
| Editing and lists | — | ic_add, ic_remove, ic_edit, ic_save, ic_search, ic_refresh, ic_check, ic_chevron_down, ic_chevron_right, ic_dropdown, ic_history, ic_note |
| Camera settings | — | ic_brightness, ic_contrast, ic_saturation, ic_tune, ic_volume_up |
| Legends | 3 | legend_path, legend_target, legend_obstacle |
| Progress and parts | 7 | progress_battery, progress_similarity, progress_splash, progress_loading, seekbar_thumb, switch_thumb, switch_track |

Every icon that appears on a button is placed by the shared helper that sizes and tints it, rather than by a fixed drawable size in the layout. That is why the sidebar could be reduced from a taller button to 32 dp without redrawing anything: the icon follows the size it is given.

## 4.25  Reach and touch

The screen is a tablet held in two hands or set on a stand, and the operator is sometimes watching the robot rather than the screen. The layout is measured for that.

| Rule | Measurement | Why |
| --- | --- | --- |
| The smallest control | 32 dp tall, full panel width, with 8 dp of padding before the icon | A sidebar button: below this the label stops fitting and the target gets hard to hit |
| Ordinary buttons | 40 to 52 dp tall | Comfortable without wasting a panel |
| Stop | In the Move Control group, last, where the thumb falls | It is the control that has to be found without looking |
| Record | 34 dp, top right of the picture, red on white | Far from anything that changes the camera, so it is not pressed by accident |
| Full screen | In the title row, not over the picture | So it does not cover what it is meant to enlarge |
| Shutdown | Last in Quick Actions, and confirmed | The one action that cannot be undone from the tablet |
| The status line | Along the foot, outside the scrolling area | It is where the answer to “did that work” appears, so it must not scroll away |
| The actions sidebar | Outside the scrolling area, scrolling within itself | So a command is reachable wherever the page is scrolled, and even with a panel full screen |

## 4.26  Text that changes length

A label that is one word in English is often three in another language, and Korean, Japanese and Chinese all set at different widths. Where that matters, the layout is built so that the text gives way rather than the control.

| Place | What happens to long text |
| --- | --- |
| Sidebar buttons | One line, clipped with an ellipsis; the icon and the button keep their size |
| Panel titles | One line, clipped; the title row stays 26 dp whatever it says |
| Field labels and values | The label takes what it needs and the value takes the rest, because the rows are weighted rather than fixed |
| Dropdown entries | Wrap in the popup, which is as wide as the field |
| Log lines | Wrap, and the log scrolls |
| Status line | One line, clipped; the most recent thing is always at the left where it is read first |
| Home cards | The title on one line, the description on two |
| Dialog buttons | The dialog grows; nothing is clipped, because a confirmation that cannot be read is worse than an untidy dialog |

# Appendix A  Screen layouts

| Layout | Screen it belongs to |
| --- | --- |
| activity_shell.xml | The frame every screen is drawn into, with the navigation sidebar |
| activity_splash.xml | The opening screen |
| activity_main.xml | Home |
| activity_robot_control.xml | Robot Control |
| activity_remote_control.xml | Remote Control |
| activity_face_recognition.xml | Face Recognition |
| activity_voice_recognition.xml | Voice Recognition |
| activity_object_detection.xml | Object Detection |
| activity_lidar_slam.xml | LiDAR SLAM |
| activity_settings.xml | Settings |
| view_page_header.xml, view_page_header_rich.xml | Page headers |
| view_system_status.xml | The tablet's own status row on Home |
| dialog_connection.xml | The connection dialog |
| dialog_camera_settings.xml | Camera settings from Settings |
| dialog_add_face.xml | Enrolling a face |
| item_home_card.xml, item_quick_status.xml | Home rows |
| item_settings_row.xml, item_setting_dropdown_row.xml | Settings rows |
| item_face_db_row.xml, item_face_history_row.xml | Face rows |
| item_voice_history_row.xml, item_voice_toggle_row.xml | Voice rows |
| item_slam_status_row.xml | LiDAR status rows |
| item_spinner.xml, item_spinner_dropdown.xml | Dropdown appearance |

# Appendix B  Glossary

| Term | Meaning |
| --- | --- |
| BLE | Bluetooth Low Energy: the radio link to the robot |
| GATT | The protocol BLE uses for services and characteristics |
| NUS | Nordic UART Service: a serial line over BLE, the shape the robot's service takes |
| MTU | Maximum transmission unit: how many bytes one BLE write carries |
| CameraX | The Android library that opens the camera and delivers frames |
| Use case | One job the camera does at a time: preview, analysis or video |
| dp | Density-independent pixel: the unit layouts are measured in |
| COCO | The eighty everyday classes the object model was trained on |
| Embedding | The vector that stands for a face, compared to recognise it |
| Keyword spotting | Recognising a fixed set of words without transcribing speech |
| Foreground service | A background component Android keeps alive, with a notification |
| Latch | A direction button that stays on, its command repeated every second |
| Demonstration link | The address connection that exercises the screens and reaches no robot |