AI Robot Control

User Manual — How to work the application

| Item | Value |
| --- | --- |
| Document | Document 6: User Manual |
| Product | AI Robot Control (package com.falcon.robot, build falcon.ai.robot.apk) |
| Version | 1.0 |
| Date | 28 September 2026 |
| Platform | Android 8.0 (API 26) and newer; landscape tablet |
| Status | Issued for review |

# Document control

## Revision history

| Revision | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 28 September 2026 | Development | First issue, covering the application as built: sixteen spoken commands, the Bluetooth Low Energy link, on-device recognition, camera recording and the four interface languages. |

## The specification set

| # | Document | Answers |
| --- | --- | --- |
| 1 | Technical Task Specification | What is to be built, on what, and what counts as finished |
| 2 | Requirements Specification | What it must do, requirement by requirement |
| 3 | System Design Specification | How it is put together |
| 4 | Screen Design Specification | What each screen contains and where |
| 5 | Test Case Specification | How each requirement is proved |
| 6 | User Manual (this document) | How to work the application |

The six documents are separate files that share one numbering. A reference such as “Document 3, section 3.8” means that section of the System Design Specification. Identifiers are stable across the set: FR-nnn and NFR-nn for requirements, TC-nnn for test cases, and S, O, D, C, R, X for the scope, objective, deliverable, constraint, risk and outstanding item tables of Document 1.

Where the application's present behaviour differs from what a reader might assume — telemetry that is simulated, a transport that is not yet written — it is stated where it matters rather than left to be discovered.

## Contents

Right-click here and choose “Update Field” to build the table of contents.

# Document 6  User Manual

This manual is for the person who operates the robot from the tablet. It assumes no knowledge of the software; it does assume the robot is switched on and within a few metres.

## 6.1  What you have

The tablet application AI Robot Control does three jobs. It drives the robot. It recognises faces, objects and spoken commands, all on the tablet itself. And it records what the camera sees. Nothing it does needs a network, and nothing it sees leaves the tablet.

| Screen | What it is for |
| --- | --- |
| Home | A card for each screen, and the tablet's own status |
| Robot Control | The screen to work from: the robot, the camera, the LiDAR map, the log and every action |
| Face Recognition | Enrolling people and recognising them |
| Voice Recognition | Listening, the command list and speech settings |
| Object Detection | What the camera can see, with boxes and labels |
| LiDAR SLAM | The point cloud, the map, mapping and navigation |
| Remote Control | Connecting to the robot over Bluetooth, and the robot's own camera |
| Settings | Connection, language, camera, system information |

## 6.2  First run

Switch the tablet's Bluetooth on.

Start AI Robot Control. The Home screen appears.

Open Robot Control. The tablet asks for the camera the first time; allow it, or the camera panel shows artwork instead of a picture. It will not ask again on every screen.

When you first switch listening on, the tablet asks for the microphone. On Android 13 and newer it also asks to show a notification, which is what lets recognition keep running while you are on another screen.

Connect the robot, as described next. Until you do, buttons and spoken commands are recognised but reach nothing — and the screen says so.

## 6.3  Connecting the robot

Open Remote Control.

Find the BLE Connection panel at the bottom left. If it says Bluetooth is off, switch Bluetooth on and come back.

Press Scan. Devices appear over the next few seconds, the closest first, each with its address and signal strength. The scan stops by itself after about twelve seconds.

Tap the robot. The panel shows Connecting, then Connected with the robot's name, and the log records it.

The link belongs to the application, not to that screen: go anywhere you like and the robot stays connected. Press Disconnect, or switch the robot off, to end it.

If the panel says the device does not offer the robot service, you have connected to something else — a headset or a watch. Disconnect and pick another entry.

## 6.4  The Robot Control screen

### 6.4.1  What is where

| Area | What it shows |
| --- | --- |
| Top left | The robot as a 3D figure, with mode, speed, battery, temperature and position |
| Top right | The live camera, with faces and objects marked on it |
| Bottom left | The LiDAR map |
| Bottom middle | Camera settings |
| Bottom right | The log |
| Right-hand strip | Every action the robot answers to |
| Along the foot | The last thing that happened, with the time |

### 6.4.2  Driving

Press Forward and the robot keeps going: the command is repeated every second until something stops it. To stop, press the lit button again, press Stop, press another direction, or leave the screen. There is no speed control on this screen.

### 6.4.3  Reading the line along the foot

This is the line to watch, because the 3D figure acts out every order whether or not anything reached the robot.

| What it says | What it means | What to do |
| --- | --- | --- |
| Sent to the robot: MOVE FORWARD | It was delivered over Bluetooth | Nothing |
| … the link is the demo one, so nothing reached the robot | You are connected to the address in the connection dialog, which is not a robot | Connect over Bluetooth from Remote Control |
| … not sent: no robot connected | There is no link at all | Connect over Bluetooth from Remote Control |
| … done on the tablet | Face, object or recording — the tablet did it itself | Nothing |

### 6.4.4  Recording what the camera sees

The red button at the top right of the picture starts recording; it becomes a red square while it runs. Press it again to stop, and a message names the file.

Recording has no sound, so that the robot can go on listening while it records.

The button beside it lists what you have recorded, newest first; tap one to play it.

A recording keeps running while you use other screens. It stops if you change the camera settings, which is announced.

### 6.4.5  The log

Press Start at the top of the Log panel and the screen writes down every recognition and every command, each with the time. This is the quickest way to see what the robot is being told. Press Stop to leave the lines in view without adding more.

## 6.5  Speaking to the robot

Press Voice in the right-hand strip to start listening. Say one of the commands below. You will hear a short confirmation as soon as the tablet understands, the button for that command lights, and the line along the foot says where it went.

| Say | The robot | Say | The robot |
| --- | --- | --- | --- |
| Forward | moves ahead until stopped | Arm Left | selects its left arm |
| Backward | moves back until stopped | Arm Right | selects its right arm |
| Turn Left | turns left until stopped | Wave | waves |
| Turn Right | turns right until stopped | Hello | greets you |
| Stop | stops | T-Pose | holds the T-pose |
| Follow me | follows you; say it again to stop | Dance | dances |
| No | cancels what it was asked for | Face Recognize | (tablet) face recognition on or off |
| Recording | (tablet) recording on or off | Object Detect | (tablet) object detection on or off |

If you say anything else, the screen shows Unknown Command and nothing is sent. It does not print what it thought it heard, because a wrong guess is worse than no guess.

### 6.5.1  Choosing the language you speak

Open Voice Recognition and find Advanced Settings at the bottom.

On the Language tab, choose English or Korean as the Recognition Language.

Press Apply. A message names the model being used — tiny-en for English, tiny-kp for Korean.

The choice is remembered, on this screen and everywhere else, until you change it.

The language you speak and the language the interface is written in are separate. The interface language is under Settings.

## 6.6  Face recognition

Open Face Recognition and switch detection on. A box follows each face.

To enrol someone: have them face the camera, press the add button in the Face Database panel, and fill in name, identifier, department and position.

From then on, that person is named in the result panel with a similarity figure.

Search the database by typing part of a name. Press and hold a row to remove it.

Everything stays on this tablet: the register is a file in the application's own storage.

## 6.7  Object detection

Open Object Detection and switch it on. Boxes, labels and masks can each be switched, and the confidence threshold decides how sure the tablet must be before it draws anything. The result panel lists what is in view with counts, and how long each frame took.

## 6.8  LiDAR and maps

The LiDAR SLAM screen shows the point cloud in 3D, from the top, or as a 2D map.

Start Mapping tells the robot to build a map; Stop ends it.

Save Map stores what has been built. Load Map offers what has been stored, from this screen or from the LiDAR panel on Robot Control.

Range, resolution, mapping mode and update rate are under Settings on that screen; press Apply to send them.

## 6.9  Settings

| Setting | What it does |
| --- | --- |
| Connection | The address link and its state. The robot itself is connected over Bluetooth from Remote Control |
| Interface Language | English, Korean, Japanese or Chinese; it changes at once |
| Camera | Which lens, and the frame size the models work on. A larger frame finds smaller things and is slower |
| System Information | Version and build details |

## 6.10  If something does not work

| What you see | Why | What to do |
| --- | --- | --- |
| The 3D figure moves but the robot does not | The command reached nothing: no link, or the demonstration link | Read the line along the foot; connect over Bluetooth from Remote Control |
| The camera panel shows artwork, not a picture | The camera permission was refused | Android Settings → Apps → AI Robot Control → Permissions → Camera |
| Scan finds nothing | Bluetooth is off, the permission was refused, or the robot is not advertising | Switch Bluetooth on; allow the permission when asked; power-cycle the robot |
| The robot connects and immediately disconnects | That device does not offer the robot's serial service | Connect to the right device; if it is the right one, its firmware identifiers differ from the ones the application expects |
| Listening does nothing | The microphone permission was refused, or command control is switched off | Allow the microphone; switch command control on in Voice Recognition |
| Unknown Command every time | The recognition language does not match the language being spoken | Set the Recognition Language and press Apply |
| Recording is refused | The camera cannot run the picture, the models and a recording at once | Switch one of the models off and record again |
| Speech says a model is missing | The model file is not in this build | Install a build with the model, or choose the other language |

## 6.11  Questions

| Question | Answer |
| --- | --- |
| Does anything I record or recognise leave the tablet? | No. There is no network code in the application and no internet permission in it. |
| Where are my recordings? | In the application's own folder, Android/data/com.falcon.robot/files/Movies. The Open button is the easy way to them. |
| Can I use the robot while recognition is running? | Yes. Both run at once; recognition even keeps running while you are on another screen. |
| Why does the confirmation sound like a tone rather than a voice? | Because the file supplied with the build is a placeholder. Replacing assets/ok.wav changes what you hear. |
| Why is there no speed control? | Speed is set from the Remote Control screen. The Robot Control sidebar is for directions and actions. |
| What happens if the robot goes out of range while it is moving? | The link drops, every screen shows offline, and the next command reports that it was not sent. The robot stops when its own controller stops hearing from the tablet. |

## 6.12  Quick reference

| To do this | Do this |
| --- | --- |
| Connect the robot | Remote Control → Scan → tap the robot |
| Drive | Robot Control → Forward, Backward, Turn Left, Turn Right; press again or Stop to end |
| Speak to the robot | Robot Control → Voice, then say one of the sixteen commands |
| Record the camera | Robot Control → the red button on the picture |
| Watch a recording | Robot Control → the button beside the red one |
| See what is being sent | Robot Control → Log panel → Start |
| Enrol a face | Face Recognition → add in the database panel |
| Load a map | Robot Control → LiDAR panel → the folder icon |
| Change the spoken language | Voice Recognition → Advanced Settings → Language → Apply |
| Change the interface language | Settings → Interface Language |
| Stop everything | The notification shade → Stop |

## 6.13  Notes for whoever maintains the tablet

| Task | How |
| --- | --- |
| Change the confirmation sound | Replace app/src/main/assets/ok.wav with a mono WAV and rebuild. With the file absent, the tablet speaks the word instead |
| Remove a spoken language | Delete its entry from the recognition language list in the string resources; the model can then be deleted from the assets |
| Make the package smaller | Delete the speech models that are not offered; the repository readme says what each is worth |
| Point at different robot firmware | Change the three identifiers in BleLink, and the command strings in the command table, and nothing else |
| Collect the recordings | Android/data/com.falcon.robot/files/Movies over USB, or through the Open button one at a time |
| Start again from nothing | Android Settings → Apps → AI Robot Control → Storage → Clear data. This removes the face register, the switch states and the language choices, and not the recordings |

## 6.14  Putting the application on a tablet

Someone will have done this for you. If you are doing it yourself, this is all of it.

| Step | What to do |
| --- | --- |
| 1 | Copy falcon.ai.robot.apk to the tablet, or install it over a cable |
| 2 | Open it. Android will ask whether to allow an application from this source; say yes |
| 3 | Open the application. It will ask for the camera when it first wants a picture, for the microphone when you first switch listening on, and for Bluetooth the first time you scan. Allow all three |
| 4 | On Android 13 and newer it will also ask to show notifications. Allow that too: the notification is how you stop the camera and the microphone when the application is not in front of you |
| 5 | Open Settings and set the interface language, if English is not what you want |
| 6 | Open Remote Control, press Scan, and connect to the robot |

Installing a newer version over the top keeps everything: the faces you enrolled, your languages, your camera choices and your recordings. Removing the application takes all of it with it, and there is no copy anywhere else.

## 6.15  A first session, start to finish

If you have never used it, do this once. It takes about ten minutes and it puts you through everything you will use daily.

Switch the robot on and give it a moment to start advertising.

Open the application. You will see Home: a card for each screen, and the tablet's own status along the bottom.

Open Remote Control from the sidebar. Find the Bluetooth Connection panel and press Scan.

Wait. Devices appear as they are found, the strongest signal first. Tap the robot.

Watch the state: Connecting, then Connected with the robot's name. Every screen now shows the robot as connected.

Open Robot Control. This is where you will spend your time.

Find Stop in the Move Control group and press it. Nothing will happen, because the robot is standing still — but the line along the foot should say the command was sent. That is your check that the link works.

Press Forward. The robot moves and keeps moving: the button stays on, and the command is repeated once a second until you stop it.

Press Stop. The robot stops.

Press Voice in the AI group. The tablet starts listening.

Say “forward”. You should hear the acknowledgement, see the command appear along the foot, and see the Forward button come on by itself.

Say “stop”.

Press the red button at the top right of the camera picture. It becomes a red square and the line along the foot says Recording.

Press it again, then press the Open icon beside it and play what you recorded.

Press Voice again to stop listening, and put the tablet down. The application keeps whatever you left switched on, and tells you so with a notification.

## 6.16  The Actions sidebar, button by button

Nineteen buttons down the right of the Robot Control screen, in five groups. Every one of the sixteen spoken commands has a button here, so anything you can say you can also press.

| Group | Button | What it does |
| --- | --- | --- |
| AI | Face | Look for faces in the picture, and name the ones you have enrolled |
| AI | Objects | Look for things in the picture and label them |
| AI | Voice | Listen. Press it again to stop listening |
| Move Control | Forward | Drive forward and keep going until you stop it |
| Move Control | Backward | Drive backwards, the same way |
| Move Control | Turn Left | Turn on the spot, and keep turning |
| Move Control | Turn Right | The same, the other way |
| Move Control | Stop | Stop. This is the one to reach for without looking |
| Arm Control | Left Arm | Choose the left arm for whatever comes next |
| Arm Control | Right Arm | Choose the right arm |
| Preset Pose | Wave | Wave |
| Preset Pose | Hello | Greet whoever is in front of it |
| Preset Pose | T-Pose | Stand with both arms out; useful for checking the arms move |
| Preset Pose | Dance | Dance |
| Quick Actions | Follow Me | Start following you. Press it again to stop |
| Quick Actions | Recording | Start recording what the tablet's camera sees; press again to stop |
| Quick Actions | No | Cancel whatever was asked for |
| Quick Actions | Go Home | Send the robot back to its base |
| Quick Actions | Shutdown | Power the robot down. You will be asked to confirm |

Four of them stay on once pressed: Face, Objects, Voice and Follow Me, and Recording makes five. The button shows you which way it is set, and saying the command aloud moves the same switch, so the button and your voice can never disagree.

## 6.17  What to say, in English

You do not have to say the exact words. The tablet looks for the important word, so “move forward please” works as well as “forward”. The left column is what it listens for.

| Say | Or | What happens |
| --- | --- | --- |
| Forward | forwards, ahead, straight | The robot drives forward |
| Backward | back, backwards, reverse | It drives backwards |
| Turn Right | right | It turns right |
| Turn Left | left | It turns left |
| Stop | halt, freeze | It stops |
| Arm Left | left arm | The left arm is selected |
| Arm Right | right arm | The right arm is selected |
| Wave | waving | It waves |
| Hello | hi, hey | It greets |
| T-Pose | t pose, tpose | It stands with both arms out |
| Dance | dancing | It dances |
| No | nope, cancel | Whatever was asked for is cancelled |
| Follow me | — | It starts following you; say it again to stop |
| Face Recognize | face | The tablet starts looking for faces |
| Object Detect | objects, detect, target | The tablet starts labelling what it sees |
| Recording | record | The tablet starts recording; say it again to stop |

Three things are worth knowing. Say “follow me” rather than “start following” — the second one is not recognised. Saying “stop recording” stops the recording and does not stop the robot; to stop the robot, say “stop” on its own. And “left arm” chooses an arm while “turn left” turns the robot, so the word arm is what keeps those apart.

## 6.18  What to say, in Korean

Set the language you speak on the Voice Recognition screen, under Advanced settings, or in Settings. It is a separate setting from the language the screens are written in, so you can read English and speak Korean, or the other way round.

| Say | Or | What happens |
| --- | --- | --- |
| 전진 | 앞으로, 직진 | The robot drives forward |
| 후진 | 뒤로, 물러 | It drives backwards |
| 오른쪽 | 우회전, 우측 | It turns right |
| 왼쪽 | 좌회전, 좌측 | It turns left |
| 정지 | 멈춰, 스톱, 그만 | It stops |
| 왼팔 | 왼쪽 팔, 왼손 | The left arm is selected |
| 오른팔 | 오른쪽 팔, 오른손 | The right arm is selected |
| 손 흔들 | 인사 | It waves |
| 안녕 | 반갑 | It greets |
| 티자세 | 티 자세 | It stands with both arms out |
| 춤 | 댄스 | It dances |
| 취소 | 아니, 그만두 | Whatever was asked for is cancelled |
| 따라와 | 날 따라, 추종 | It starts following; say it again to stop |
| 얼굴 | — | The tablet starts looking for faces |
| 물체 | 대상, 객체 | The tablet starts labelling what it sees |
| 록화 | 녹화, 촬영 | The tablet starts recording; say it again to stop |

Endings do not matter. 왼쪽, 왼쪽으로 and 왼쪽 돌아 all turn the robot left, because the tablet looks for the part of the word that does not change.

## 6.19  Recording, and watching what you recorded

| To do this | Do this |
| --- | --- |
| Start recording | Press the red circle at the top right of the camera picture, or say “recording” |
| See that it is recording | The red circle becomes a red square, and the line along the foot says Recording |
| Keep recording while you work | Change screens freely. Recording carries on, and the notification says so |
| Stop | Press the red square, or say “recording” again |
| Watch one back | Press the Open icon beside the record button. The newest is at the top. Tap one to play it |
| Find the files | They are in the application's own Movies folder, named with the date and time |
| Free up room | Delete them from the tablet's file manager; the application never deletes them for you |

The recordings have no sound. That is deliberate: it leaves the microphone free, so the robot still answers you while you are recording.

## 6.20  Maps

| To do this | Do this |
| --- | --- |
| See the map | It is in the LiDAR panel on Robot Control, and on the LiDAR SLAM screen in full |
| Open the LiDAR screen | Tap the LiDAR panel's title, or use the sidebar |
| Start mapping | Start Mapping, on the LiDAR screen |
| Keep a map | Save Map |
| Open a map you kept | Load Map, either on the LiDAR screen or from the corner of the LiDAR panel on Robot Control |
| Look around the map | The zoom buttons, and Fit to bring the whole map back |
| See where the robot thinks it is | The pose panel: X, Y, the heading and a compass |

## 6.21  Settings, row by row

| Row | What it does |
| --- | --- |
| Connection | Shows the address and whether you are connected, and opens the connection dialog. Connecting to a robot is normally done from Remote Control, over Bluetooth; this row is the older address connection, which is used for demonstrations |
| Interface language | English, Korean, Japanese or Chinese. The screens redraw at once. This does not change the language you speak to it in |
| Camera | Which camera to use, and the frame size the recognition works on. A smaller frame is faster and less accurate |
| System information | Which build this is. Worth reading out when reporting a fault |

## 6.22  What the messages mean

The line along the foot of the Robot Control screen, and the log beside it, tell you what actually happened. These are the ones worth recognising.

| If it says | It means | Do this |
| --- | --- | --- |
| Sent over Bluetooth | The command was written to the robot | Nothing. If the robot did not move, the fault is on the robot's side |
| Sent to the demonstration link only | You are connected to the address link, not to a robot. Nothing reached any machine | Connect over Bluetooth from Remote Control |
| Not sent — no robot connected | Nothing was sent | Connect first |
| Connected · <name> | The Bluetooth link is up and the robot can answer | Nothing |
| Connecting to <name> | The link is being made | Wait a few seconds |
| Scanning | Looking for devices | Wait; it stops by itself after twelve seconds |
| Not connected | There is no link | Scan and connect |
| Bluetooth is off | The tablet's radio is switched off | Switch Bluetooth on in the tablet's own settings |
| This device does not offer the robot service | You connected to something that is not the robot | Scan again and choose the robot |
| Camera permission is needed | The picture cannot be shown | Allow the camera in the tablet's application settings |
| Microphone permission is needed | It cannot listen | Allow the microphone the same way |
| Unknown Command | It heard you but matched nothing | Say one of the words in section 6.17 or 6.18 |
| Loading <model> | A recognition model is being made ready | Wait a moment; it is normal the first time |
| Speech engine not built | The optional extra engine is not in this build | Nothing; the ordinary models are being used |
| Recording | The tablet's camera is recording | Press the red square to stop |
| Recording could not start | Usually there is no room on the tablet | Delete some recordings |
| Unrecognised | A face was seen but is not one you enrolled | Enrol them from the Face Recognition screen |

## 6.23  The robot is not moving

Work down this list. It is in the order that finds the fault fastest.

| Ask | If no | If yes |
| --- | --- | --- |
| Does the top of the screen say connected? | Connect from Remote Control | Go on |
| Does it say Connected · with a name, rather than just connected? | You are on the demonstration link; connect over Bluetooth | Go on |
| When you press Forward, does the line along the foot say it was sent over Bluetooth? | That is a tablet-side fault; note exactly what it says and report it | Go on |
| Is the robot itself powered, and out of any fault state of its own? | Fix that first | Go on |
| Does the figure on the screen move while the robot does not? | — | The figure moves because the tablet understood you, not because the robot did anything. It proves recognition, not movement |
| Does the robot's own console show the command arriving? | It is a radio problem: range, or the robot's Bluetooth | The command arrived and the robot did nothing with it — that is a robot-side fault, and the wording it expects should be checked against Document 1, section 1.19 |

The one to remember is the fifth. The figure on the screen moves as soon as the tablet recognises a command, whether or not anything was sent and whether or not a robot is listening. It is never evidence that the robot received anything. The line along the foot is.

## 6.24  Looking after the tablet

| Matter | What to do |
| --- | --- |
| Battery | Recognition and the camera use a good deal of power. On a long session, keep it on charge |
| Heat | With faces, objects and listening all on at once the tablet gets warm and the picture may slow down. Switch off whichever of the three you are not using |
| Storage | Recordings are the only thing that grows. Nothing deletes them but you |
| The faces you enrolled | They live on this tablet and nowhere else. A tablet that is wiped or replaced starts with an empty register |
| Cleaning | Be careful of the camera lens; a smudged lens looks exactly like a bad recognition model |
| Updating | Installing a newer build over the top keeps everything |
| Reporting a fault | The build number from Settings, what you did, what the line along the foot said, and whether the robot's console showed anything |

## 6.25  Working safely

The tablet drives a machine that moves. Very little of this is about the software.

Know where Stop is before you press anything else. It is the last button in the Move Control group, and it is also the word “stop”.

A direction button keeps going. It is not a nudge: it repeats until you stop it, or until the link drops.

If the link drops while the robot is moving, the robot should stop by itself. Do not rely on that alone — stay where you can reach the robot's own power.

Keep the floor clear in front of the robot before you press Forward, and behind it before you press Backward: it cannot see behind itself.

Do not drive it out of sight. The tablet shows you what it was told, not what it hit.

Shutdown cannot be undone from the tablet. You will be asked to confirm; read the question.

When you have finished, stop the robot, then switch off listening and the camera — or press stop on the notification, which does all of it at once.

## 6.26  Things people ask

| Question | Answer |
| --- | --- |
| Can I use it without a robot? | Yes. Every screen works, recognition works, and recording works. Commands will tell you plainly that no robot received them |
| Why does it need the internet? | It does not. Recognition runs on the tablet, and there is no network connection in the application at all |
| Can I read the screens in English and speak Korean? | Yes. They are two separate settings |
| Why is there no sound on my recordings? | So that the microphone stays free and the robot still answers you while you record |
| Why did it hear me but do nothing? | Either it matched no command — it will say Unknown Command — or it matched one and could not send it. The line along the foot says which |
| Why does the camera prompt not come back after I refused it? | It is asked once on purpose. Allow it in the tablet's application settings when you want it |
| Can two people use two tablets on one robot? | Not over this link. One tablet holds the Bluetooth connection at a time |
| Where did my faces go after the tablet was replaced? | They were on the old tablet. There is no copy |
| How far away can I be? | As far as Bluetooth reaches in that room, which is usually a good deal less than it is in an empty corridor |
| It says the command was sent, but nothing happened. | Then the tablet did its part. Section 6.23 tells you how to find out which side the fault is on |

## 6.27  Enrolling someone, step by step

The tablet can only name people it has been shown. Enrolling takes about a minute a person, and it is worth doing in the light the robot will actually work in.

Open Face Recognition and make sure detection is on. A box should follow each face in the picture.

Have the person stand about a metre from the camera, facing it, with nothing over their face.

Press the add button in the Face Database panel.

Hold still while it collects. It takes several samples and averages them, which is why it is worth a few seconds of stillness.

Fill in the name, the identifier, the department and the position. The name is what appears when they are recognised.

Save. The person appears in the register with their photograph.

Check it: step out of the picture, then back in. The result panel should give their name and a similarity figure.

| If | Then |
| --- | --- |
| Nobody is boxed at all | Detection is off, or the camera permission was refused. The message in the panel says which |
| They are boxed but not named | They are not enrolled, or the threshold is set too high. The threshold is in the settings panel on that screen |
| The wrong name comes up | Two enrolments are too close. Remove one — press and hold the row — and enrol it again in better light |
| The similarity figure is low but right | Enrol them a second time in the light they are usually seen in |
| Someone leaves | Press and hold their row in the register to remove them. It is removed from the tablet and there is no copy elsewhere |

## 6.28  Object detection in use

Switch it on from the Object Detection screen, or with the Objects button in the AI group, or by saying “object detect”. Boxes and labels appear over whatever it knows: people, furniture, vehicles, everyday things.

| Control | What it is for |
| --- | --- |
| Detection | The whole thing on or off |
| Boxes | Draw the outlines |
| Labels | Name what was found, with how sure it is |
| Masks | Shade the shape as well as the box; heavier, and slower |
| Tracking | Keep the same number on a thing as it moves, rather than renumbering it each frame |
| Confidence threshold | How sure it must be before it draws anything. Raise it if the picture fills with doubtful boxes; lower it if things are being missed |

The result panel lists what is in view with counts, and the time each frame took. If that time climbs, switch off whichever of face recognition, object detection and listening you are not using: the three of them share one camera and one processor.

## 6.29  The Remote Control screen

This is where the link is made, and where the robot's own camera and recording are worked. Everything to do with connecting lives here.

| Panel | What you do with it |
| --- | --- |
| Bluetooth Connection | Scan, pick the robot, watch it connect, and disconnect when you are done |
| Camera View | The robot's own cameras, front, rear, left and right, with a photograph button and full screen |
| Robot Status | Mode, speed, battery, temperature, how it is connected, where it thinks it is, and which way it faces |
| Map View | The map in 2D or 3D, with zoom and a button to find the robot |
| Camera and Recording | Ask the robot for a photograph, a video or an audio recording, and see what has been captured |
| Operation Log | Every command sent and everything the robot sent back, with times. This is the record to read when something did not work |

If the Devices Found list stays empty: check the robot is powered and advertising, check Bluetooth is on in the tablet's own settings, and stand closer. The scan stops by itself after twelve seconds, so press Scan again rather than waiting.

## 6.30  Driving with a gamepad

If a gamepad is attached to the tablet, it drives the robot from any screen. The tablet says so when the pad is connected.

| Control | What it does |
| --- | --- |
| Left stick | Drive: the robot moves in the direction you push, and stops when the stick centres |
| Right stick | Turn |
| D-pad up | Forward |
| D-pad down | Backward |
| D-pad left | Turn left |
| D-pad right | Turn right |
| B | Stop |
| A | Go home |
| X | Wave |
| Y | Dance |

The sticks and the buttons send the same commands as the screen, so a pad is a convenience rather than a separate mode. Stop is on B, and it works from any screen.

## 6.31  Two minutes before a demonstration

Worth doing every time, in this order. It takes two minutes and it catches the things that are embarrassing in front of people.

Charge the tablet, or have it on charge.

Power the robot and give it time to advertise.

Connect, and watch for Connected with the robot's name — not merely a connected indicator.

Press Stop, and check the line along the foot says the command was sent over Bluetooth.

Press Forward, then Stop. Watch the robot, not the screen.

Switch listening on and say one command. Listen for the acknowledgement.

Set the interface language to whatever your audience reads.

Delete old recordings if you are going to record.

Decide where you will stand: within Bluetooth range, and where you can reach the robot's own power.

## 6.32  Words used in this manual

| Word | What it means here |
| --- | --- |
| The link | The Bluetooth connection between this tablet and the robot |
| Connected | The link is up and the robot can answer. The name of the robot is shown beside it |
| The demonstration link | An older address connection, used to show the screens working with no robot present. It reaches no machine |
| Latched | A direction button that stays on: the robot keeps going until you stop it |
| A switch | A button that stays on until pressed again: Face, Objects, Voice, Follow Me and Recording |
| The sidebar | Two different things: the narrow strip down the left that moves between screens, and the Actions strip down the right of Robot Control |
| The line along the foot | The status line at the bottom of Robot Control, which says what just happened |
| The log | A panel that keeps those lines with their times |
| Enrol | Teach the tablet a person's face so that it can name them |
| Unknown Command | It heard you and matched nothing you can ask for |

## 6.33  Getting it to hear you better

Most trouble with spoken commands is the room rather than the tablet. Try these in order; the first two fix nearly everything.

Stand closer. Two metres in a quiet room is comfortable; across a workshop is not.

Say the command on its own. “Forward” works better than a sentence with forward somewhere in it.

Check which language it is listening for. Speaking Korean to a tablet set to English will produce nothing, or nonsense.

Switch noise suppression on, in Advanced settings on the Voice Recognition screen.

Raise the sensitivity if it is not noticing that you have spoken, and lower it if it keeps waking up on other people.

Give it a moment after a screen change: if the model is still loading, the line says so.

Switch off face recognition and object detection while you test. Three things at once slows everything, including this.

| Panel on the Voice screen | What it is for |
| --- | --- |
| Listening | The microphone button, the live waveform, and whether it is listening |
| Recognition result | What it heard, in which language, how sure it was, and when |
| Matched action | Which of the sixteen it took that to be, and what it does. A tap on Execute runs it |
| Voice commands | Every command with an example phrase. Tapping one runs it exactly as though you had said it — the quickest way to check the robot without speaking |
| Voice control | Command control, continuous listening, and noise suppression |
| History | What was said, when, and what came of it. Useful after a session that did not go well |
| Advanced settings | Three tabs: the language and how it is detected, the sensitivity and timeout, and the audio settings. Press Apply |

The command list is worth knowing about for another reason: it is the one place that shows you every phrase the tablet will answer to, in the language you are speaking.

## 6.34  Where to go for what

An index by the thing you want to do rather than by the screen it lives on.

| To do this | Go here |
| --- | --- |
| Connect to the robot | Remote Control, Bluetooth Connection panel |
| Disconnect | The same panel, Disconnect |
| See whether you are connected | The indicator at the top of any screen |
| Drive the robot | Robot Control, Move Control group |
| Stop the robot | Robot Control, Stop — or say “stop”, or press B on a gamepad |
| Make it wave, greet, pose or dance | Robot Control, Preset Pose group |
| Choose an arm | Robot Control, Arm Control group |
| Send it home | Robot Control, Quick Actions, Go Home |
| Have it follow you | Robot Control, Quick Actions, Follow Me |
| Switch listening on | Robot Control, AI group, Voice |
| See what it heard | Voice Recognition, Recognition result |
| Change the language you speak | Voice Recognition, Advanced settings — or Settings |
| Change the language you read | Settings, Interface language |
| Record what the camera sees | The red button at the top right of the camera picture |
| Watch a recording | The Open icon beside it |
| Enrol a face | Face Recognition, Face Database panel, add |
| Remove a face | The same panel: press and hold the row |
| Label what the camera sees | Object Detection, or the Objects button in the AI group |
| Build or load a map | LiDAR SLAM, Control panel — or Load Map in the corner of the LiDAR panel on Robot Control |
| Change the camera or the frame size | Settings, Camera |
| Find the build number | Settings, System information |
| See what was sent and what came back | Remote Control, Operation Log — or the Log panel on Robot Control |
| Stop everything at once | The notification: press stop |

# Appendix A  Glossary

| Term | Meaning |
| --- | --- |
| BLE | Bluetooth Low Energy: the radio link to the robot |
| GATT | The protocol BLE uses for services and characteristics |
| NUS | Nordic UART Service: a serial line over BLE, the shape the robot's service takes |
| MTU | Maximum transmission unit: how many bytes one BLE write carries |
| CameraX | The Android library that opens the camera and delivers frames |
| Use case | One job the camera does at a time: preview, analysis or video |
| dp | Density-independent pixel: the unit layouts are measured in |
| COCO | The eighty everyday classes the object model was trained on |
| Embedding | The vector that stands for a face, compared to recognise it |
| Keyword spotting | Recognising a fixed set of words without transcribing speech |
| Foreground service | A background component Android keeps alive, with a notification |
| Latch | A direction button that stays on, its command repeated every second |
| Demonstration link | The address connection that exercises the screens and reaches no robot |