AI Robot Control

Tablet application for the Falcon service robot

| Item | Value |
| --- | --- |
| Document | Specification Set: technical task, requirements, system design, screen design, test cases and user manual |
| Product | AI Robot Control (package com.falcon.robot, build falcon.ai.robot.apk) |
| Version | 1.0 |
| Date | 28 September 2026 |
| Platform | Android 8.0 (API 26) and newer; landscape tablet |
| Status | Issued for review |

# Document control

## Revision history

| Revision | Date | Author | Change |
| --- | --- | --- | --- |
| 1.0 | 28 September 2026 | Development | First issue, covering the application as built: sixteen spoken commands, the Bluetooth Low Energy link, on-device recognition, camera recording and the four interface languages. |

## The specification set

| # | Document | Answers |
| --- | --- | --- |
| 1 | Technical Task Specification | What is to be built, on what, and what counts as finished |
| 2 | Requirements Specification | What it must do, requirement by requirement |
| 3 | System Design Specification | How it is put together |
| 4 | Screen Design Specification | What each screen contains and where |
| 5 | Test Case Specification | How each requirement is proved |
| 6 | User Manual | How to work the application |

The six documents are in one file so that a reader can follow one thread from end to end: an objective in Document 1 becomes a numbered requirement in Document 2, is realised by a component in Document 3 and a panel in Document 4, and is proved by a case in Document 5. Document 6 is written for the operator and assumes none of the other five.

Identifiers are stable: FR-nnn and NFR-nn for requirements, TC-nnn for test cases, and S, O, D, C, R, X for the scope, objective, deliverable, constraint, risk and outstanding item tables of Document 1.

Where the application's present behaviour differs from what a reader might assume — telemetry that is simulated, a transport that is not yet written — it is stated where it matters rather than left to be discovered.

## Contents

Right-click here and choose “Update Field” to build the table of contents.

# Document 1  Technical Task Specification

This document states what is to be built, why, on what, and what counts as finished. It is the terms of reference for the AI Robot Control tablet application; the detailed behaviour it calls for is written down in Document 2 (Requirements Specification), and the way that behaviour is arranged in software is Document 3 (System Design Specification).

## 1.1  Purpose of the work

The robot is an indoor service and inspection platform. It carries a Bluetooth Low Energy control board, a camera, a LiDAR head and an arm, but it has no screen and no keyboard of its own. The work covered here is the Android tablet application that the operator uses instead: one screen from which the robot is watched and driven, with the recognition work (faces, objects, speech) done on the tablet rather than on the robot or in a data centre.

Three properties decide most of the design and are treated as given throughout this set of documents:

The application must work with no network of any kind. Every model it needs is inside the installed package; nothing is fetched, uploaded or reported anywhere.

The tablet is the operator's whole interface. The screen layout is therefore part of the specification, not a matter of taste, and it is written down in Document 4.

The link to the robot is Bluetooth Low Energy, made and held by the application, and shared by every screen in it.

## 1.2  Scope

### 1.2.1  In scope

| # | Item | Summary |
| --- | --- | --- |
| S-01 | Tablet application | An Android application, package com.falcon.robot, built as falcon.ai.robot.apk, running on Android 8.0 (API 26) and newer, laid out for a landscape tablet. |
| S-02 | Robot link | Discovery of, connection to and command traffic with the robot over Bluetooth Low Energy, using a serial-over-BLE service. |
| S-03 | Robot Control screen | The main screen: robot state, live camera with recognition overlays, LiDAR view, camera settings, a log, and a sidebar of every action the robot answers to. |
| S-04 | Face recognition | On-device face detection, enrolment and identification against a register held on the tablet. |
| S-05 | Object detection | On-device detection, segmentation and tracking of the eighty COCO classes. |
| S-06 | Speech recognition | On-device speech recognition in English and Korean, and a fixed vocabulary of sixteen spoken commands. |
| S-07 | LiDAR and SLAM screen | Point cloud and map views, mapping and navigation controls, and saved maps. |
| S-08 | Video recording | Recording of the camera view to the tablet, and playback of what has been recorded. |
| S-09 | Localisation | Complete English, Korean, Japanese and Chinese interface. |
| S-10 | Background operation | Recognition and recording continue while the operator moves between screens, and while the screen is off. |

### 1.2.2  Out of scope

Firmware on the robot. The application sends commands and reads back whatever the robot chooses to say; what the robot does with a command is the firmware's business.

Any server, account, cloud storage or remote telemetry. There is none, by requirement.

Training of the recognition models. The models are supplied as assets; the scripts under tools/ are for whoever retrains them and are not part of the shipped product.

Phone-sized layouts beyond graceful degradation. The application runs on a phone, but the design target is a ten-inch landscape tablet.

Hardware selection, mechanical design and robot-side sensors.

## 1.3  Operating environment

| Item | Detail |
| --- | --- |
| Device class | Android tablet, 10 inch, landscape, 1280 × 800 dp or larger |
| Operating system | Android 8.0 (API 26) minimum, Android 14 (API 34) target |
| Processor | 64-bit ARM (arm64-v8a); x86_64 is built for the emulator |
| Memory | 3 GB RAM or more recommended, because two neural models may be resident |
| Storage | Approximately 600 MB for the installed application with all models, plus room for recordings |
| Radio | Bluetooth 4.0 or newer with Low Energy support |
| Sensors used | Front and rear camera, microphone, speaker |
| Network | None required, and none used |

## 1.4  Objectives and success criteria

| # | Objective | How it is judged |
| --- | --- | --- |
| O-01 | One screen to work from | Robot state, camera, LiDAR, recognition results and every action are reachable on the Robot Control screen without scrolling past the fold on a ten-inch tablet. |
| O-02 | Spoken control of the robot | Each of the sixteen commands, spoken in a quiet room, is recognised and reaches the robot; the screen states plainly whether it did. |
| O-03 | Recognition without a network | Faces, objects and speech are recognised with the device in flight mode. |
| O-04 | A link that can be trusted | Connection state, and the fate of each command, is visible at all times; a dropped link is reported rather than silently absorbed. |
| O-05 | Operable in four languages | Every string the operator can see exists in English, Korean, Japanese and Chinese. |
| O-06 | Recognition survives navigation | Switching screens does not stop face, object or speech recognition, or a recording. |

## 1.5  Deliverables

| # | Deliverable | Form |
| --- | --- | --- |
| D-01 | Application source | Android Gradle project, Java 8, in the repository |
| D-02 | Installable package | falcon.ai.robot.apk |
| D-03 | Model assets | Face embedding, object detection and speech models under app/src/main/assets |
| D-04 | Specification set | This document and the five that follow it |
| D-05 | Model tooling | tools/train_commands.py and tools/train_moonshine.py, with the notes in the repository readme |
| D-06 | Test record | The test cases of Document 5, with results recorded against the build under test |

## 1.6  Constraints

| # | Constraint | Consequence for the work |
| --- | --- | --- |
| C-01 | No network at run time | Models ship inside the package, which sets the package size; speech models are chosen per language rather than one multilingual model. |
| C-02 | Minimum API level 26 | Fixed by the speech library, which declares it. Everything else in the application runs on API 21. |
| C-03 | Framework user interface only | No AppCompat or Material dependency: screens are built from framework widgets, styles and the project's own views. |
| C-04 | Java 8 language level | No newer language features; lambdas and method references are used, records and var are not. |
| C-05 | Single camera owner | The camera is opened once, by the background service, and shared; no screen may open it for itself. |
| C-06 | Microphone belongs to speech | Video recording is silent, so that recording and listening can run at the same time. |
| C-07 | Speech model licences | The English model is MIT licensed; the Korean models are under a non-commercial community licence, which has to be cleared before the product is sold. |

## 1.7  Assumptions

The robot advertises a serial-over-BLE service and accepts one text command per line. The application uses the Nordic UART Service identifiers, which are given in full in Document 3, section 3.8.

The robot's own controller carries out a movement until it is told to stop, or until the command is repeated; the application repeats a held direction once a second so that either behaviour produces continuous movement.

One operator uses one tablet with one robot at a time.

Room lighting is adequate for the camera; recognition accuracy in darkness is not part of the acceptance criteria.

## 1.8  Risks

| # | Risk | Effect | What is done about it |
| --- | --- | --- | --- |
| R-01 | Robot firmware uses a different command vocabulary | Commands are delivered but ignored | Every command the application can send is listed in one table (Document 2, section 2.7) and defined in one place in the source; the screen reports delivery, so the two failures can be told apart. |
| R-02 | Camera cannot bind preview, analysis and video together | Recording refused on some devices | The picture and the models are kept and the recording is refused with a message, rather than the screen going black. |
| R-03 | Speech in a noisy room | Commands missed or mistaken | Noise suppression, a confirmation sound on a match, and a fixed vocabulary rather than free dictation; an unrecognised phrase is reported as Unknown Command and nothing is sent. |
| R-04 | Korean model licence not cleared | Korean cannot ship commercially | The language list is data: removing one entry removes the language, and the English model is unaffected. |
| R-05 | Package size | Slow installation, storage pressure | Models that are not selected can be deleted from the assets folder; the readme says which and how much each saves. |

## 1.9  Acceptance

The work is accepted when every requirement marked Mandatory in Document 2 has a passing test case in Document 5, executed against the build under test on the target tablet with the robot connected over Bluetooth Low Energy, and when the outstanding items in section 1.10 have either been closed or accepted in writing as they stand.

## 1.10  Outstanding items at this revision

These are stated plainly because they affect what a tester will see. None of them is a defect; each is work that has not been commissioned, or a decision that rests with the robot side.

| # | Item | State |
| --- | --- | --- |
| X-01 | Wi-Fi transport | The connection dialog offers an address and port. There is no socket behind it: connecting that way exercises the screens only. Every screen labels such a link as the demo link. Bluetooth Low Energy is the real transport. |
| X-02 | Robot telemetry | Battery, temperature, speed and position are placeholder values. When the robot reports them, the fields are already in place to receive them. |
| X-03 | Keyword model | The application can use a small keyword-spotting model (commands.tflite) in place of full transcription. No such model is in the assets, so the speech models are used. |
| X-04 | Acknowledgement sound | assets/ok.wav is a placeholder tone, not a recorded voice. Replacing the file changes what is heard. |
| X-05 | Korean command file | The Korean speech model ships with a commands.json describing what it was trained on. The application matches Korean by phrase and does not read that file. |
| X-06 | LiDAR data | The point cloud, the map and the pose are generated on the tablet for demonstration. The controls send real commands. |

## 1.11  Applicable documents

| Reference | Title |
| --- | --- |
| DOC-1 | Technical Task Specification (this document) |
| DOC-2 | Requirements Specification |
| DOC-3 | System Design Specification |
| DOC-4 | Screen Design Specification |
| DOC-5 | Test Case Specification |
| DOC-6 | User Manual |
| EXT-1 | Nordic UART Service, characteristic identifiers as used by the robot link |
| EXT-2 | Android foreground service types: camera and microphone |
| EXT-3 | Repository readme: model preparation, licences and build prerequisites |

## 1.12  Work breakdown

| Phase | Work | Finished when |
| --- | --- | --- |
| P1 Shell | Navigation, page frame, styles, four languages, screen skeletons | Every screen opens and is laid out on a tablet and a phone |
| P2 Service | The foreground service, camera ownership, the listener interface | The camera is opened once and shared; recognition survives a screen change |
| P3 Vision | Face detection, embedding and the register; object detection, segmentation and tracking | A person is enrolled and recognised; the eighty classes are detected |
| P4 Speech | Recorder, models, the command vocabulary, the acknowledgement | All sixteen commands are recognised with no network |
| P5 Link | Bluetooth scanning, connection, the write queue, session state | A command reaches the robot and the screen reports it |
| P6 Control screen | Panels, the action sidebar, the movement latch, the log | The screen passes its part of Document 5 |
| P7 Recording | Video use case, files, playback, the provider grant | A recording is made, kept and played back |
| P8 Hardening | Diagnostics, honesty about delivery, static checks, documents | The acceptance criteria of section 1.9 are met |

## 1.13  How each kind of requirement is verified

| Method | Applied to | Evidence |
| --- | --- | --- |
| Demonstration | Screen behaviour, recognition, driving | A test case in Document 5, run on the target tablet with the robot present |
| Inspection | Layout measurements, palette, resource completeness | The layout and dimension resources, read against Document 4 |
| Static analysis | Resource references, translations, unused strings, balance | The check of Document 3, section 3.17, run before a build is offered |
| Measurement | Inference time, repeat interval, reaction time | The figures the application itself reports, and a stopwatch |
| Analysis | Privacy and security claims | The manifest and a search of the source for network use |

## 1.14  Roles

The work touches three parties: whoever operates the tablet, whoever writes the application, and whoever writes the software on the robot. Much of what goes wrong in a job of this shape goes wrong on the seam between the second and the third, so the obligations of each are written down rather than assumed.

| Role | Responsible for | Depends on |
| --- | --- | --- |
| Operator | Working the tablet: connecting to the robot, driving it, speaking to it, enrolling faces, keeping the tablet charged | A build that is installed and a robot that is advertising |
| Application development | The tablet application described by this set: screens, link, recognition, recording, translations, and the static check that guards them | The command vocabulary being agreed and stable |
| Robot firmware | Advertising the serial service, accepting the command lines, acting on them, and sending back whatever telemetry it has | The same agreed vocabulary |
| Integration | Putting tablet and robot in the same room and running the field part of Document 5 | Both of the above being ready at once |
| Test | Running the cases of Document 5 against a named build and recording what was seen | A build and, for the link cases, a robot |
| Acceptance | Deciding whether the objectives of section 1.4 are met | A complete test record |

## 1.15  Development standards

These are the rules the existing source already follows. They are written down so that later work does not drift away from them, and so that a reviewer has something to measure against.

### 1.15.1  Platform and language

| Rule | Detail |
| --- | --- |
| Language level | Java 8. Lambdas and method references are used; newer language features are not |
| Minimum platform | API 26 (Android 8.0). Anything newer is reached through a version check, never assumed |
| Target and compile platform | API 34 |
| User interface toolkit | The framework widgets only. AppCompat, Material Components and view binding are not used, so a screen is a layout file and findViewById |
| Third-party libraries | CameraX, TensorFlow Lite, the Moonshine voice package and ONNX Runtime. A new dependency needs a reason that cannot be met with what is already there |
| Native code | Only the optional speech fallback, built with CMake, and the application must run without it |
| Package | com.falcon.robot; the build output is falcon.ai.robot.apk |

### 1.15.2  Structure

| Rule | Detail |
| --- | --- |
| One screen, one activity | Each screen is an Activity extending BaseActivity, with one layout file named after it |
| Shared behaviour goes to the base | Anything two screens both need — the sidebar, the connection indicator, the column weights, the log — belongs in BaseActivity, not copied |
| Long-lived work goes to the service | The camera, the microphone, the models and the recorder are owned by RobotService and outlive any screen |
| One place for the link | Every command leaves through RobotSession.send; no screen talks to Bluetooth itself |
| One place for the commands | The sixteen actions, their phrases and their command strings are in VoiceCommands and nowhere else |
| No text in code | Anything the operator can read is a string resource, in all four languages |
| No sizes in code | Padding, widths and heights are dimension resources, so that the tablet set and the compact set both work |
| No colour in code | Except where a value is chosen by state, colours are resources |

### 1.15.3  Habits the review looks for

A callback that touches a view is posted to the main thread by whoever raises it, not by each listener in turn.

A listener added when a screen opens is removed when it closes.

A camera use case is bound and unbound through the service's one binding routine, never directly.

A permission is asked for at the moment it is needed, and refusing it leaves a screen that still works and says why.

A failure is reported where the operator is looking, not only in the log.

A string added to one language is added to all four in the same change.

An identifier a screen uses is reachable from that screen's own layout.

A file that is opened is closed, and a recording that is started can be stopped from somewhere the operator can reach.

## 1.16  Work packages

The work divides so that each package can be finished and shown on its own. The order below is the order the dependencies allow; where two packages do not depend on each other they can be taken in either order.

| WP | Package | Contains | Needs first |
| --- | --- | --- | --- |
| WP-1 | Shell and navigation | The frame, the sidebar, the eight screens as pages, the splash, the theme, both dimension sets | — |
| WP-2 | Background service | RobotService, its binder, its listener list, the notification, the camera binding rules | WP-1 |
| WP-3 | Camera | Preview, analysis, the lens choice, the analysis size, the permission behaviour, the artwork fallback | WP-2 |
| WP-4 | Link | RobotSession, BleLink, scanning, connecting, the write queue, the notification path, the state broadcast | WP-1 |
| WP-5 | Robot Control | The panels, the actions sidebar, the latch, the status line, the log | WP-2, WP-4 |
| WP-6 | Speech | Model loading for each language, the listener, the matcher, the sixteen actions, the acknowledgement sound | WP-2 |
| WP-7 | Face recognition | Detector, embeddings, the register, enrolment, the recognition screen | WP-3 |
| WP-8 | Object detection | The detector, the tracker, the overlay, the detection screen | WP-3 |
| WP-9 | Recording | The video use case, start and stop, the file list, playback through the file provider | WP-3 |
| WP-10 | LiDAR and maps | The point cloud and map views, the status panels, Load Map | WP-1 |
| WP-11 | Remote Control | The robot camera panel, the Bluetooth connection panel, the recording column, the operation log | WP-4 |
| WP-12 | Settings | The rows, the connection dialog, the interface language, the camera dialog, the about box | WP-1 |
| WP-13 | Translations | Korean, Japanese and Chinese for every string, and the parity check that keeps them level | All of the above |
| WP-14 | Static check | The script that reads the resources and the sources and refuses a build that has lost an identifier | WP-1 |
| WP-15 | Field integration | Running the link and command cases against the real robot, and correcting whichever side is wrong | WP-5, WP-6, firmware |

## 1.17  Stages

Dates are agreed separately; what is fixed here is the order, and what has to be true before a stage can be called finished.

| Stage | Finished when |
| --- | --- |
| Frame | Every screen opens from the sidebar, the theme and both dimension sets are in place, and no page is blank |
| Sight and sound | The camera picture appears on the screens that use it, the microphone is heard, and both survive leaving a screen and coming back |
| Recognition | Faces, objects and speech each produce a result on their own screen, with the model in use named |
| Link | A real robot is found by a scan, connected to, and shown as connected on every screen |
| Commands | All sixteen actions leave the tablet in the agreed wording, by button and by voice, and the screen says which of them reached the robot |
| Languages | All four interface languages are complete and level, and speech works in English and Korean |
| Field | The robot moves on command, in a room, repeatedly, and whatever still fails is written down |
| Acceptance | Document 5 has been run end to end against one named build, with the mandatory cases passing |

## 1.18  What the robot side has to provide

The application cannot be finished against a robot that is not there, and it cannot be accepted against one whose vocabulary is different. These are the obligations the other side carries.

| No. | Obligation | Why it matters |
| --- | --- | --- |
| RB-1 | Advertise over Bluetooth Low Energy with a readable name | The scan lists devices by name; an unnamed advertisement is ignored |
| RB-2 | Offer a serial service with a write characteristic and a notify characteristic | This is the whole transport; without it the application reports the device as unsuitable and disconnects |
| RB-3 | Use the identifiers of Document 3, section 3.8, or say which ones it does use | They are constants in one file and can be changed, but only if they are known |
| RB-4 | Accept a command as one line of UTF-8 text ending in a newline | That is exactly what is written |
| RB-5 | Tolerate a command arriving in more than one write | A line longer than the negotiated transmission unit is split |
| RB-6 | Act on each of the thirteen movement and pose commands of section 1.19 | Three of the sixteen actions are the tablet's own work; the rest are the robot's |
| RB-7 | Treat a repeated movement command as a continuation, not a fresh start | A held direction button repeats its command every second |
| RB-8 | Stop on STOP, whatever it was doing | This is the one command that has to work when everything else has gone wrong |
| RB-9 | Stop by itself if commands stop arriving | The tablet may lose the link in the middle of a movement; the robot must not carry on |
| RB-10 | Answer with text lines if it has anything to say | Whatever arrives is shown in the Remote Control log; nothing is required, but anything sent is displayed |
| RB-11 | Report battery, temperature, mode, speed and position if it can | Until it does, those readings on the tablet are simulated, and are marked as such |
| RB-12 | Say what it does on FOLLOW START and FOLLOW STOP | Following is a switch on the tablet; what it means is the robot's to define |
| RB-13 | Keep the vocabulary stable once agreed | A change on the robot that is not told to the application team looks, on the tablet, exactly like a broken radio link |

## 1.19  The command vocabulary as an interface agreement

This is the whole of what the tablet will ever write to the robot for the sixteen actions. It is repeated in Document 2, section 2.4, as a requirement and in Document 3, section 3.8, as a design; here it is the thing the two sides agree on before either can finish.

| Action | Line written | Whose work |
| --- | --- | --- |
| Forward | MOVE FORWARD | Robot |
| Backward | MOVE BACKWARD | Robot |
| Turn Left | TURN LEFT | Robot |
| Turn Right | TURN RIGHT | Robot |
| Stop | STOP | Robot |
| Arm Left | ARM SELECT LEFT_ARM | Robot |
| Arm Right | ARM SELECT RIGHT_ARM | Robot |
| Wave | POSE WAVE | Robot |
| Hello | GREET | Robot |
| T-Pose | POSE T_POSE | Robot |
| Dance | POSE DANCE | Robot |
| No | CANCEL | Robot |
| Follow me | FOLLOW START, then FOLLOW STOP when said again | Robot |
| Face Recognize | Nothing is sent | Tablet |
| Object Detect | Nothing is sent | Tablet |
| Recording | Nothing is sent | Tablet |

If the firmware answers to other words, the application can be changed to match in one file; what it cannot do is guess. Until the two lists agree, the tablet will report commands as delivered — they were, to the radio — while the robot does nothing, which is the most misleading failure this system has.

## 1.20  Changes carried into this revision

The application was reworked several times before this issue. What changed, and why, is recorded here because several of the changes are the reason a section reads as it does.

| Change | What was asked for | What was done |
| --- | --- | --- |
| Sidebar icons | Icons on the action buttons sat hard against the left edge | The sidebar button style was given a start padding, a drawable padding and start gravity, so icon and label both have room |
| Move Control | A group of movement buttons was wanted | A Move Control group was added to the sidebar with Forward, Backward, Turn Left, Turn Right and Stop |
| Button height | The buttons were too tall | The sidebar button height was reduced to 32 dp, and the text to 11 sp, which is what let nineteen buttons fit |
| Remote Control | Remote control is over Bluetooth, so the screen needed connection settings | Movement Control, Arm Control and Quick Actions were removed from that screen and replaced by the Bluetooth Connection panel |
| Camera permission | The permission dialog appeared again on every screen change | The fact that it has been asked is remembered application-wide, so it is asked once |
| The sixteen commands | The actions everywhere should be the commands the recogniser puts out | VoiceCommands was rewritten around the sixteen names, each with a sidebar button |
| Acknowledgement | A correct recognition should say OK from a prepared sound | assets/ok.wav is played on a recognised command, with spoken acknowledgement as a fallback |
| Recording button | A red circular record button in the corner of the camera view, and a way to watch what was recorded | Record and Open sit at the top right of the camera panel; Open lists the files and plays one |
| Korean speech | Korean recognition was missing | The English model is tiny-en and the Korean model is tiny-kp, chosen by the recognition language |
| Panel titles | The title row of each panel was too tall | The panel title style was reduced to 26 dp, returning ten dp to each panel |
| Load Map | A Load Map control was wanted on the LiDAR view | It sits in the LiDAR panel title row, beside the full-screen control |
| Delivery honesty | Testing showed the figure moving while the robot did not | The voice path and the button path were made to send the same wording, and every command now reports whether it reached the robot, only reached the demonstration link, or was not sent at all |

## 1.21  Quality gates

A build passes through these in order. A gate that fails sends the build back; it is not waived by agreement, because each of them exists to catch something that has already gone wrong once.

| Gate | What is checked | How |
| --- | --- | --- |
| Compiles | The package builds | gradlew assembleDebug |
| Resources | Every drawable, string, colour, dimension and style a layout names exists | The static check of Document 3, section 3.17 |
| Identifiers | Every identifier a screen looks up is reachable from that screen's layout | The same check |
| Translations | The four languages have the same strings, the same format specifiers and the same array lengths | The same check |
| Dead strings | No string is left that nothing uses | The same check |
| Starts | The application opens and every screen in the sidebar opens | TC-001 by hand |
| Link | A scan finds the robot and a connection is made | TC-010 and TC-012 |
| Commands | All sixteen leave the tablet in the agreed wording | TC-083 with the robot's console open |
| Regression | The short set of Document 5, section 5.16 | By hand |
| Full pass | Every case of Document 5 | Before acceptance only |

## 1.22  Definition of done

A piece of work is finished when all of these are true of it, not when the code exists.

It does what its requirement says, in the wording of that requirement.

It behaves when the thing it depends on is missing: no robot, no camera permission, no model, no Bluetooth.

It says what happened where the operator is looking.

Its text is in all four languages.

Its sizes come from the dimension resources, so it survives a narrower screen.

Its test case in Document 5 has been run and passes.

The static check passes with it in place.

Anything it leaves undone is written into section 1.10 rather than remembered.

## 1.23  Acceptance procedure

Acceptance is a single pass against one named build, in one sitting where practical, with the robot present.

Fix the build: note the package, its version and the date; install it on the target tablet and clear the application's data.

Run the static check and record its output.

Work through Document 5 in group order, recording pass, fail or blocked for every case.

Where a case fails, record what was seen instead, and whether it is on the tablet side or the robot side.

Check the objectives of section 1.4 one by one against the case results.

List the mandatory requirements whose cases did not pass; these are the only grounds for refusing acceptance.

Agree what happens to the outstanding items of section 1.10, which are known and are not grounds for refusal.

Sign the result sheet of Document 5, section 5.19.

## 1.24  Handover

What is handed over with the build, so that someone who was not part of the work can carry it on.

| Item | What it is |
| --- | --- |
| The package | falcon.ai.robot.apk, built from a named commit |
| The source | The repository, including the tools folder and the readme |
| The models | The assets the application needs, with a note in the readme of which are committed and where the others come from |
| This specification set | Six documents, the last of which is written for the operator |
| The static check | The script, which is the fastest way to find out whether a change has broken something |
| The test record | The result sheet from the acceptance pass |
| The outstanding list | Section 1.10, which is the honest statement of what is not finished |
| The interface agreement | Section 1.19, which is what the robot side has to keep to |

## 1.25  Keeping it running

| Job | How often | What it involves |
| --- | --- | --- |
| Model refresh | When a better model is available | Replace the asset, run the recognition cases again; nothing else changes |
| New command | When the robot learns a new trick | One rule in VoiceCommands, one button in the sidebar, four strings, one test case |
| New interface language | On request | One values folder; the parity check tells you what is missing |
| Platform update | When the target platform moves | Check the permission behaviour first: it is what changes most between platform versions |
| Tablet replacement | As needed | Install the package, grant the permissions, re-enrol the faces — the register lives on the tablet and is not copied anywhere |
| Log review | After any field session | The operation log is the record of what was sent and what came back |

## 1.26  From objective to evidence

Each objective of section 1.4 is met by a group of requirements and proved by a group of test cases. This table is the thread: it is what an acceptance meeting follows, and it is how an objective that nobody can demonstrate becomes visible.

| Objective | Requirements | Proved by |
| --- | --- | --- |
| One application reaches every part of the robot | FR-001 to FR-005 | TC-001 to TC-005 |
| The tablet finds the robot and connects to it over Bluetooth | FR-010 to FR-013 | TC-010 to TC-013 |
| A command reaches the robot, in an agreed wording | FR-014 to FR-018, FR-030 to FR-034 | TC-014 to TC-018, TC-030 to TC-034, and the field pass |
| The operator is never told a command arrived when it did not | FR-019, FR-044 | TC-019, TC-044, and section 5.26 of Document 5 |
| The robot can be driven continuously and stopped at once | FR-041 to FR-043 | TC-041 to TC-043, and the seven latch scripts |
| Sixteen spoken commands work wherever the operator is | FR-040, FR-080 to FR-092 | TC-040, TC-080 to TC-092, and sections 5.22 to 5.24 |
| Recognition runs on the tablet, with nothing sent anywhere | FR-080, FR-130 to FR-133 | TC-080, TC-130 to TC-133 |
| The camera is shared rather than fought over | FR-050, FR-051 | TC-050, TC-051 |
| What the camera sees can be recorded and watched | FR-052 to FR-057 | TC-052 to TC-057 |
| Faces can be enrolled and recognised, on this tablet only | FR-060 to FR-066 | TC-060 to TC-066 |
| The interface is complete in four languages | The localisation requirements of section 2.3 | TC-150, and section 5.27 |
| The application is usable on a tablet and survives a smaller screen | The portability requirements of section 2.3 | TC-151, and section 5.28 |

## 1.27  What this application deliberately does not need

Worth stating plainly, because each of these is an assumption someone will otherwise make, and each of them would change the work if it were true.

| Not needed | What follows from that |
| --- | --- |
| A network connection | Recognition runs on the tablet. There is no server to install, no address to configure, nothing to keep running |
| An account or a sign-in | Anyone who can unlock the tablet can work the robot; control of the application is control of the tablet |
| A cloud service | Nothing is uploaded, so nothing has to be agreed about where data is held |
| A pairing code or a bonded device | The link is made from the scan list; there is nothing to pair in advance |
| Wi-Fi in the room | The link is Bluetooth; the Wi-Fi indicator on Home describes the tablet, not the robot |
| A second operator | One person can connect, drive, speak and record |
| A charger during a session | Though on a long session with everything switched on, one is wanted |
| A particular tablet | Any Android 8.0 device runs it; the layout is built for a landscape tablet and degrades to a phone |

## 1.28  The tablet

What the application asks of the device it runs on, and what each requirement is for.

| Requirement | Why |
| --- | --- |
| Android 8.0 or newer | The platform features the service and the camera depend on |
| Landscape, 1280 × 800 dp or larger | The panels sit side by side above 840 dp; below that they stack and the page scrolls |
| A camera | Face recognition, object detection and recording |
| A microphone | Spoken commands |
| Bluetooth Low Energy | The link to the robot |
| About 300 MB free | The package, the models, and room for recordings |
| A processor with four or more cores | Three models can run at once; fewer cores means a slower picture rather than a failure |
| A speaker | The acknowledgement, and spoken feedback |

## 1.29  Decisions already taken

These were settled during the work and are not reopened without a reason that is written down. Each is here because it looks, from outside, like something that could be changed cheaply.

| Decision | The reason it stands |
| --- | --- |
| The sixteen actions are named exactly as the recogniser labels them | A keyword model can then find its action with no translation table in between |
| Three of the sixteen never reach the robot | Face recognition, object detection and recording are the tablet doing its own work; sending them would be asking the robot to do something it cannot |
| Every action has a sidebar button | Anything that can be said can be pressed, which is what makes a noisy room workable |
| Direction buttons latch rather than nudge | A robot that moves while a finger is held is unusable at arm length from a tablet |
| The repeat is once a second | Slow enough not to flood the link, fast enough for the robot to treat it as continuous |
| Delivery is reported in three states, not two | Sent, demonstrated and not sent are different things, and collapsing them is what hid the original fault |
| The camera lives in the service | Screens that each open the camera is the commonest cause of a black picture |
| Recording carries no sound | So that listening continues while recording |
| The interface language and the spoken language are separate settings | Operators often read one and speak another |
| Korean uses North Korean forms throughout | It is the language the users of this system read and speak |
| The face register never leaves the tablet | There is nothing to secure elsewhere, and nothing to agree about where it is held |
| No AppCompat, no Material Components | A smaller package and no theme conflicts on a device that is already fixed |

## 1.30  Questions still open

Not risks, and not defects: decisions that belong to the customer and have not been taken. Each is recorded with what the application does in the meantime, so that the present behaviour is a choice rather than an accident.

| Question | What happens meanwhile | What would change |
| --- | --- | --- |
| Should the spoken Recording start the tablet camera, or the robot camera? | It records on the tablet, which is the camera the application can actually work | One line in the rule, and the Remote Control recording panel would gain the spoken route |
| Does the firmware answer to the wording of section 1.19? | The application writes that wording and reports honestly that it did | The command strings in one file; the tablet can match whatever the robot uses |
| Will the robot send telemetry, and in what shape? | Battery, temperature, mode, speed and position are simulated, and said to be | A parser for the incoming lines; the panels are already there |
| Is there to be a video transport from the robot? | The Remote Control camera panel offers the controls but has no picture to show | A second transport; Bluetooth serial will not carry it |
| Should Go Home become the seventeenth spoken command? | It has a button but no phrase, because the recogniser does not label it | One rule, four strings, and a model that can produce the label |
| Who keeps the face register when a tablet is replaced? | Nobody: the new tablet starts empty | An export and import, which is the only place a copy would exist |

## 1.31  Who should read what

Six documents is more than anyone needs to read. This is the short answer to which parts belong to whom.

| If you are | Read | And skim |
| --- | --- | --- |
| Deciding whether to accept the work | This document, sections 1.4, 1.9, 1.10 and 1.26; the result sheet of Document 5 | The requirement tables of Document 2, for what was asked |
| Writing the robot firmware | Sections 1.18 and 1.19 of this document; Document 3, section 3.8 | Document 2, section 2.4, which says the same thing as a requirement |
| Taking over the application | Document 3 end to end; Appendices A and B of that document | Document 2, for why a thing is as it is |
| Changing a screen | Document 4, and section 3.24 of Document 3 | The control inventory for the screen you are touching |
| Testing a build | Document 5 end to end | Document 2, for the requirement behind a case you doubt |
| Operating the robot | Document 6, and nothing else | Section 6.23, before you report a fault |
| Writing a similar application | Document 3, sections 3.1, 3.3 and 3.25 | Section 1.29 of this document, for the decisions and their reasons |
| Estimating a change | Section 3.24 and section 3.26 of Document 3 | Section 5.33 of Document 5, for what the change puts at risk |

## 1.32  Keeping this set current

A specification that describes last month is worse than none, because it is believed. These are the changes that oblige a document to be updated, and which one.

| If this changes | Update |
| --- | --- |
| A command the tablet writes | Section 1.19 here, section 2.4 of Document 2, section 3.8 of Document 3, and the manual card in Document 6 |
| A spoken phrase or a matching rule | Section 2.10 of Document 2, sections 5.22 to 5.24 of Document 5, and sections 6.17 and 6.18 of Document 6 |
| A requirement | Document 2, and the case that proves it in Document 5 |
| A screen | Document 4: the panel table and the control inventory for that screen |
| A class that other classes call | Section 3.20 of Document 3 |
| A dimension, colour or style | Sections 4.21 to 4.23 of Document 4 |
| Something an operator has to do differently | Document 6, and only Document 6 |
| An outstanding item being finished | Section 1.10 here, and the revision history |
| A decision in section 1.29 | That section, with the reason it was reopened |

## 1.33  Asking for a change

So that a change can be costed rather than argued about. A request that answers these can be answered the same day.

| Say | Why it is needed |
| --- | --- |
| What should happen instead | The wording usually becomes the requirement |
| Where you saw the present behaviour | Which screen, and which build from Settings |
| Whether it is the tablet or the robot | Section 6.23 of Document 6 settles this in four questions, and it decides which team the work belongs to |
| Whether anything else depends on it | A command wording change is also a firmware change; a screen change usually is not |
| How urgent | Whether it goes into the next build or waits for the next issue |
| Whether the present behaviour is dangerous | Anything that could leave the robot moving is treated differently from anything else |

# Document 2  Requirements Specification

Each requirement carries an identifier, the area it belongs to, a priority and the test case that verifies it. Priorities are M (mandatory: acceptance depends on it), S (should: expected, and deferrable only by agreement) and C (could: included where it costs little).

## 2.1  Definitions

| Item | Detail |
| --- | --- |
| Service | The background component that owns the camera, the microphone and the models, and outlives the screens. |
| Link | The Bluetooth Low Energy connection to the robot, held by the application. |
| Demo link | The address-and-port connection offered by the connection dialog, which has no transport behind it and reaches no robot. |
| Command | One line of text sent to the robot, such as MOVE FORWARD. |
| Action | One of the sixteen things the operator can ask for, by button or by voice. |
| Latch | A direction button that stays on, with its command repeated every second. |
| Panel | A bordered section of a screen with a title row. |

## 2.2  Functional requirements

### 2.2.1  Shell

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-001 | The application shall present eight screens — Home, Robot Control, Face Recognition, Voice Recognition, Object Detection, LiDAR SLAM, Remote Control and Settings — reachable from a sidebar present on every screen. | M | TC-001 |
| FR-002 | The sidebar shall mark the screen that is open. | M | TC-002 |
| FR-003 | The application shall run full screen, with the system bars hidden, and shall not rotate away from the layout the device reports. | S | TC-003 |
| FR-004 | A splash screen shall be shown while the first screen is prepared. | S | TC-004 |
| FR-005 | Every screen shall show the robot's connection state and shall offer the connection dialog from that indicator. | M | TC-005 |

### 2.2.2  Link

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-010 | The application shall scan for Bluetooth Low Energy devices that advertise a name, and shall list them strongest signal first. | M | TC-010 |
| FR-011 | A scan shall stop by itself after twelve seconds. | S | TC-011 |
| FR-012 | The application shall connect to a device chosen from the list, discover the serial service, enable notifications and only then report the link as connected. | M | TC-012 |
| FR-013 | The application shall request the permissions the platform version needs before scanning, and shall say so when they are refused. | M | TC-013 |
| FR-014 | Commands shall be sent as one line of UTF-8 text terminated by a newline, split to the negotiated maximum transmission unit. | M | TC-014 |
| FR-015 | The application shall write using whichever write type the robot's characteristic offers. | M | TC-015 |
| FR-016 | A write the platform refuses shall be retried when the link is next free, and shall not block the commands behind it. | M | TC-016 |
| FR-017 | Text the robot sends back shall be shown in the Remote Control log. | S | TC-017 |
| FR-018 | A device that does not offer the serial service shall be reported and disconnected. | M | TC-018 |
| FR-019 | When the link drops, every screen shall show the robot as offline, and commands shall fail rather than appear to succeed. | M | TC-019 |
| FR-020 | The link shall be held by the application, not by a screen: leaving the Remote Control screen shall not disconnect the robot. | M | TC-020 |
| FR-021 | The connection dialog's address link shall be labelled in the interface as a demonstration link that reaches no robot. | M | TC-021 |

### 2.2.3  Robot Control

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-030 | The screen shall show robot status: mode, speed, battery, temperature and estimated position. | M | TC-030 |
| FR-031 | The screen shall show a 3D figure of the robot that acts out each command as it is given. | S | TC-031 |
| FR-032 | The screen shall show the live camera with face and object results drawn over it. | M | TC-032 |
| FR-033 | The screen shall offer a choice of camera source from the camera view. | M | TC-033 |
| FR-034 | The screen shall offer camera settings — brightness, contrast, saturation, resolution and frame rate — in their own panel, scrollable by dragging. | M | TC-034 |
| FR-035 | The screen shall show the LiDAR map, at the same width as the 3D view above it, with its own full-screen control. | S | TC-035 |
| FR-036 | The LiDAR panel shall offer Load Map from its title row. | S | TC-036 |
| FR-037 | The screen shall offer a log that, when started, records face, object and speech results and the commands sent, each with the time. | M | TC-037 |
| FR-038 | A status line along the foot shall always show the most recent activity. | M | TC-038 |
| FR-039 | Robot Status, Camera and LiDAR panels shall each be able to fill the screen and be restored. | S | TC-039 |
| FR-040 | The sidebar shall carry a button for every spoken command. | M | TC-040 |
| FR-041 | Buttons shall switch face recognition, object detection and listening on and off. | M | TC-041 |
| FR-042 | A direction button shall latch on, and the command shall be repeated to the robot every second until it is stopped. | M | TC-042 |
| FR-043 | Pressing the latched direction again, pressing another direction, pressing Stop, or leaving the screen shall end the movement. | M | TC-043 |
| FR-044 | Every command sent from this screen shall be reported as delivered to the robot, sent to the demonstration link only, or not sent at all. | M | TC-044 |
| FR-045 | A spoken command shall light the same button, and drive the same latch, as the button for it. | M | TC-045 |

### 2.2.4  Camera

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-050 | The camera shall be opened once by the background service and shared by every screen that shows it. | M | TC-050 |
| FR-051 | The camera permission shall be asked for once; a refusal shall not be asked again on every screen change. | M | TC-051 |
| FR-052 | A red record button in the top right of the camera view shall start and stop recording. | M | TC-052 |
| FR-053 | Recordings shall be written to the application's own folder as MP4, named with the date and time. | M | TC-053 |
| FR-054 | Recording shall carry no sound, so that listening continues while recording. | M | TC-054 |
| FR-055 | An Open button beside the record button shall list the recordings, newest first, and hand a chosen one to a video player. | M | TC-055 |
| FR-056 | Recording shall continue while the operator is on another screen. | S | TC-056 |
| FR-057 | Where the camera cannot bind the video use case, the picture shall be kept and the recording refused with a message. | M | TC-057 |

### 2.2.5  Face

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-060 | The application shall detect faces in the camera frame and draw a box for each. | M | TC-060 |
| FR-061 | The application shall identify a detected face against the register held on the tablet and show name, identifier, similarity, department, position and time. | M | TC-061 |
| FR-062 | A face that matches nothing shall be reported as unrecognised. | M | TC-062 |
| FR-063 | The operator shall be able to enrol a face with name, identifier, department and position. | M | TC-063 |
| FR-064 | The register shall be searchable, and a record shall be removable. | S | TC-064 |
| FR-065 | The register shall be held on the device and shall survive restart. | M | TC-065 |
| FR-066 | Recognition events shall be listed with time and result. | S | TC-066 |

### 2.2.6  Objects

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-070 | The application shall detect the eighty COCO classes in the camera frame. | M | TC-070 |
| FR-071 | Boxes, labels and segmentation masks shall each be switchable. | S | TC-071 |
| FR-072 | Objects shall be tracked between frames and keep their identity. | S | TC-072 |
| FR-073 | The confidence threshold shall be adjustable. | S | TC-073 |
| FR-074 | The screen shall list what is in frame with counts, and report the inference time. | S | TC-074 |

### 2.2.7  Speech

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-080 | The application shall recognise speech on the device, with no network connection. | M | TC-080 |
| FR-081 | English and Korean shall be offered; the English model shall be used for English and the tiny-kp model for Korean. | M | TC-081 |
| FR-082 | The language chosen shall be remembered across screen changes and restarts. | M | TC-082 |
| FR-083 | The application shall recognise the sixteen commands of section 2.7 and carry each one out. | M | TC-083 |
| FR-084 | A phrase that matches no command shall be reported as Unknown Command, without showing what was heard, and nothing shall be sent. | M | TC-084 |
| FR-085 | A recognised command shall be acknowledged audibly. | M | TC-085 |
| FR-086 | Face Recognize, Object Detect and Recording shall act on the tablet rather than being sent to the robot. | M | TC-086 |
| FR-087 | Spoken switches and the buttons for them shall never disagree about which way they are set. | M | TC-087 |
| FR-088 | Listening shall continue while the operator is on another screen. | S | TC-088 |
| FR-089 | The Voice screen shall show the transcript, the matched command, the confidence and a history of what was said. | S | TC-089 |
| FR-090 | The Voice screen shall list every command with an example phrase, and running one from the list shall behave as though it had been spoken. | S | TC-090 |
| FR-091 | The operator shall be able to switch command control, continuous listening and noise suppression. | S | TC-091 |
| FR-092 | A live waveform shall be shown while listening, at ninety per cent of the panel width. | C | TC-092 |

### 2.2.8  LiDAR

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-100 | The LiDAR screen shall offer a 3D point cloud, a top view and a 2D map. | S | TC-100 |
| FR-101 | Mapping shall be startable and stoppable, and the state shown. | S | TC-101 |
| FR-102 | A map shall be savable, and a saved map loadable, from the LiDAR screen and from the Robot Control screen. | S | TC-102 |
| FR-103 | Range, resolution, mapping mode and update rate shall be settable and applied to the robot. | C | TC-103 |
| FR-104 | Pose and scan quality shall be shown. | C | TC-104 |

### 2.2.9  Remote

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-110 | The Remote Control screen shall show the robot camera, robot status with position, and the map. | S | TC-110 |
| FR-111 | The screen shall hold the Bluetooth Low Energy panel: state, scan, device list and disconnect. | M | TC-111 |
| FR-112 | Photographs, video and audio recording on the robot shall be commandable, and what has been captured listed. | C | TC-112 |
| FR-113 | An operation log shall record what happened, and be clearable. | S | TC-113 |

### 2.2.10  Settings

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-120 | Settings shall offer the connection, the interface language, the camera and an about box. | S | TC-120 |
| FR-121 | The interface language shall be selectable, and shall take effect at once. | M | TC-121 |
| FR-122 | Camera lens and analysis frame size shall be selectable and remembered. | S | TC-122 |

### 2.2.11  Service

| ID | Requirement | Pri. | Test |
| --- | --- | --- | --- |
| FR-130 | Face, object and speech recognition shall run in a foreground service that outlives the screens. | M | TC-130 |
| FR-131 | The service shall show a notification while it holds the camera or the microphone, and offer to stop from it. | M | TC-131 |
| FR-132 | Switch states shall be remembered across restarts. | S | TC-132 |
| FR-133 | The service shall stop by itself when nothing is switched on. | S | TC-133 |

## 2.3  Non-functional requirements

| ID | Class | Requirement | Test |
| --- | --- | --- | --- |
| NFR-01 | Performance | Object detection shall report its inference time; on the target tablet a 640 × 480 frame shall be processed in under 150 ms. | TC-140 |
| NFR-02 | Performance | The camera preview shall stay fluid while both models run. | TC-141 |
| NFR-03 | Performance | A spoken command shall be acted on within two seconds of the speaker finishing. | TC-142 |
| NFR-04 | Performance | A held direction shall be repeated to the robot every second, within ±200 ms. | TC-143 |
| NFR-05 | Availability | Recognition and recording shall survive screen changes and the screen going off. | TC-144 |
| NFR-06 | Robustness | No single refused Bluetooth write shall stop later commands. | TC-016 |
| NFR-07 | Robustness | A missing or unreadable model shall be reported and shall not stop the rest of the application. | TC-145 |
| NFR-08 | Privacy | No image, recording, transcript or face record shall leave the device. | TC-146 |
| NFR-09 | Privacy | Recordings shall be written where no other application can read them, and shared only when the operator opens one. | TC-147 |
| NFR-10 | Security | The application shall hold no credentials and shall open no listening socket. | TC-148 |
| NFR-11 | Usability | Every control shall be operable by finger or mouse; panels that hold more than fits shall scroll by dragging. | TC-149 |
| NFR-12 | Usability | The operator shall always be able to tell whether a command reached the robot. | TC-044 |
| NFR-13 | Localisation | Every visible string shall exist in English, Korean, Japanese and Chinese, with the same placeholders in each. | TC-150 |
| NFR-14 | Portability | The layout shall adapt to screen width, using stacked panels below 600 dp and side-by-side panels above 840 dp. | TC-151 |
| NFR-15 | Maintainability | Every command the application can send shall be defined in one place in the source. | TC-152 |
| NFR-16 | Maintainability | Resource, translation and reference checks shall pass before a build is offered for test. | TC-153 |
| NFR-17 | Offline | The application shall be fully functional in flight mode. | TC-146 |
| NFR-18 | Storage | Recordings shall be kept until the operator removes them; the application shall not delete them by itself. | TC-154 |

## 2.4  Interface to the robot

The robot link is a serial line over Bluetooth Low Energy. The application writes one command per line in UTF-8, terminated by a newline, and reads whatever the robot sends back as text. The service and characteristic identifiers are in Document 3, section 3.8; they are constants in one source file and are changed there if the firmware uses its own.

### 2.4.1  The sixteen spoken commands

These are what the speech recognition puts out. Each has a button in the Robot Control sidebar, and the name below is exactly the label the recogniser uses, so a keyword model's output finds its action without translation.

| Command | Sent to the robot | Effect |
| --- | --- | --- |
| Forward | MOVE FORWARD | Move ahead; repeated every second while latched |
| Backward | MOVE BACKWARD | Move back; repeated every second while latched |
| Turn Right | TURN RIGHT | Turn right; repeated every second while latched |
| Turn Left | TURN LEFT | Turn left; repeated every second while latched |
| Follow me | FOLLOW START / FOLLOW STOP | Switch: alternates on each order |
| Face Recognize | — (tablet) | Switches face recognition on the tablet |
| Object Detect | — (tablet) | Switches object detection on the tablet |
| Dance | POSE DANCE | One-off pose |
| Stop | STOP | Stops movement and releases the latch |
| Arm Left | ARM SELECT LEFT_ARM | Selects the left arm |
| Arm Right | ARM SELECT RIGHT_ARM | Selects the right arm |
| Wave | POSE WAVE | One-off pose |
| Hello | GREET | Greeting |
| T-Pose | POSE T_POSE | One-off pose |
| No | CANCEL | Cancels what was asked for; releases the latch |
| Recording | — (tablet) | Switches the tablet's camera recording |

### 2.4.2  Commands from other controls

| Command | Raised by | Meaning |
| --- | --- | --- |
| GO_HOME | Go Home button | Return to the docking position |
| SHUTDOWN | Shutdown button, after confirmation | Power the robot down |
| VOICE LISTEN ON / OFF | Listening switched on or off | Tells the robot the tablet is listening |
| CAMERA SELECT <source> | Camera chooser | Choose the robot's camera |
| CAMERA B<n> C<n> S<n> | Camera settings panel | Brightness, contrast, saturation |
| CAMERA <resolution> | Camera settings panel | Frame size |
| CAMERA PHOTO | Remote Control | Take a photograph |
| CAMERA VIDEO START / STOP | Remote Control | Robot-side video recording |
| AUDIO RECORD START / STOP | Remote Control | Robot-side audio recording |
| SLAM MAPPING START / SLAM STOP | LiDAR screen | Mapping |
| SLAM SAVE_MAP | LiDAR screen | Save the current map |
| SLAM LOAD_MAP <name> | LiDAR screen, Robot Control LiDAR panel | Load a saved map |
| SLAM AUTO_NAV ON / OFF | LiDAR screen | Autonomous navigation |
| FACE REGISTER <id> | Face screen | A face was enrolled |
| MOVE <x> <y> | Remote Control joystick | Analogue drive, one decimal each |
| MOVE STOP | Remote Control joystick released | Stop analogue drive |
| SPEED <percent> | Remote Control | Drive speed |
| MODE <name> | Remote Control | Control mode |
| ODOMETRY RESET | Remote Control | Zero the reported position |

## 2.5  Permissions

| Permission | When it is asked for | If it is refused |
| --- | --- | --- |
| CAMERA | The first time a screen shows the picture, or a camera feature is switched on | The design artwork stays in the panel; the prompt is not repeated on every screen change |
| RECORD_AUDIO | Listening is switched on | Listening stays off and says why |
| POST_NOTIFICATIONS | A background feature is switched on (Android 13+) | Recognition still runs; the notification is hidden |
| BLUETOOTH_SCAN, BLUETOOTH_CONNECT | Scan is pressed (Android 12+) | The scan does not start and the reason is logged |
| ACCESS_FINE_LOCATION | Scan is pressed (Android 11 and older) | As above; the platform requires it for scanning on those versions |
| FOREGROUND_SERVICE and its camera and microphone types | Declared, not asked | Not applicable |
| ACCESS_NETWORK_STATE | Declared, not asked | Used only to draw the Wi-Fi indicator in the status row |

## 2.6  Data held on the device

| Data | Where | Lifetime |
| --- | --- | --- |
| Face register | files/faces/faces.json (records and embeddings) | Until the operator removes a record or clears the application's data |
| Face photographs | files/faces/ | As above |
| Recordings | Android/data/com.falcon.robot/files/Movies/ | Until the operator removes them |
| Switch states, language, camera choices | SharedPreferences file “settings” | Until changed or the data is cleared |
| Custom spoken phrases | Application files | Until removed |
| Speech and vision models | Assets inside the package, unpacked on first use | For the life of the installation |

## 2.7  Languages

The interface exists in English (default), Korean, Japanese and Chinese. The Korean interface uses North Korean forms. Speech recognition offers English and Korean; the interface language and the recognition language are separate settings, because the operator may prefer to read one and speak another.

## 2.8  Traceability

Every requirement above names its test case, and Document 5 section 5.9 carries the reverse index. A requirement with no passing test case is not accepted, whatever the state of the code.

## 2.9  Use cases

| ID | Use case | Actors | Pre-condition | Main flow | Outcome | Requirements |
| --- | --- | --- | --- | --- | --- | --- |
| UC-01 | Take the robot out and drive it | Operator, robot | The tablet is on and the robot is powered | 1. Open Remote Control and press Scan. 2. Tap the robot. 3. Open Robot Control. 4. Press Forward; the robot moves and keeps moving. 5. Press Stop. | The robot stands still where it was driven to, and each command was reported as delivered | FR-010, FR-012, FR-042, FR-044 |
| UC-02 | Drive by voice | Operator, robot | The robot is connected | 1. Press Voice in the sidebar. 2. Say “forward”. 3. Hear the acknowledgement and watch the Forward button light. 4. Say “stop”. | The robot moved on the spoken order and stopped on the spoken order | FR-083, FR-045, FR-085 |
| UC-03 | Enrol a visitor and recognise them | Operator, visitor | The camera permission is granted | 1. Open Face Recognition. 2. Have the visitor face the camera. 3. Press add and fill in the details. 4. Have them step away and return. | The visitor is named, with a similarity figure, and the event is in the history | FR-060, FR-061, FR-063 |
| UC-04 | Record an inspection | Operator | The camera permission is granted | 1. Open Robot Control. 2. Press the red button. 3. Drive the robot along the route. 4. Press the red button again. 5. Press Open and play the file. | An MP4 of the route is on the tablet and plays | FR-052, FR-053, FR-055 |
| UC-05 | Watch for objects while driving | Operator | The robot is connected | 1. Press Objects in the sidebar. 2. Press Start on the Log panel. 3. Drive the robot. | Boxes follow what the camera sees and the log lists what was found, with times | FR-070, FR-037 |
| UC-06 | Load a map before a patrol | Operator, robot | The robot is connected and maps have been saved | 1. Press Load Map in the LiDAR title row. 2. Choose the map. | The robot is told to load that map and the status line says so | FR-036, FR-102 |
| UC-07 | Work in Korean | Operator | None | 1. Settings → Interface Language → Korean. 2. Voice Recognition → Advanced Settings → Language → Korean → Apply. 3. Speak a command in Korean. | The interface is in Korean and Korean speech is recognised by the tiny-kp model | FR-081, FR-121 |
| UC-08 | Find out why the robot is not moving | Operator | Commands appear to do nothing | 1. Look at the line along the foot of Robot Control. 2. Follow what it says. | The operator learns whether the command was delivered, went to the demonstration link or was not sent, and what to do about it | FR-044, FR-021 |
| UC-09 | Hand the tablet to a colleague mid-task | Two operators | Recognition is running | 1. Leave the application with the home button. 2. The colleague returns to it. | Recognition and any recording have continued; the robot is still connected | FR-130, FR-056, FR-020 |
| UC-10 | Stop everything quickly | Operator | Several features are on | 1. Open the notification shade. 2. Press Stop on the notification. | The camera and the microphone are released and the service stops | FR-131, FR-133 |

## 2.10  The command dictionary

A requirement that says “the application shall recognise Forward” is not testable until it says what counts as Forward. This section is that statement, and it is the same data the application holds: one rule per action, in the recogniser's own order.

### 2.10.1  English

English is matched by keyword, not by whole phrase, so that small recognition errors still find the action: “move forward please” and “go forwards” both reach Forward. Where two words are listed as both, the phrase has to contain one from each group — that is what separates Arm Left from Turn Left. Excluded words rule a rule out even when its keyword is present.

| Action | Any of these words | Ruled out by | Sent |
| --- | --- | --- | --- |
| Forward | forward, forwards, ahead, straight | — | MOVE FORWARD |
| Backward | back, backward, backwards, reverse | — | MOVE BACKWARD |
| Turn Right | right | arm | TURN RIGHT |
| Turn Left | left | arm | TURN LEFT |
| Follow me | follow | — | FOLLOW START / FOLLOW STOP |
| Face Recognize | face | — | carried out on the tablet |
| Object Detect | object, objects, target, detect, detection | — | carried out on the tablet |
| Dance | dance, dancing | — | POSE DANCE |
| Stop | stop, halt, freeze | record, recording | STOP |
| Arm Left | arm and left, both | — | ARM SELECT LEFT_ARM |
| Arm Right | arm and right, both | — | ARM SELECT RIGHT_ARM |
| Wave | wave, waving | — | POSE WAVE |
| Hello | hello, hi, hey | — | GREET |
| T-Pose | t, tee or tpose, with pose or tpose | — | POSE T_POSE |
| No | no, nope, cancel | — | CANCEL |
| Recording | recording, record | — | carried out on the tablet |

### 2.10.2  Chinese, Japanese and Korean

These three are matched as substrings instead. Chinese and Japanese put no spaces between words, and Korean glues its endings onto the verb, so 왼쪽으로 and 왼쪽 돌아 share 왼쪽 and little else. Every rule holds all of them at once, whatever the interface is set to, because what matters is the language the operator speaks.

| Action | Chinese | Japanese | Korean |
| --- | --- | --- | --- |
| Forward | 前进, 向前, 往前 | 前進, 前へ, 進め | 전진, 앞으로, 직진 |
| Backward | 后退, 向后, 往后 | 後退, 下がれ, バック | 후진, 뒤로, 물러 |
| Turn Right | 右转, 向右 | 右折, 右へ, 右に | 오른쪽, 우회전, 우측 |
| Turn Left | 左转, 向左 | 左折, 左へ, 左に | 왼쪽, 좌회전, 좌측 |
| Follow me | 跟我走, 跟随, 跟着我 | ついてきて, 追従, フォロー | 따라와, 날따라, 따르시오, 추종 |
| Face Recognize | 人脸识别, 识别人脸 | 顔認識, 顔の認識 | 얼굴 |
| Object Detect | 目标检测, 物体识别, 追踪目标 | 物体検出, 対象追跡 | 대상, 물체, 객체 |
| Dance | 跳舞, 跳个舞 | 踊って, ダンス | 춤, 댄스 |
| Stop | 停止, 停下, 别动 | 停止, 止まれ, ストップ | 정지, 멈춰, 스톱, 그만 |
| Arm Left | 左臂, 左手 | 左腕, 左手 | 왼팔, 왼쪽 팔, 왼손 |
| Arm Right | 右臂, 右手 | 右腕, 右手 | 오른팔, 오른쪽 팔, 오른손 |
| Wave | 挥手, 打招呼 | 手を振, 挨拶 | 손 흔들, 손흔들, 인사 |
| Hello | 你好, 您好 | こんにちは, やあ | 안녕, 반갑 |
| T-Pose | t姿势, 标准姿势 | tポーズ, ティーポーズ | 티 자세, 티자세, t 자세 |
| No | 取消, 不要 | いいえ, キャンセル, やめ | 아니, 취소, 그만두 |
| Recording | 录像, 录制 | 録画, レコーディング | 록화, 녹화, 촬영 |

The Korean phrases include the ones the tiny-kp model in the assets was trained to produce, which is what the Voice screen hears when Korean is the recognition language. Korean here is in the North Korean forms the interface uses; 록화 and 녹화 are both accepted for the same reason.

### 2.10.3  Rules the matcher follows

| No. | Rule |
| --- | --- |
| 1 | The transcript is lowercased and matched against every rule in the order above; the first rule that matches wins |
| 2 | An English rule matches when the transcript contains at least one word from every one of its groups |
| 3 | A Chinese, Japanese or Korean rule matches when the transcript contains one of its phrases anywhere |
| 4 | A rule does not match if the transcript contains one of its excluded words, in any language |
| 5 | Nothing matching leaves the result as Unknown Command, and nothing is sent |
| 6 | A match is acknowledged with the prepared sound before anything is sent |

## 2.11  Actions that are switches

Four of the sixteen are switches rather than one-off orders: saying one again undoes what it started. The switch is held on the tablet, so a button and a spoken order move the same switch, and the button shows the state whichever of them changed it.

| Action | On | Off | Where the state shows |
| --- | --- | --- | --- |
| Follow me | FOLLOW START | FOLLOW STOP | The Quick Actions button and the status line |
| Face Recognize | Face analysis is switched on in the service | Switched off | The AI group button, the Face screen switch, and the overlay |
| Object Detect | Object analysis is switched on | Switched off | The AI group button, the Object screen switch, and the overlay |
| Recording | Video recording starts on the tablet | Recording stops and the file is closed | The record button turns into a red square, and the status line says Recording |

The other twelve are momentary: each says one line and is finished. Forward, Backward, Turn Left and Turn Right are momentary as spoken orders but are latched when they are pressed as buttons, which is described in Document 3, section 3.9.

## 2.12  States the application must be able to show

Most of the ways this application can mislead an operator are states it fails to show. Each state below is required to be visible, and to be distinguishable from the ones next to it.

### 2.12.1  The link

| State | How it is shown | What a command does in it |
| --- | --- | --- |
| Not connected | Offline, grey dot, on every screen | Nothing is sent, and the screen says so |
| Scanning | Scanning, amber dot; the button reads Stop Scan | As above |
| Connecting | Connecting to <name>, amber dot | As above |
| Connected | Connected · <name>, teal dot, on every screen | The line is written to the robot and the screen says it was delivered |
| Bluetooth off | Bluetooth is off, grey dot | Nothing is sent; the scan refuses to start and says why |
| Demonstration link | Connected, from the address dialog | Nothing reaches a robot; the screen says the command went to the demonstration link only |

### 2.12.2  The camera

| State | How it is shown |
| --- | --- |
| Permission never asked | The prompt appears once, at the moment a picture is wanted |
| Permission refused | The design artwork stays in the panel, with one line saying the permission is needed; the prompt does not come back on every screen change |
| Running | The picture, with the resolution in the corner |
| Recording | The record control is a red square and the status line says Recording |
| In use by another screen | The picture follows the screen that is open; the service rebinds rather than opening a second camera |

### 2.12.3  Recognition

| State | How it is shown |
| --- | --- |
| Model loading | The AI line names the model that is loading |
| Model missing or failed | The line says which model, and what is wrong; the rest of the screen keeps working |
| Listening | The listening state and the live waveform on the Voice screen; the Voice button in the AI group is on |
| Heard and matched | The transcript, the confidence, the matched action, and the acknowledgement sound |
| Heard and not matched | Unknown Command; what was heard is not offered as a command |
| Speech engine not built | Said plainly: the optional native engine is absent from the build, and the other models are used |

## 2.13  What must happen when something is missing

These are required behaviours, not accidents of implementation. Each of them exists because the opposite behaviour — a silent failure, or a success that did not happen — has been seen in testing.

| Missing | Required behaviour | Related requirement |
| --- | --- | --- |
| The robot | Commands fail rather than appear to succeed; the screen names the command and says it was not sent | FR-019 |
| A transport behind the demonstration link | The command is logged, the screen says it reached the demonstration link only, and no claim is made about the robot | FR-019 |
| Camera permission | The screen keeps its layout and says what it needs; no repeated prompting | FR-051 |
| Microphone permission | Listening stays off and says why | Section 2.5 |
| Bluetooth permissions | The scan does not start, and the reason is in the log | FR-013 |
| Bluetooth turned off | The state says so, rather than reporting an empty scan | FR-013 |
| A speech model | The screen names the model and carries on with what it has | FR-081 |
| The optional native engine | The application says the engine was not built; nothing else is affected | FR-080 |
| A face in frame | The result panel keeps its last result and says nothing was seen | FR-060 |
| A recognised face | Unrecognised, rather than the nearest name | FR-062 |
| Storage for a recording | The recording does not start, and says why | FR-057 |
| A recording to play | The list says there is nothing yet | FR-055 |
| A map to load | Load Map says there is no map rather than opening an empty view | FR-102 |

## 2.14  Timing

These are the targets the application is designed to; they are measured in Document 5, section 5.28, on the target tablet. Where a figure depends on the device they are written as what the operator should experience rather than as a promise about any particular chip.

| What | Target | Why this figure |
| --- | --- | --- |
| A button press to a line leaving the tablet | Under 100 ms | It has to feel like the button did something |
| A held direction repeating | Every 1000 ms | Often enough for the robot to treat it as continuous, seldom enough not to flood the link |
| Stop reaching the robot | Under 200 ms from the press | Stop is the safety control; it goes to the front of the queue |
| A scan finding a device in range | Within a few seconds; the scan itself lasts 12 s | Long enough for a device advertising slowly, short enough not to drain the tablet |
| Connect to usable | Under 5 s, including service discovery and notifications | Any longer and the operator presses it again |
| Camera picture after opening a screen | Under 1 s once permission is granted | The service keeps the camera open, so this is a rebind, not a cold start |
| Face inference | Under 200 ms per frame | So the boxes track the person |
| Object inference | Under 200 ms per frame | As above |
| Speech: end of an utterance to a result | Under 2 s | Longer than this and the operator repeats themselves |
| The acknowledgement sound | Immediately on a match, before the command is sent | It is the operator's evidence that they were heard |
| Screen change | Under 500 ms | The camera and the models are not restarted |

## 2.15  Languages and text

The interface exists in four languages and the application is built so that adding a fifth is one folder of strings. What follows is what “complete” means for a language.

| Requirement | Detail |
| --- | --- |
| Every visible string is a resource | No text that an operator can read is written in code |
| The four sets are level | English, Korean, Japanese and Chinese hold the same string names; at this revision 443 strings and 22 arrays in each, plus 4 language names that are deliberately shown in their own language everywhere |
| Format specifiers match | A string with %1$s in one language has %1$s in all of them; a mismatch is a crash, not a cosmetic fault |
| Arrays match in length | A dropdown that offers five choices in English offers five in Korean, in the same order, because the code selects by position |
| Korean is North Korean in form | Throughout the interface, and in the spoken phrases |
| Nothing is left unused | A string that nothing shows is removed rather than left to be translated again |
| Interface and speech are separate | The language the operator reads and the language they speak are two settings, because they are often not the same |
| Layouts survive translation | A label that is one word in English may be three in another language; the panels are measured so that this does not push a control off the screen |

## 2.16  What is kept, and what is not

The application keeps very little, and none of it leaves the tablet.

| Kept | Not kept |
| --- | --- |
| Face records: name, identifier, department, position, a photograph and the numeric embedding | Any recording of a voice; audio is used and discarded |
| Recognition history for the current session | Any copy of the face register outside the tablet |
| Video files the operator recorded, in the application's own media folder | Any upload: there is no network transport in the application at all |
| Switch states, the two languages, camera choices | Any analytics or usage reporting |
| Custom spoken phrases the operator added | Any crash reporting to a service |

Clearing the application's data removes all of it, including the face register. There is no backup: a tablet that is replaced starts with an empty register.

## 2.17  Counts

| Group | Mandatory | Should | Could | Total |
| --- | --- | --- | --- | --- |
| Functional requirements | 50 | 28 | 4 | 82 |
| Non-functional requirements | — | — | — | 18 |
| Test cases | — | — | — | 97 |
| Spoken actions | — | — | — | 16 |
| Commands written to the robot for those actions | — | — | — | 13 |
| Actions carried out on the tablet | — | — | — | 3 |
| Commands raised by other controls | — | — | — | 19 |
| Screens | — | — | — | 8 |
| Interface languages | — | — | — | 4 |
| Speech languages | — | — | — | 2 |

The priority split is counted from the priority recorded against each functional requirement in section 2.2; the non-functional requirements carry a class rather than a priority and are all expected. Acceptance turns on the mandatory ones; the others are deferred only by agreement, and only in writing.

## 2.18  Where each action can be raised

An action is not a button and not a phrase: it is the thing both of them ask for. Each of the sixteen is reachable from at least two places, and every place must produce the same command.

| Action | Sidebar | Spoken | Voice screen list | Gamepad |
| --- | --- | --- | --- | --- |
| Forward | Yes, latched | Yes | Yes | D-pad up |
| Backward | Yes, latched | Yes | Yes | D-pad down |
| Turn Left | Yes, latched | Yes | Yes | D-pad left |
| Turn Right | Yes, latched | Yes | Yes | D-pad right |
| Stop | Yes | Yes | Yes | B |
| Arm Left | Yes | Yes | Yes | — |
| Arm Right | Yes | Yes | Yes | — |
| Wave | Yes | Yes | Yes | X |
| Hello | Yes | Yes | Yes | — |
| T-Pose | Yes | Yes | Yes | — |
| Dance | Yes | Yes | Yes | Y |
| No | Yes | Yes | Yes | — |
| Follow me | Yes, a switch | Yes | Yes | — |
| Face Recognize | Yes, a switch | Yes | Yes | — |
| Object Detect | Yes, a switch | Yes | Yes | — |
| Recording | Yes, a switch | Yes | Yes | — |

The gamepad is a convenience rather than a requirement: a pad, where one is attached, drives the robot with the same commands as the on-screen stick, and its four face buttons stand for Stop, Go Home, Wave and Dance. Go Home has a button in Quick Actions but is not one of the sixteen spoken commands.

## 2.19  What the non-functional classes mean

Section 2.3 gives each non-functional requirement a class. This is what each class is judged by, so that “the application shall be responsive” is never the whole of a requirement.

| Class | Judged by |
| --- | --- |
| Responsiveness | The figures of section 2.14, measured rather than felt |
| Robustness | The application survives the cases of Document 5, section 5.30, without crashing and without leaving the robot moving |
| Honesty | Every state the application reports can be checked against something outside the application: the robot console, the file system, the platform settings |
| Availability | Recognition and the link survive screen changes, leaving the application, and an hour of continuous use |
| Portability | The layouts hold from a phone to the target tablet, and the platform differences are handled rather than assumed away |
| Localisation | The four languages are level, by the check rather than by inspection |
| Maintainability | A command, a language or a screen can each be added by touching the files listed in Document 3, section 3.24, and no others |
| Privacy | Nothing leaves the tablet, because there is no transport by which it could |

## 2.20  Requirements the tablet cannot close on its own

These are written down separately because a test report that marked them failed against the application would be pointing at the wrong side of the link.

| Requirement | What the tablet does | What is still needed |
| --- | --- | --- |
| A command makes the robot move | Writes the agreed line and reports that it did | Firmware that answers to that wording |
| Battery, temperature, mode and speed are real | Displays whatever the robot sends, and simulates it otherwise | Telemetry from the robot |
| The position is real | Displays what it is given | Odometry from the robot |
| Following works | Sends FOLLOW START and FOLLOW STOP and holds the switch | A following behaviour on the robot |
| The map is the room | Draws what it is given, and loads what was saved | A LiDAR and a mapper on the robot |
| The robot cameras | Offers the controls and the tabs | A video transport, which this link does not have |
| The robot stops when the link drops | Stops repeating, and reports the link as down | A timeout on the robot |

# Document 3  System Design Specification

This document describes how the application is put together: the parts, what each one owns, how they talk to each other, and the rules that keep a tablet with one camera, one microphone and one radio from contradicting itself.

## 3.1  Architecture at a glance

There are four layers. Screens are thin: they draw, and they ask. Everything that must outlive a screen belongs to the service or to one of the two singletons.

Screens (Activity)

Home  Robot Control  Face  Voice  Object  LiDAR  Remote  Settings

|  bindService / listener callbacks        |  send(command)

v                                          v

RobotService (foreground)                   RobotSession (singleton)

camera, microphone, models, recorder        connection state, transport

|                                          |

v                                          v

CameraX   ML Kit   TFLite   Moonshine       BleLink (singleton)

Preview   Face     YOLO     whisper.cpp      scan, GATT, write queue

Analysis  detect   26n      (optional)            |

Video                                             v

the robot

| Layer | Responsibility | Chief classes |
| --- | --- | --- |
| Screens | Draw the page, bind controls, show results. No screen opens the camera, the microphone or the radio itself. | BaseActivity and the eight page activities |
| Service | Own the camera, the microphone, the models and the recorder; carry out the orders that belong to the tablet; tell the screens. | RobotService, RobotService.Listener |
| Link | Hold the connection state and carry commands to the robot. | RobotSession, BleLink |
| Engines | Recognition proper, each behind a small interface. | FaceAnalyzer, FaceEmbedder, DetectionAnalyzer, YoloSegmenter, ObjectTracker, MoonshineEngine, WhisperEngine, CommandRecognizer, VoiceCommands |

## 3.2  Screens and the shell

Every screen extends BaseActivity, which inflates a shell (activity_shell.xml) with the navigation sidebar and drops the page's own layout into it. BaseActivity also provides the things every page needs, so that no page reinvents them.

| Member | What it does |
| --- | --- |
| setPage(layout, navId, titleRes) | Inflates the page into the shell and marks the sidebar entry. |
| setupColumns(containerId) | Reads each child's android:tag as a weight and lays the row out side by side or stacked, depending on R.bool.two_columns. |
| bindRobotService(listener) | Binds the service and registers the page's listener; onRobotServiceReady follows. |
| sendCommand(command) | Sends through RobotSession; when there is no link it offers the connection dialog and returns false. |
| setIcon(view, drawable, sizeDp, colour, gravity) | Draws a compound icon at an exact size and tint. |
| appendLog(list, scroll, colour, message, max) | Adds a timestamped line to a log panel and trims it. |
| ensureNotificationPermission() | Asks once, on Android 13 and newer. |
| cameraPermissionAsked() / noteCameraPermissionAsked() | Remembers, for the whole application, that the camera prompt has been shown; this is what stops the prompt returning on every screen change. |
| connectionLabel() | “Connected · <device>” for a Bluetooth link, or the address for the demonstration link. |

## 3.3  The background service

RobotService is a bound, started, foreground service. It is bound by whichever screens are open and started by itself, so that it outlives them; it stops when nothing is switched on. It runs with the camera and microphone foreground types, which Android requires for background use of either.

### 3.3.1  What it owns

The CameraX provider, and the use cases bound to it: Preview when a screen is showing the picture, ImageAnalysis when either vision model is on, VideoCapture while a recording is running.

The microphone, through SpeechRecorder, and the speech engines.

The face analyzer, the face embedder and the face register; the object segmenter and tracker.

The media player for the acknowledgement sound and the text-to-speech voice.

The switch states, which it writes to SharedPreferences so they survive a restart.

### 3.3.2  Callbacks to the screens

| Callback | When |
| --- | --- |
| onFaceFrame(faces, width, height, ms) | Every analysed frame, with faces in upright image coordinates |
| onFaceEvent(event) | A face was identified, or given up on as unknown |
| onRegistrationResult(embedding, crop) | An enrolment finished |
| onObjects(objects, width, height, ms) | Every analysed frame, with tracked objects |
| onVoiceLevel(level) | About every 20 ms while listening |
| onTranscript(text, confidence, action, result) | A phrase was recognised, with the action it matched and what became of it |
| onServiceState() | A switch changed, a model finished loading, a recording started or stopped |
| onMessage(text) | Something the operator should be told |

All callbacks arrive on the main thread. A page implements RobotService.Adapter and overrides only what it shows.

### 3.3.3  Camera binding rules

One method, bindCamera(), decides what is bound, and every change goes through it: a preview attaching or detaching, a model switching, the lens changing, the analysis size changing, a recording starting or ending. It unbinds everything and rebinds what is wanted now.

| Condition | Use cases bound |
| --- | --- |
| A screen is showing the picture | Preview |
| Face recognition or object detection is on | ImageAnalysis at the chosen frame size |
| A recording is running | VideoCapture (Recorder, HD with a lower-quality fallback) |
| None of the above | Nothing; the camera is released |

Two rules keep this honest. A recording in progress is finished before anything is rebound, so a settings change closes the file rather than truncating it. Where the camera refuses three use cases at once, the video one is dropped, the failure is reported, and the preview and the models are rebound without it.

### 3.3.4  Frame path

ImageAnalysis delivers a frame on the camera executor. It is converted once to an upright bitmap and handed to whichever models are on, so that two models never decode the same frame twice; results are posted to the main thread. The analysis size defaults to 640 × 480 and is a setting, because a larger frame finds smaller objects and a smaller one is faster.

### 3.3.5  Audio path

SpeechRecorder records 16 kHz mono from the VOICE_RECOGNITION source in 20 ms chunks, keeps a short pre-roll so that the first syllable is not lost, and cuts an utterance when the speaker stops. The samples go to whichever engine is loaded; the transcript goes to VoiceCommands, and the action that comes back is carried out.

## 3.4  Speech recognition

| Engine | Role | Notes |
| --- | --- | --- |
| CommandRecognizer | Keyword spotting from assets/commands.tflite | Tried first when the model is present; classifies straight into an action name. No such model ships at this revision. |
| MoonshineEngine | Full transcription, one model per language | tiny-en for English; for Korean, tiny-kp when that folder is present, and tiny-ko otherwise. Models are unpacked from the assets on first use. |
| WhisperEngine | Fallback transcription | Needs whisper.cpp cloned and compiled with the application; where it is absent the service says so rather than failing silently. |

### 3.4.1  From sound to action

SpeechRecorder  ->  engine  ->  transcript

|

CustomPhrases.match   (the operator's own phrases)

|  none

VoiceCommands.match   (keywords, or CJK substrings)

|

Action (one of sixteen)

|

command == null ----------+---------- command != null

|                                   |

RobotService.runHere                RobotSession.send(command)

face / objects / recording            exactly what the button sends

VoiceCommands holds the rules. English is matched by keyword, so ordinary padding (“move forward please”) still works; Chinese, Japanese and Korean are matched as substrings, because those languages do not separate words the way the keyword rule expects. A rule can exclude words, which is how “arm left” is kept from matching Turn Left and “stop recording” from matching Stop.

Three actions carry no command: Face Recognize, Object Detect and Recording are the tablet's own work. Each is a switch, and the direction it switches to is read from what is actually running, so a spoken order and the button for it cannot disagree.

## 3.5  Face recognition

Detection: ML Kit face detection, bundled with the package, on the analysis frame.

Identification: MobileFaceNet (assets/mobilefacenet.tflite) produces an embedding for the largest face; the register is searched for the nearest neighbour above a similarity threshold.

Register: files/faces/faces.json, holding identifier, name, department, position, the embedding and the enrolment time, with photographs beside it.

Tracking: recognitions are held briefly so that the panel does not flicker between frames of the same person.

## 3.6  Object detection

YoloSegmenter runs assets/yolo26n.tflite over the analysis frame and produces boxes, class scores and mask coefficients for the eighty COCO classes.

ObjectTracker matches detections between frames so that an object keeps its identity and its colour while it is in view.

TrackingOverlayView draws boxes, labels and masks over the picture; each is switchable, and the confidence threshold is a setting.

## 3.7  The link

### 3.7.1  RobotSession

RobotSession is the single answer to “are we connected, and where does a command go?”. It holds the connection flag, the address of the demonstration link, and whether the live link is the Bluetooth one.

| Member | Behaviour |
| --- | --- |
| isConnected() | True while connected; a Bluetooth link that has dropped is not connected, whatever was set earlier. |
| isLive() | True only when a command sent now actually goes somewhere — that is, over Bluetooth. The screens use this to tell a real link from the demonstration one. |
| setBleConnected() | The Remote Control screen hands the Bluetooth link over. |
| setConnected(false) | Drops the link, and disconnects Bluetooth if that is what was holding it. |
| send(command) | Writes over Bluetooth when that is the link; otherwise logs and returns success, which is why the screens label that case. |

### 3.7.2  BleLink

BleLink is the Bluetooth Low Energy client: scanning, connecting, service discovery, notifications and a write queue. Android delivers its callbacks on a binder thread; everything a screen sees is posted to the main thread first.

| Stage | What happens |
| --- | --- |
| Scan | Low-latency scan, devices with a name only, deduplicated by address, sorted by signal strength, stopped after twelve seconds. |
| Connect | connectGatt over the LE transport, then a request for a 185-byte transmission unit, then service discovery. |
| Discover | The serial service is looked up. Without it the operator is told and the link is dropped. |
| Notify | Notifications are enabled on the read characteristic and the descriptor is written. The link is reported connected in the descriptor's completion — not before, or the first command races that very operation and loses. |
| Write | One command at a time. The write type is taken from the characteristic's properties, because a firmware that offers only write-without-response refuses the other kind outright. A write the platform turns down leaves the chunk at the head of the queue for the next completion. |
| Read | Notified text is trimmed and passed to the screens as a message. |

## 3.8  Bluetooth interface

| Item | Value |
| --- | --- |
| Service | 6e400001-b5a3-f393-e0a9-e50e24dcca9e |
| Write characteristic | 6e400002-b5a3-f393-e0a9-e50e24dcca9e |
| Notify characteristic | 6e400003-b5a3-f393-e0a9-e50e24dcca9e |
| Client configuration descriptor | 00002902-0000-1000-8000-00805f9b34fb |
| Requested transmission unit | 185 bytes; payload is the negotiated size less three |
| Encoding | UTF-8, one command per line, terminated by \n |
| Scan duration | 12 seconds |

These are constants in BleLink. A firmware with its own identifiers is accommodated by changing them there; nothing else in the application knows them.

## 3.9  Movement and the latch

A robot controller usually stops a movement when its own timer runs out. The requirement is that Forward keeps going, so the Robot Control screen latches the direction and repeats the command every second. The latch is a small state machine, and a spoken direction drives it exactly as the button does.

| Event | Result |
| --- | --- |
| A direction is pressed | The command is sent, the button lights, the repeat starts |
| The lit direction is pressed again | STOP is sent and the latch is released |
| Another direction is pressed | The new command is sent and takes the latch over |
| Stop, or No, is pressed | STOP or CANCEL is sent and the latch is released |
| A direction is spoken | The service sends it; the screen takes over the repeating with the same wording |
| Stop is spoken | The service sends STOP; the screen releases the latch |
| The screen is left | STOP is sent, because nothing would repeat the command |

## 3.10  Recording

Recording is the service's work, so it continues while the operator is on another screen. The file is written straight to the application's own external files folder; no storage permission is involved, and nothing else on the tablet can read it. There is no audio track, deliberately: the microphone belongs to speech recognition.

| Step | Detail |
| --- | --- |
| Start | VideoCapture is bound, then a recording is prepared to files/Movies/VID_<date>_<time>.mp4 |
| During | The service stays in the foreground with the camera type |
| Stop | The recording is stopped; the file is closed by the finalize event, which reports the name and rebinds the camera without the video use case |
| Open | The Robot Control screen lists the files newest first and hands a chosen one to a video player through a FileProvider grant |

## 3.11  Threading

| Thread | Work |
| --- | --- |
| Main | All user interface work, all listener callbacks, the movement repeat, the Bluetooth state machine's visible half |
| Camera executor | Frame conversion, face detection, embedding, object detection and tracking |
| Speech executor | Model loading, transcription, keyword classification |
| CameraX internal | Capture and encoding |
| Binder | Bluetooth callbacks, immediately posted to main |

## 3.12  Persistence

| Store | Keys or content |
| --- | --- |
| SharedPreferences “settings” | face_enabled, detection_enabled, voice_enabled, voice_language, lens_facing, analysis_width, analysis_height, camera_permission_asked, language |
| files/faces/faces.json | The face register and its embeddings |
| files/Movies/ | Recordings |
| Application files | Unpacked speech models, custom phrases |

## 3.13  Resources and localisation

Strings, arrays, colours, dimensions and styles are resources; nothing user-visible is written in the source. Four locales are kept in step: values (English), values-ko, values-ja and values-zh. A check run before every build compares the four for missing names, differing array lengths and differing format placeholders.

| Resource | Purpose |
| --- | --- |
| values/strings.xml and its three translations | Every visible string and list |
| values/colors.xml | The dark palette: background #060A14, surface #0D172B, blue #2F80FF, teal #1ED9A4, cyan #22D3EE, amber #FFB547, red #FF4D5E |
| values/themes.xml | Panel, title, button, label and tab styles |
| values/dimens.xml, values-sw600dp, values-w840dp | Padding, gaps and sidebar widths per screen size |
| values/bools.xml, values-w840dp/bools.xml | two_columns, which decides side-by-side or stacked panels |

## 3.14  Custom views

| View | Purpose |
| --- | --- |
| FitScrollView | On a tablet, measures its child to exactly the viewport so that panel weights decide heights; below 520 dp it scrolls normally. |
| PanelScrollView | A panel's own scroll view that claims the drag, so the page beneath cannot steal it. This is what makes the camera settings reachable by dragging. |
| Robot3DView, RobotMesh, RobotPose | The figure of the robot, and the animation it plays for each command. |
| FaceOverlayView, TrackingOverlayView | Boxes, labels and masks over the picture. |
| LidarMapView, CompassView, JoystickView, WaveformView | The LiDAR map, the heading dial, the analogue stick and the live audio trace. |
| CoverImageView | Artwork cropped to a focus rectangle behind the live picture. |

## 3.15  Build

| Item | Detail |
| --- | --- |
| Application id | com.falcon.robot |
| Version | 1.0 (version code 1) |
| SDK | compile 34, target 34, minimum 26 |
| Language level | Java 8 |
| Native | CMake, for the optional whisper.cpp engine; arm64-v8a and x86_64 |
| Output | falcon.ai.robot.apk |
| Uncompressed assets | tflite, bin, ort and wav, so they can be memory-mapped or opened as a file descriptor |

### 3.15.1  Dependencies

| Dependency | Version | Why |
| --- | --- | --- |
| androidx.activity:activity | 1.8.2 | Lifecycle owner and the permission request API |
| androidx.camera:camera-core, camera2, lifecycle, view | 1.3.4 | Preview and frame analysis |
| androidx.camera:camera-video | 1.3.4 | Recording the camera view |
| com.google.mlkit:face-detection | 16.1.6 | Face detection, bundled model |
| org.tensorflow:tensorflow-lite | 2.14.0 | MobileFaceNet and YOLO |
| ai.moonshine:moonshine-voice | 0.1.5 | Speech recognition; sets the minimum SDK |
| org.jetbrains.kotlin:kotlin-bom | 1.8.22 | Aligns the Kotlin runtime CameraX brings |

## 3.16  Error handling

| Situation | Behaviour |
| --- | --- |
| A model is missing or will not load | Said once through a message; the rest of the application carries on |
| The camera cannot be opened | The design artwork stays in the panel and the operator is told |
| A permission is refused | The feature stays off and says why; the camera prompt is not repeated on every screen change |
| The Bluetooth adapter is off | The Remote Control panel says so; Scan does nothing until it is on |
| The robot's service is absent | Reported in the log, and the link is dropped |
| A command cannot be delivered | Reported on the status line and in the log, naming the command |
| A recording fails | Reported; the picture and the models are left running |

## 3.17  Verification of the source

There is no device or SDK in the environment where this application is edited, so a static check is run instead of a compilation, before any build is offered for test. It checks that every resource a layout or a class names exists; that every view identifier a screen uses is declared by a layout that screen inflates; that no string is left unused; that the four translations carry the same names, the same list lengths and the same placeholders; that every XML file parses; and that every source file's braces and parentheses balance.

## 3.18  Sequences

### 3.18.1  Opening a screen that shows the picture

| # | Step |
| --- | --- |
| 1 | The activity inflates the shell and its page, and binds the service |
| 2 | The service is created if it was not running, and reads its switch states from the settings file |
| 3 | onRobotServiceReady: the page attaches its preview and asks for what is already known — the last faces, the last objects |
| 4 | The service updates its notification and binds the camera to what is now wanted |
| 5 | Frames arrive; results are posted to the page, which draws them |
| 6 | On leaving, the page detaches its preview; the service rebinds without it and releases the camera when nothing else wants it |

### 3.18.2  A spoken command

| # | Step |
| --- | --- |
| 1 | SpeechRecorder cuts an utterance when the speaker stops |
| 2 | The keyword model classifies it, or the speech model transcribes it |
| 3 | The transcript is matched against the operator's own phrases, then the sixteen rules |
| 4 | With no match: the pages are told Unknown Command and nothing is sent |
| 5 | With a match and no command: the service switches face recognition, object detection or recording, reading the direction from what is running |
| 6 | With a match and a command: RobotSession.send is called with exactly the string the button for that action sends |
| 7 | The acknowledgement sound is played |
| 8 | onTranscript reaches every page: the Robot Control screen animates the figure, lights the button, takes over the repeat for a direction, and reports where the command went |

### 3.18.3  Scanning and connecting

| # | Step |
| --- | --- |
| 1 | Scan is pressed; the adapter and the permissions are checked |
| 2 | A low-latency scan starts; named devices are collected and sorted by signal |
| 3 | The scan stops after twelve seconds, or when it is pressed again |
| 4 | A device is tapped: any earlier link is dropped and connectGatt is called |
| 5 | On connection, a 185-byte transmission unit is requested |
| 6 | Services are discovered; without the serial service the operator is told and the link is dropped |
| 7 | Notifications are enabled and the descriptor is written |
| 8 | The descriptor's completion reports the link connected; the session is handed the link and every screen shows it |

### 3.18.4  A recording

| # | Step |
| --- | --- |
| 1 | The red button asks the service to record |
| 2 | The camera is rebound with the video use case added |
| 3 | A recording is prepared to a new file and started; the pages are told |
| 4 | The service stays in the foreground with the camera type while it runs |
| 5 | Stopping, or a camera rebind, ends the recording |
| 6 | The finalize event closes the file, names it, rebinds the camera without video and tells the pages |

## 3.19  Source inventory

| File | Responsibility |
| --- | --- |
| BaseActivity.java | The shell, navigation, the connection dialog, shared helpers |
| SplashActivity.java | The opening screen |
| MainActivity.java | Home: the cards and the tablet's status |
| RobotControlActivity.java | The main screen: panels, sidebar, latch, log, recording, reporting |
| RemoteControlActivity.java | The robot camera, status, map, and the Bluetooth panel |
| FaceRecognitionActivity.java | Detection, identification, the register, history |
| ObjectDetectionActivity.java | Detection, overlays, settings, statistics |
| VoiceRecognitionActivity.java | Listening, results, the command list, speech settings |
| LidarSlamActivity.java | Point cloud, map, SLAM controls and settings |
| SettingsActivity.java | Connection, language, camera, about |
| RobotService.java | The camera, the microphone, the models, the recorder, the orders the tablet carries out |
| RobotSession.java | Connection state and which transport a command takes |
| ble/BleLink.java | Scanning, GATT, notifications, the write queue |
| voice/VoiceCommands.java | The sixteen actions, the matching rules, the switches |
| voice/MoonshineEngine.java | Speech models, one per language |
| voice/WhisperEngine.java | The optional native speech engine |
| voice/CommandRecognizer.java | Keyword spotting, when a model is present |
| voice/SpeechRecorder.java | The microphone, pre-roll and end-of-speech |
| voice/CustomPhrases.java | The operator's own phrases for an action |
| voice/MelFeatures.java | Features for the keyword model |
| face/FaceAnalyzer.java | Detection and tracking of faces in a frame |
| face/FaceEmbedder.java | MobileFaceNet embeddings |
| face/FaceDatabase.java | The register on disk |
| detect/YoloSegmenter.java | The object model |
| detect/ObjectTracker.java | Identity between frames |
| detect/CocoLabels.java | The eighty class names |
| widget/ | The figure, the overlays, the map, the compass, the stick, the waveform, the scrolling containers |
| GamepadController.java | Directions from a connected gamepad |
| LocaleHelper.java | The interface language |

## 3.20  Class reference

What each of the classes that other classes depend on offers, and what a caller may assume about it. Private helpers are not listed; if it is not here, nothing outside the class should be calling it.

### 3.20.1  RobotService

The one long-lived component. It owns the camera, the microphone, the models and the recorder, and it outlives every screen. A screen binds to it, adds a listener, and asks it for what it needs; a screen never opens a camera or a model itself.

| Member | What it does |
| --- | --- |
| LocalBinder.getService() | How a bound screen reaches the service |
| intent(Context) | The intent that starts it, used by the screens and the notification |
| ACTION_STOP | The intent action the notification's stop button sends |
| Listener / Adapter | The callback interface, and an empty implementation so a screen overrides only what it wants |
| addListener / removeListener | Registration; every callback is delivered on the main thread |
| getLifecycle() | The service is its own lifecycle owner, which is what lets CameraX bind to it rather than to a screen |
| isFaceEnabled / setFaceEnabled | The face switch, remembered across restarts |
| isDetectionEnabled / setDetectionEnabled | The object switch, likewise |
| isVoiceEnabled / setVoiceEnabled | The listening switch, likewise |
| attachPreview / detachPreview | A screen offers its surface; the service binds the preview to it and rebinds when the screen goes |
| getAnalysisSize / setAnalysisSize | The frame size the models are fed |
| getLensFacing / setLensFacing | Front or rear, remembered |
| getFaceDatabase / getFaceAnalyzer | The register and the detector, for the Face screen |
| getLastFaces | The most recent frame's faces, so a screen that opens mid-stream has something to draw |
| startRegistration(samples, timeoutMs) | Collect embeddings for an enrolment and report the average with a crop |
| getDetectionAnalyzer / getLastObjects | The object detector and its last result |
| getDetectorModel / getDetectorError / isLoadingDetector | Which model, what went wrong, and whether it is still loading — the three things the AI status line says |
| loadDetector(modelName) | Swap the object model at runtime |
| getFrameWidth / getFrameHeight / getLastInferenceMs | What the overlays need to scale boxes, and what the status line reports |
| getEngine / getSpeechModel / isLoadingSpeechModel / isTranscribing | The speech engine and its state |
| loadSpeechModel(modelName) | Load a named model; used when the recognition language changes |
| savedLanguage(Context) | The recognition language, readable before the service exists |
| setLanguage(code) | Change it, which loads the matching model |
| setCommandControl / setNoiseSuppression | The two switches the Voice screen offers |
| getCustomPhrases | Phrases the operator added, matched alongside the built-in ones |
| runHere(Action) | Carry out one of the three tablet-side actions — Face Recognize, Object Detect, Recording — from the service's own state, and report what happened |
| acknowledge() | Play assets/ok.wav, falling back to spoken acknowledgement if the file cannot be played |
| speak(text) | Say something through the text-to-speech engine |
| setRecording(on) / startRecording / stopRecording / isRecording | The tablet's own video recording |
| recordings() / recordingDir() | The files, newest first, and where they live |

Two rules hold the service together. Every callback is posted to the main thread before it leaves the service, so no listener has to think about which thread it is on. And every change to what the camera is doing goes through the one binding routine, so the preview, the analysis and the video use case are always bound as a set.

### 3.20.2  BaseActivity

What every screen inherits. A screen that needs one of these does not write it again; a screen that writes it again is a defect, because the two copies drift.

| Member | What it does |
| --- | --- |
| setPage(layout, navId, title) | Inflate the screen into the shell, mark its entry in the sidebar, set the title |
| navigate(navId) | Open another screen without stacking a second copy of it |
| setupColumns() / setupColumns(id) | Read the weight from each panel's tag and lay the rows out; stack them on a narrow screen |
| bindRobotService(listener) | Bind to the service and add this screen's listener |
| getRobotService() / onRobotServiceReady(service) | The service once it is there, and the hook for work that needs it |
| sendCommand(command) | Send one line to the robot and say whether it went |
| commandFor(keyCode) | The command a gamepad button stands for |
| onGamepadDirection / onGamepadPress | What a screen does with a gamepad, if it wants one |
| showConnectionDialog() | The address dialog, from the connection indicator on any screen |
| connectionLabel() | How the current link should be described, Bluetooth or demonstration |
| bindConnectionStatus(view) | Keep a status view in step with the link |
| onConnectionChanged() | The hook a screen overrides to react to the link changing |
| cameraPermissionAsked() / noteCameraPermissionAsked() | The application-wide flag that stops the camera prompt reappearing on every screen change |
| ensureNotificationPermission() | Ask for the notification permission where the platform needs it |
| readBatteryPercent() / activeNetworkType() | What the system status row shows |
| appendLog(list, scroll, colour, ...) | Add one timestamped line to a log panel and keep it scrolled |
| LOG_INFO / LOG_OK / LOG_WARN / LOG_ERROR | The four dot colours a log line can have |
| setStatus(view, text, dot) | A status line with its coloured dot |
| setIcon(view, drawable, sizeDp, colour, gravity) | Put a sized, tinted icon on a label, which is how nearly every button in the application is drawn |
| setupRichHeader / refreshRichHeader / tintTopIcon | The page header some screens use |
| bindSpeed(seekId, valueId) | A slider and its readout, kept in step |
| color(res) / toast(...) | Small helpers, so a screen does not reach for the context |
| createPulse / resetPulse / startScan | The shared animations: the microphone ring, and the scanning line |

### 3.20.3  BleLink

The Bluetooth client, and the only class in the application that knows what a GATT is. It is a singleton because the link belongs to the application rather than to any screen, and because a second connection attempt from a second screen is a bug, not a feature.

| Member | What it does |
| --- | --- |
| get() | The one instance |
| SERVICE / WRITE / NOTIFY | The serial service and its two characteristics; change these if the firmware uses its own |
| SCAN_MS | How long a scan runs before it stops itself: 12 000 ms |
| State | IDLE, SCANNING, CONNECTING, CONNECTED, and the off state |
| Found | One discovered device: address, name and the signal strength, which is updated as further advertisements arrive |
| Listener / addListener / removeListener | State changes, the device list, and incoming text |
| getState / getDeviceName / getDeviceAddress / isConnected | What the screens display |
| devices() | The devices found so far, strongest signal first |
| missingPermissions(Context) | Which permissions this platform version still needs, which differs between Android 11 and 12 |
| isReady(Context) | Whether a scan could start at all: adapter present, radio on, permissions granted |
| startScan(Context) / stopScan() | Begin and end a scan |
| connect(Context, address, name) / disconnect() | Make and drop the link |
| send(command) | Queue one line for writing, split to the negotiated transmission unit |

Three details in this class are worth knowing before changing it. The state does not become CONNECTED when the GATT connects, nor when the service is discovered, but only when the notification descriptor has been written — before that, the robot can be written to but cannot answer. The write type is taken from what the characteristic says it supports, rather than assumed. And a write the platform refuses is left at the head of the queue and tried again when the link is next free, rather than being dropped or blocking everything behind it.

### 3.20.4  RobotSession

| Member | What it does |
| --- | --- |
| get() | The one instance, held for the life of the application |
| isConnected() | Whether anything is connected, Bluetooth or the demonstration link |
| isLive() | Whether that connection is a real robot; this is what separates a delivered command from a demonstrated one |
| setConnected(connected) | The demonstration link, from the address dialog |
| setBleConnected() | Called when the Bluetooth link reaches CONNECTED |
| getHost / getPort / setAddress | The address the dialog holds |
| isLightsOn / setLightsOn | One robot switch that several screens share |
| send(command) | Write the line if there is a live link; otherwise report that it was not delivered |

### 3.20.5  VoiceCommands

| Member | What it does |
| --- | --- |
| Action | One of the sixteen: its recogniser name, its label and description resources, the command it sends, and whether it is a switch |
| FOLLOW / FACE / OBJECT / RECORDING | The names of the four switches, so that other classes refer to them by constant rather than by spelling |
| match(transcript) | The first rule whose words are present, or nothing |
| commandFor(action) | The line to send: the plain command for a one-off, or the command with START or STOP appended for a switch, flipping it as it goes |
| isSwitchedOn(name) / setSwitchedOn(name, on) | The switch states the buttons and the spoken orders share |
| actions() | Every action, in recogniser order, which is what the sidebar and the Voice screen list are built from |
| actionByName(name) | The action a recogniser label stands for |
| examplePhrase(action, language) | The phrase the Voice screen offers as an example, in the language being spoken |

The rules are declared in one static block at the top of the class, in the recogniser's order, each with its English keywords, its exclusions and its Chinese, Japanese and Korean phrases. Adding a command is adding one entry there; nothing else in the application enumerates the sixteen.

### 3.20.6  The custom views

| View | What it draws |
| --- | --- |
| Robot3DView, RobotMesh, RobotPose | The articulated figure on the Robot Control screen, and the pose it is holding |
| FaceOverlayView | Boxes and names over the camera picture |
| TrackingOverlayView | Object boxes, labels and masks over the same picture |
| WaveformView | The live microphone level while listening |
| LidarMapView | The 2D map, its path and its markers |
| RadarSweepView | The LiDAR sweep indicator |
| CompassView | The robot's heading |
| DonutChartView | The recognition statistics on the Face screen |
| DPadView, JoystickView | The direction pad and the analogue stick |
| PanelScrollView, FitScrollView | Scrolling that behaves inside a panel rather than fighting the page |
| AspectImageView, CoverImageView | Artwork that keeps its shape in a panel of any width |

## 3.21  State machines

Three pieces of the application are state machines, and are written here as tables because that is how they are tested.

### 3.21.1  The link

| From | Event | To | Also |
| --- | --- | --- | --- |
| IDLE | startScan with permissions and the radio on | SCANNING | The device list is emptied |
| SCANNING | A named advertisement | SCANNING | The device is added, or its signal strength updated |
| SCANNING | 12 s elapse, or stopScan | IDLE | The button returns to Scan |
| SCANNING or IDLE | connect(address) | CONNECTING | Any running scan is stopped |
| CONNECTING | GATT connected | CONNECTING | A larger transmission unit is requested, then services are discovered |
| CONNECTING | Serial service found | CONNECTING | Notifications are enabled on the notify characteristic |
| CONNECTING | Notification descriptor written | CONNECTED | The session is told, every screen updates, the queue starts |
| CONNECTING | No serial service on the device | IDLE | The device is reported as unsuitable and disconnected |
| CONNECTED | disconnect, or the link drops | IDLE | The session is told; every screen shows offline; the queue is discarded |
| Any | The radio is turned off | IDLE | The state says Bluetooth is off |

### 3.21.2  The movement latch

| From | Event | To | Also |
| --- | --- | --- | --- |
| Stopped | A direction button pressed | Held on that direction | Its command is sent at once, the button is marked, and the repeat begins |
| Held | 1000 ms elapse | Held | The same command is sent again |
| Held | The same button pressed again | Stopped | STOP is sent and the mark cleared |
| Held | A different direction pressed | Held on the new direction | The old mark is cleared, the new command sent, the repeat restarted |
| Held | Stop pressed | Stopped | STOP is sent |
| Held | A spoken movement order | Held on that direction | The button follows what was said, so the screen and the robot agree |
| Held | The link drops, or the screen closes | Stopped | The repeat is cancelled; the robot's own timeout is what stops the machine |

### 3.21.3  Recording

| From | Event | To | Also |
| --- | --- | --- | --- |
| Not recording | Record pressed, or Recording spoken | Recording | The video use case is bound, a file is opened, the button becomes a red square |
| Recording | Record pressed again, or Recording spoken again | Not recording | The file is closed and appears in the Open list |
| Recording | The operator moves to another screen | Recording | The service holds it; the notification says so |
| Not recording | The video use case cannot bind | Not recording | The picture is kept and the refusal is explained |

## 3.22  What is stored, and under what name

Everything the application remembers is in one preferences file called settings, or in its own folder. There is no database.

| Key or path | Holds | Written by |
| --- | --- | --- |
| face_enabled | The face switch | The service |
| detection_enabled | The object switch | The service |
| voice_enabled | The listening switch | The service |
| voice_language | The language the operator speaks | The Voice screen and Settings |
| language | The language the interface is drawn in | Settings |
| lens_facing | Front or rear camera | The camera dialog |
| analysis_width, analysis_height | The frame size the models are fed | As above |
| camera_permission_asked | That the camera prompt has been shown once | Any screen that wants the picture |
| files/faces/faces.json | The face register: records and embeddings | The face database |
| files/faces/ | The enrolment photographs | As above |
| Android/data/…/files/Movies/ | Recordings, named with the date and time | The recorder |

## 3.23  Lifecycle

Where the work happens as a screen opens, closes and returns, and what is deliberately not done at each point.

| Moment | What happens | What does not |
| --- | --- | --- |
| Application starts | The splash screen; the locale is applied before any layout is inflated | The service is not started until something needs it |
| A screen opens | The layout is inflated into the shell, the sidebar marked, the service bound, a listener added | No camera or model is opened by the screen itself |
| The service arrives | The screen is told, and draws whatever the service already has — the last faces, the last objects, the current switches | Nothing is reloaded |
| A screen wants the picture | It offers its surface; the service binds the preview to it | The camera is not reopened |
| A screen closes | Its listener is removed and its surface detached; the service rebinds without it | The camera, the models and any recording are not stopped |
| Another screen opens | As above, in the other order, so the picture moves with the operator | Nothing is restarted |
| The application is left | The service keeps whatever is switched on, with its notification | The link is not dropped |
| Everything is switched off | The service stops itself | — |
| The notification's stop is pressed | Everything is switched off and the service stops | — |

## 3.24  Adding to the application

The three changes most likely to be wanted, and everything each of them touches. If a change needs more files than these, it is probably in the wrong place.

| Change | Files | Also |
| --- | --- | --- |
| A new spoken command | One rule in VoiceCommands; one button in activity_robot_control.xml; label and description strings in all four languages | One test case; the recogniser has to be able to produce the label |
| A new interface language | One values-xx folder of strings | The static check lists what is missing; nothing in the code changes |
| A new recognition language | The model in the assets, one entry in the language list and its code array, and phrases on each rule | The arrays are selected by position, so all four languages get the entry |
| A new screen | One activity extending BaseActivity, one layout, one sidebar entry, one entry in the manifest | setPage and setupColumns do the rest |
| A different robot vocabulary | The command strings in VoiceCommands, and the other commands listed in Document 2, section 2.4.2 | Nothing else in the application spells a command |
| Different service identifiers | The three constants at the top of BleLink | Nothing else knows them |
| A different object model | The asset, and the model name passed to loadDetector | The labels file has to match the model |

## 3.25  Where the design gives something up

Every one of these is a decision rather than an oversight, and each is recorded so that it is not rediscovered as a defect.

| Decision | What is given up | Why |
| --- | --- | --- |
| Framework widgets only, no AppCompat | Some newer widget behaviour, and some styling shortcuts | A smaller package, fewer dependencies, and no theme fighting on a device that is already fixed |
| One camera, owned by the service | A screen cannot have a second, differently configured camera at the same time | Two screens opening the camera is the commonest cause of a black picture |
| Recording without sound | No audio on the recordings | The microphone stays free, so listening continues while recording |
| Bluetooth serial rather than a network transport | Range, and bandwidth: no video from the robot over this link | It is what the robot offers, and it needs no infrastructure in the room |
| The demonstration link kept | It can be mistaken for a real connection | It is how the screens are shown and tested without a robot; every command it carries says plainly that no robot was reached |
| Simulated telemetry where the robot sends none | Battery, temperature and position are not real until the firmware sends them | The panels have to show something; where it is simulated, the specification says so |
| The face register on the tablet only | A replaced tablet starts empty | No server, no network, nothing to secure off the device |
| Switch states on the tablet | The tablet's idea of following can differ from the robot's if the robot restarts | The alternative is asking the robot, which it has no way to answer |
| Keyword matching rather than grammar | Some odd phrasings match nothing | Small recognition errors still find the right action, which matters more in a noisy room |

## 3.26  Size of the thing

For anyone estimating a change, the shape of what is there at this revision.

| Part | Count |
| --- | --- |
| Java source files | 46, about 14 300 lines |
| Screens | 8 activities, plus the splash |
| Layout files | One per screen, plus headers, dialogs and row layouts |
| Custom views | 16 classes in the widget package |
| Strings | 447 in English; 443 in each of the other three, plus 4 language names shown in their own language |
| String arrays | 22 in each language |
| Drawables | 122, nearly all vector |
| Styles | 44 |
| Spoken actions | 16 |
| Sidebar action buttons | 19 |

# Document 4  Screen Design Specification

This document fixes what each screen contains, where it sits and how it behaves when the screen is a different size. Measurements are in density-independent pixels (dp) and are the values in the layout and dimension resources, not sketches.

## 4.1  Common rules

### 4.1.1  Frame

| Element | Rule |
| --- | --- |
| Window | Full screen; the system bars are hidden and return on a swipe |
| Navigation sidebar | Always on the left: Home, Robot Control, Face Recognition, Voice Recognition, Object Detection, LiDAR SLAM, Remote Control, Settings. 100 dp wide on a tablet, 78 dp below that; the open screen is marked |
| Page area | To the right of the sidebar, padded by 18 dp on a tablet and 12 dp otherwise |
| Panel | Rounded dark surface, 12 dp of padding, a title row and a divider |
| Panel title | 26 dp tall, 14 sp semibold, a 16 dp icon, 8 dp between icon and text |
| Gap between panels | 14 dp on a tablet, 10 dp otherwise |
| Rows of panels | Weighted by each panel's tag attribute; side by side above 840 dp, stacked below 600 dp |

### 4.1.2  Palette and type

| Role | Colour | Used for |
| --- | --- | --- |
| Background | #060A14 | The window behind everything |
| Surface | #0D172B | Panels, status bar |
| Divider | #17233A | The line under a title |
| Primary text | #E8EEF8 | Values, button labels |
| Secondary text | #8A97AE | Field names, hints |
| Muted text | #5D6A82 | Empty states |
| Blue / light blue | #2F80FF / #55A6FF | Selection, links, log titles |
| Teal | #1ED9A4 | Connected, running, good |
| Cyan | #22D3EE | Camera and AI accents |
| Amber | #FFB547 | Working, warning |
| Red | #FF4D5E | Recording, failure, shutdown |

| Text | Size |
| --- | --- |
| Panel title | 14 sp semibold |
| Field label | 12 sp secondary |
| Value | 13 sp primary |
| Sidebar button | 11 sp primary, 32 dp tall |
| Log line | 11 sp |
| Status bar | 12 sp light blue, 40 dp tall |

## 4.2  Robot Control (the main screen)

The working screen. It has no page header: the space goes to the panels. Two rows of panels, a sidebar of actions down the right, and a status line along the foot; the sidebar and the status line are outside the scrolling area, so they stay in reach wherever the page is scrolled and even with a panel full screen.

+--------------------------------------------------------------+--------+

| Robot Status            | Camera & AI                        | Actions|

| 3D figure, mode, speed  | live picture, overlays             |  AI    |

| battery, temperature    | record / open  camera chooser      |  Move  |

| position X Y Z          | one line: model, time, listening   |  Arm   |

+-------------------------+----------------+-------------------+  Pose  |

| LiDAR                   | Camera Settings| Log               |  Quick |

| map, load map, full     | brightness ... | start/stop, lines |        |

+--------------------------------------------------------------+--------+

| status line: the most recent activity, with the time                   |

+------------------------------------------------------------------------+

### 4.2.1  Panels

| Panel | Weight | Width on a 1280 × 800 dp tablet | Contents |
| --- | --- | --- | --- |
| Robot Status | 1 | 498 dp | 3D figure; mode, speed, battery, temperature; estimated position X, Y, Z; full-screen and collapse controls |
| Camera & AI | 1 | 498 dp | Live picture with face and object overlays; record and Open buttons top right; camera chooser bottom right; resolution bottom left; one status line underneath |
| LiDAR | matched | 498 dp, pinned to the width of Robot Status | Map view; Load Map and full screen in the title row; the title opens the LiDAR screen |
| Camera Settings | 1.15 | 248 dp | Brightness, contrast, saturation, resolution and frame rate; scrolls by dragging |
| Log | 1.1 | 237 dp | Start/stop button in the title row; timestamped lines |
| Actions sidebar | fixed | 115 dp (84 dp below 600 dp) | Nineteen buttons in five groups |

### 4.2.2  The actions sidebar

One button per row, 32 dp tall, icon at the left with 8 dp before it and 6 dp after, label at 11 sp. The list scrolls within the panel. Every spoken command has a button here.

| Group | Buttons | What each one does |
| --- | --- | --- |
| AI | Face, Objects, Voice | Switch face recognition, object detection and listening |
| Move Control | Forward, Backward, Turn Left, Turn Right, Stop | Latch a direction (repeated every second) or stop |
| Arm Control | Arm Left, Arm Right | Select an arm |
| Preset Pose | Wave, Hello, T-Pose, Dance | One-off poses |
| Quick Actions | Follow me, Recording, No, Go Home, Shutdown | Following switch, tablet recording, cancel, return to base, power down (confirmed) |

### 4.2.3  Camera view controls

| Control | Position | Behaviour |
| --- | --- | --- |
| Record | Top right, red disc in a white ring, 34 dp | Starts recording; becomes a red square while recording |
| Open | Top right, left of Record, 34 dp | Lists the recordings, newest first; one tap plays |
| Camera chooser | Bottom right | Front, rear or a robot camera |
| Resolution | Bottom left | The frame size in use |
| Full screen | Title row | The panel fills the page; pressed again, it returns |

### 4.2.4  Status line

Always the last thing that happened, with the time: a recognition result, or the fate of a command — delivered to the robot, sent to the demonstration link only, or not sent at all. With the log running, the same lines accumulate in the Log panel.

## 4.3  Remote Control

The screen where the robot link is made, and where the robot's own camera and recording are worked.

| Row | Panels |
| --- | --- |
| Header | Page title and subtitle, with the connection indicator |
| Top | Camera View (robot camera, source tabs, photo, switch, full screen) ¦ Robot Status (mode, speed, battery, temperature, connection, estimated position, compass) ¦ Map View (2D and 3D tabs, zoom, legend) |
| Bottom | BLE Connection ¦ a column of Camera & Recording above the Operation Log |

### 4.3.1  The BLE Connection panel

| Control | Behaviour |
| --- | --- |
| State line | Not connected / Scanning / Connecting to <name> / Connected · <name> / Bluetooth is off, with a coloured dot |
| Scan | Starts a twelve-second scan; becomes Stop Scan while it runs |
| Disconnect | Drops the link; greyed out when there is none |
| Devices Found | A row per device: name, then address and signal strength. The row for the connected device is marked. A tap connects |

## 4.4  Face Recognition

| Panel | Contents |
| --- | --- |
| Live camera | Source and resolution in the title row; picture with boxes; capture, rescan, source and full-screen tools |
| Recognition result | Photograph, name, identifier, similarity with a bar, department, position, access time; Open Door and Add to Log |
| Face database | Search, add, and a row per record with photograph, name, identifier, department and an active mark; long press removes |
| History | Recognitions with time and result |
| Statistics | Counts for the day |
| Settings | Detection switch, boxes, labels, threshold, camera |

## 4.5  Voice Recognition

| Panel | Contents |
| --- | --- |
| Listening | Microphone button, live waveform at ninety per cent of the panel width, Speak Now prompt, listening state |
| Recognition result | Transcript, language, time, confidence; the matched command with its icon and description, or Unknown Command |
| Voice commands | Every command with an example phrase and a built-in mark; a tap runs it as though it had been spoken; search |
| Voice control | Command control, continuous listening and noise suppression |
| History | What was said, when, and what came of it |
| Advanced settings | Language tab (recognition language, auto detect, punctuation), recognition tab (sensitivity, timeout, confirmation sound), audio tab (noise suppression, echo cancellation, gain, output volume), and Apply |

## 4.6  Object Detection

| Panel | Contents |
| --- | --- |
| Live camera | Picture with boxes, labels and masks; source chooser; full screen |
| Detection result | What is in frame with counts, and the inference time |
| Detection settings | Detection switch, boxes, labels, masks, tracking, confidence threshold |
| Detected objects | A row per tracked object with its class and confidence |

## 4.7  LiDAR SLAM

| Panel | Contents |
| --- | --- |
| Point cloud | 3D, top and map tabs; layers, centre, snapshot and full-screen tools |
| 2D map | Zoom in, out and fit; a legend for robot, path, target and obstacle |
| SLAM status | Mapping, localisation, navigation and map quality |
| Robot pose | X, Y, yaw and a compass |
| LiDAR | Range, points per scan and a sweep indicator |
| Control | Start and stop mapping, Save Map, Load Map, automatic navigation, follow, navigation target |
| Settings | Range, resolution, mapping mode, update rate and Apply |
| Log | Timestamped lines, clearable |

## 4.8  Home and Settings

| Screen | Contents |
| --- | --- |
| Home | A card per screen with its name and one line about it; the system status row: connection, Wi-Fi, battery and time |
| Settings | Connection (address and state, opens the connection dialog), Interface language, Camera (lens and analysis frame size), System information |

## 4.9  Behaviour when the screen is a different size

| Width | Layout |
| --- | --- |
| 840 dp and wider | Panels side by side in weighted rows; the tablet dimension set: 18 dp padding, 14 dp gaps, 100 dp navigation, 115 dp action sidebar |
| 600 dp to 840 dp | Tablet dimensions, but rows stack |
| Below 600 dp | Compact dimensions: 12 dp padding, 10 dp gaps, 78 dp navigation, 84 dp action sidebar; rows stack and the page scrolls |

## 4.10  States a screen must be able to show

| State | How it appears |
| --- | --- |
| No robot | Offline in the status row; values show a dash; a command reports that it was not sent |
| Demonstration link | Connected, with the command line saying plainly that nothing reached the robot |
| No camera permission | The design artwork stays in the panel, with a line saying why |
| Model loading | The AI line says which model is loading |
| Recording | The record button is a red square; the status line says Recording |
| Nothing recognised | Unknown Command; what was heard is not shown |
| Panel full screen | One panel fills the page; the sidebar and status line remain |

## 4.11  Control inventory: Robot Control

Every control on the main screen, by the identifier it carries in the layout. This is the list a tester or a maintainer works from.

| Identifier | Type | Behaviour |
| --- | --- | --- |
| robot_fullscreen / robot_collapse | Icon buttons, 26 dp | Fill the page with Robot Status; fold the panel to its title |
| camera_fullscreen | Icon button, 26 dp | Fill the page with the camera panel |
| camera_record | Icon button, 34 dp | Start and stop recording; red disc, red square while recording |
| camera_open | Icon button, 34 dp | List the recordings and play one |
| camera_source | Chip on the picture | Choose the camera |
| camera_info | Label on the picture | The frame size in use |
| ai_status | Line under the picture | Model, inference time, listening state |
| lidar_load_map / lidar_fullscreen | Icon buttons, 26 dp | Load a saved map; fill the page with the LiDAR panel |
| lidar_title | Panel title | Opens the LiDAR SLAM screen |
| brightness / contrast / saturation | Sliders | Sent as one camera command |
| camera_resolution / camera_fps | Dropdowns | Frame size and frame rate |
| btn_log | Button in the log title row | Start and stop the log |
| toggle_face / toggle_object / toggle_voice | Sidebar buttons | Switch the two models and listening |
| move_forward / move_backward / move_left / move_right | Sidebar buttons | Latch a direction, repeated every second |
| move_stop | Sidebar button | Stop and release the latch |
| arm_left / arm_right | Sidebar buttons | Select an arm |
| pose_wave / pose_hello / pose_tpose / pose_dance | Sidebar buttons | One-off poses |
| qa_follow | Sidebar button | Following switch, shared with the spoken order |
| qa_recording | Sidebar button | The tablet's recording, same as the red button |
| qa_no | Sidebar button | Cancel; releases the latch |
| qa_home / qa_shutdown | Sidebar buttons | Return to base; power down after confirmation |
| robot_tip | Status line | The last activity, with the time |

## 4.12  Navigation

Splash  ->  Home

|

+--> Robot Control  --(LiDAR panel title)-->  LiDAR SLAM

+--> Face Recognition

+--> Voice Recognition

+--> Object Detection

+--> LiDAR SLAM

+--> Remote Control   (the Bluetooth link is made here)

+--> Settings

The sidebar is on every screen, so any screen reaches any other in one press.

The connection dialog opens from the status indicator on every screen.

## 4.13  Panel states

| State | Entered by | Left by |
| --- | --- | --- |
| Normal | Default | — |
| Full screen | The panel's full-screen button | The same button; the sidebar and status line stay in reach |
| Collapsed | The Robot Status collapse button | The same button |
| Scrolled | Dragging inside a panel that holds more than fits | Dragging back; the page underneath never steals the drag |

## 4.14  How the inventories are to be read

Section 4.11 lists the controls of the Robot Control screen. The sections that follow do the same for the other screens. Each row is one control that has an identifier in the layout: the identifier is what the screen looks up, and the static check of Document 3, section 3.17 refuses a build in which a screen looks up something its own layout does not have. A control with no identifier — a label that never changes, a divider, a spacer — is not listed.

## 4.15  Face Recognition

Six panels: the camera down the left, the result beside it, the register below, and a row of history, statistics and settings along the foot.

### 4.15.1  Live camera

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_camera | Panel | The whole camera panel |
| feed_dot | Indicator | Whether the picture is live |
| feed_source | Label | Which camera is in use |
| feed_resolution | Label | The frame size |
| feed_fullscreen | Button | Fill the page with this panel, and return |
| feed_frame | Frame | Holds the picture and everything drawn over it |
| camera_preview | Preview | The picture itself |
| face_overlay | Overlay | Boxes and names over the picture |
| camera_message | Label | Why there is no picture, when there is none |
| tool_capture | Button | Take the frame for an enrolment |
| tool_rescan | Button | Look again |
| tool_source | Button | Change camera |
| tool_fullscreen | Button | As feed_fullscreen, from the tool strip |

### 4.15.2  Recognition result

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_result | Panel | The result panel |
| result_title | Title | Its title row |
| result_photo | Image | The photograph held for the person recognised |
| result_name | Value | Their name |
| result_id | Value | Their identifier |
| result_similarity | Value | How close the match was, as a percentage |
| result_similarity_bar | Bar | The same figure as a bar |
| result_department | Value | Their department |
| result_position | Value | Their position |
| result_access_time | Value | When they were recognised |
| btn_open_door | Button | The action a recognition is allowed to take |
| btn_add_log | Button | Put this recognition in the history |

### 4.15.3  Register, history and statistics

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_database | Panel | The face register |
| db_title | Title | Its title row |
| db_add | Button | Enrol a new face |
| db_search | Field | Filter the register |
| db_list | List | One row per record: photograph, name, identifier, department, and whether it is active; a long press removes it |
| history_title | Title | The history panel |
| btn_view_all | Button | The whole history rather than the recent part |
| history_list | List | Recognitions with time and result |
| stats_title | Title | The statistics panel |
| stats_donut | Chart | Recognised against unrecognised |
| stats_total | Value | How many faces were seen |
| stats_recognized, stats_recognized_pct | Values | How many were recognised, and the share |
| stats_unrecognized, stats_unrecognized_pct | Values | The other side of the same figure |
| today_title | Title | Today's row |
| today_recognized_icon, today_recognized | Value | Recognised today |
| today_unrecognized | Value | Unrecognised today |

### 4.15.4  Settings

| Identifier | Kind | What it is |
| --- | --- | --- |
| settings_title | Title | The settings panel |
| icon_detection, sw_face_detection | Switch | Detect faces at all |
| icon_recognition, sw_face_recognition | Switch | Identify the faces that are detected |
| spinner_mode | Dropdown | Recognition mode |
| seek_threshold, value_threshold | Slider | How close a match has to be; the readout moves with the slider |
| spinner_database | Dropdown | Which register to use |
| spinner_camera | Dropdown | Which camera to use |

## 4.16  Voice Recognition

### 4.16.1  Listening

| Identifier | Kind | What it is |
| --- | --- | --- |
| listen_panel | Panel | The listening panel |
| voice_visual | Frame | Everything that reacts to sound |
| mic_hit | Button | The microphone: press to listen, press again to stop |
| speak_hint | Label | Speak Now, while it is listening |
| live_wave | Waveform | The live level, at ninety per cent of the panel width |
| listen_badge, listen_dot, listen_state | Indicator | Listening or not, with its dot |
| mic_selector, mic_selector_icon, mic_device | Chooser | Which microphone |

### 4.16.2  Result and action

| Identifier | Kind | What it is |
| --- | --- | --- |
| result_title | Title | The result panel |
| result_quote | Frame | The transcript, set apart |
| btn_speak | Button | Listen again |
| result_text | Value | What was heard |
| result_confidence, result_confidence_bar | Value and bar | How sure the engine was |
| result_language | Value | Which language it was recognised in |
| result_time | Value | When |
| action_title | Title | The matched action |
| action_icon, action_name, action_desc | Value | Which of the sixteen it was, and what it does; Unknown Command when nothing matched |
| btn_execute | Button | Carry it out |

### 4.16.3  Commands, control and history

| Identifier | Kind | What it is |
| --- | --- | --- |
| voice_db_title | Title | The command list |
| voice_search | Field | Filter it |
| voice_db_list | List | Every command with an example phrase and a built-in mark; a tap runs it as though it had been spoken |
| control_title, voice_toggles | Switches | Command control, continuous listening, noise suppression |
| history_title | Title | The history panel |
| btn_clear_history | Button | Empty it |
| voice_history_list | List | What was said, when, and what came of it |

### 4.16.4  Advanced settings

| Identifier | Kind | What it is |
| --- | --- | --- |
| advanced_title | Title | The advanced panel |
| adv_tab_language | Tab | Recognition language, automatic detection, punctuation |
| adv_tab_hotword | Tab | Sensitivity, timeout, confirmation sound |
| adv_tab_audio | Tab | Noise suppression, echo cancellation, gain, output volume |
| adv_rows | Rows | Whichever tab's settings are showing |
| btn_apply_settings | Button | Apply them |

## 4.17  Object Detection

| Identifier | Kind | What it is |
| --- | --- | --- |
| feed_panel | Panel | The camera panel |
| camera_preview | Preview | The picture |
| tracking_overlay | Overlay | Boxes, labels and masks |
| camera_message | Label | Why there is no picture, when there is none |
| feed_fullscreen | Button | Fill the page, and return |
| feed_live | Indicator | Whether the picture is live |
| feed_source | Chooser | Which camera |
| feed_info | Label | Frame size and inference time |
| panel_result | Panel | What is in frame |
| result_list | List | One row per class with its count |
| settings_title | Title | The settings panel |
| toggle_list | Switches | Detection, boxes, labels, masks, tracking |
| spinner_threshold | Dropdown | The confidence below which a detection is ignored |
| total_count, total_trend | Values | How many objects, and which way that is going |
| count_list | List | One row per tracked object with its class and confidence |

## 4.18  LiDAR SLAM

The widest screen in the application: a column of views on the left, a column of status and control on the right.

### 4.18.1  Point cloud and map

| Identifier | Kind | What it is |
| --- | --- | --- |
| view_frame, view_3d, view_alt, view_top | Views | The 3D cloud, the alternative rendering and the top view |
| view_live | Indicator | Whether scanning is live |
| tab_3d, tab_top, tab_map | Tabs | Which of the three is showing |
| tool_layers | Button | Which layers are drawn |
| tool_center | Button | Centre on the robot |
| tool_snapshot | Button | Keep a picture of the view |
| tool_fullscreen | Button | Fill the page, and return |
| map_title | Title | The 2D map panel |
| map_zoom_in, map_zoom_out, map_fit | Buttons | Zoom, and fit the map to the panel |
| map_frame, map_2d | View | The map itself |
| map_legend | Legend | Robot, path, target and obstacle |

### 4.18.2  Status

| Identifier | Kind | What it is |
| --- | --- | --- |
| slam_title | Title | The status panel |
| status_mapping | Value | Whether a map is being built |
| status_localization | Value | Whether the robot knows where it is |
| status_navigation | Value | Whether it is navigating |
| status_quality | Value | How good the map is |
| pose_title | Title | The pose panel |
| pose_x, pose_y, pose_yaw | Values | Where it is and which way it faces |
| pose_compass | Compass | The same heading, drawn |
| lidar_title | Title | The sensor panel |
| lidar_range | Value | How far it can see |
| radar_image, radar_sweep | View | The sweep indicator |
| lidar_points | Value | Points per scan |

### 4.18.3  Control, settings and log

| Identifier | Kind | What it is |
| --- | --- | --- |
| control_title | Title | The control panel |
| btn_start_mapping, btn_stop | Buttons | Begin and end mapping |
| btn_save_map, btn_load_map | Buttons | Keep a map, and open one |
| sw_auto_nav | Switch | Automatic navigation |
| sw_follow | Switch | Following, the same switch the spoken Follow me moves |
| target_title, btn_set_target | Button | Choose a navigation target |
| slam_dpad | Direction pad | Drive from this screen |
| drive_forward, drive_backward, drive_rotate_left, drive_rotate_right | Buttons | The four directions, sending the same commands as the Robot Control sidebar |
| settings_title | Title | The settings panel |
| seek_range, value_range | Slider | Sensor range |
| spinner_resolution | Dropdown | Map resolution |
| spinner_mapping_mode | Dropdown | Mapping mode |
| spinner_update_rate | Dropdown | How often the map is updated |
| btn_apply | Button | Send the settings to the robot |
| log_title, btn_clear_log, log_scroll, log_list | Log | Timestamped lines, and a way to empty them |

## 4.19  Remote Control

### 4.19.1  Camera and status

| Identifier | Kind | What it is |
| --- | --- | --- |
| panel_camera, camera_title | Panel | The robot's camera |
| cam_front, cam_rear, cam_left, cam_right | Tabs | Which of the robot's cameras |
| feed_frame, camera_feed | View | The picture |
| tool_photo | Button | Take a photograph |
| tool_switch | Button | Change camera |
| tool_fullscreen | Button | Fill the page, and return |
| camera_info | Label | What the picture is |
| status_title, status_online | Panel | Robot status, and whether it is there |
| value_mode, value_speed, value_battery, bar_battery, value_temperature | Values | Mode, speed, battery with its bar, temperature |
| value_connection | Value | How it is connected |
| pos_x, pos_y, pos_z | Values | Where it thinks it is |
| compass | Compass | Which way it faces |

### 4.19.2  Map, connection, recording and log

| Identifier | Kind | What it is |
| --- | --- | --- |
| map_title, map_2d_tab, map_3d_tab | Tabs | The map panel and its two views |
| map_frame, map_image, map_live | View | The map, and whether it is live |
| map_zoom_in, map_zoom_out, map_locate | Buttons | Zoom, and find the robot |
| legend_robot | Legend | What the marks mean |
| ble_title | Title | The Bluetooth Connection panel |
| ble_status | Value | Not connected, Scanning, Connecting, Connected with the device name, or Bluetooth is off, each with its dot |
| ble_scan | Button | Start a twelve-second scan; it reads Stop Scan while one runs |
| ble_disconnect | Button | Drop the link; disabled when there is none |
| ble_devices_label, ble_list | List | A row per device: name, then address and signal strength; the connected one is marked; a tap connects |
| recording_title | Title | The camera and recording panel |
| rec_photo, rec_video, rec_audio | Buttons | Ask the robot for a photograph, a video or an audio recording |
| rec_gallery | Button | What has been captured |
| log_title, btn_clear_log, log_scroll, log_list | Log | The operation log: what was sent, and whatever the robot sent back |

## 4.20  Home, Splash and Settings

| Screen | Identifier | What it is |
| --- | --- | --- |
| Splash | splash_image | The opening artwork |
| Splash | splash_status | What is being prepared |
| Splash | splash_progress, splash_percent | How far along |
| Splash | splash_version | Which build this is |
| Home | hero, hero_image | The header artwork |
| Home | home_system_status | The tablet's own status row |
| Home | status_wifi, home_network | Whether the tablet has a network |
| Home | status_battery, home_battery_bar | The tablet's battery |
| Home | quick_status | The robot's state at a glance |
| Home | home_grid | A card per screen, with one line about it |
| Home | home_scroll, columns | The page, and its rows of panels |
| Settings | settings_list | The four rows: connection, interface language, camera, system information |

Settings is one list rather than a set of panels because each of its rows opens a dialog: the connection dialog, the language chooser, the camera dialog and the about box. The rows show their current value as a subtitle, so the screen answers what it is set to without being opened further.

## 4.21  Style reference

Every repeated piece of the interface is a style, so that a change to how a button looks is one edit rather than two hundred. These are the ones a layout uses by name.

| Style | Height | Text | Used for |
| --- | --- | --- | --- |
| NavRailItem | wrap | 11 sp | An entry in the navigation sidebar, icon above label |
| Panel | wrap | — | A panel on the older screens, 14 dp of padding |
| RobotPanel | wrap | — | A panel on the reworked screens, 12 dp of padding |
| RobotPanelTitle | 26 dp | 14 sp | A panel title row, icon and text, 8 dp between them |
| SectionTitle | wrap | 15 sp | A heading inside a panel |
| PanelDivider | 1 dp | — | The line under a title |
| FieldLabel | wrap | 12 sp | The name of a value |
| StatusText | wrap | 13 sp | A value with a coloured dot before it |
| Input | 44 dp | 14 sp | A single-line field |
| Dropdown | 44 dp | — | A chooser, with its own popup background |
| Segment | 40 dp | 14 sp | One of a row of tabs |
| CameraTab | 34 dp | 13 sp | A camera chooser tab |
| ActionButton | 68 dp | 12 sp | A large button with the icon above the label |
| ControlButton | 52 dp minimum | 13 sp | A button in a row of equal buttons |
| ListControlButton | 40 dp minimum | 13 sp | A button in a column, icon at the left |
| QuickButton | 46 dp minimum | 14 sp | A quick action, icon at the left |
| SidebarButton | 32 dp | 11 sp | One of the nineteen action buttons: icon at the left with 8 dp before it and 6 dp after, label clipped rather than wrapped |
| ListAction | 48 dp | 14 sp | A row that does something when tapped |
| PrimaryButton | 46 dp | 14 sp | The main button of a panel |
| SettingLabel | wrap | — | A row in Settings |

## 4.22  Dimension reference

Two complete sets. The compact set is the default, so a device that reports nothing unusual gets the tighter layout; the tablet set replaces it from 600 dp of width upwards. Nothing in the code carries a size that is in this table.

| Dimension | Compact | Tablet (600 dp and wider) | What it sets |
| --- | --- | --- | --- |
| screen_padding | 12 dp | 18 dp | Around the page |
| gap | 10 dp | 14 dp | Between panels |
| sidebar_width | 78 dp | 100 dp | The navigation sidebar |
| action_sidebar_width | 84 dp | 115 dp | The actions sidebar on Robot Control |
| title_text | 18 sp | 20 sp | A page title |
| hero_min_height | 240 dp | 300 dp | The artwork on Home |
| dpad_size | 180 dp | 200 dp | The direction pad |
| joystick_size | 150 dp | 200 dp | The analogue stick |
| mic_ring_size | 190 dp | 230 dp | The microphone ring on the Voice screen |
| preview_height | 220 dp | 340 dp | A camera panel |
| robot_preview_height | 170 dp | 230 dp | The robot's camera on Remote Control |

One more resource decides the shape of a page: the boolean two_columns, which is true from 840 dp of width. Below that, rows of panels stack instead of sitting side by side, whatever weights they carry.

## 4.23  Colour reference

Twenty-four colours, and no others. A screen that wants a colour takes one of these; a screen that needs a new one adds it here, so that the palette stays countable.

| Name | Value | Used for |
| --- | --- | --- |
| bg | #060A14 | The window behind everything |
| bg_elevated | #0B1324 | A surface that sits above the background |
| surface | #0D172B | Panels |
| surface_stroke | #1C2A45 | The edge of a panel |
| divider | #17233A | The line under a title |
| blue | #2F80FF | Selection and the main accent |
| blue_light | #55A6FF | Links, log titles |
| blue_dim | 20% blue | A blue wash behind something |
| teal | #1ED9A4 | Connected, running, good |
| teal_dim | 20% teal | The same, as a wash |
| purple | #7B5CFF | A second accent |
| purple_light | #9A82FF | Its lighter form |
| purple_dim | 20% purple | Its wash |
| amber | #FFB547 | Working, warning |
| orange | #F59E0B | A stronger warning |
| red | #FF4D5E | Recording, failure, shutdown |
| cyan | #22D3EE | Camera and AI accents |
| white | #FFFFFF | Text on a coloured button |
| white_80 | 80% white | The same, softened |
| nav_label | #B4C0D6 | A sidebar label |
| text_primary | #E8EEF8 | Values, button labels |
| text_secondary | #8A97AE | Field names, hints |
| text_muted | #5D6A82 | Empty states |
| card_subtitle | #BFDDF7 | The second line of a Home card |

## 4.24  Icons

A hundred and twenty-two drawables, nearly all vector, so that they are sharp at any density and can be tinted by state rather than duplicated. They divide as follows.

| Group | How many | Examples |
| --- | --- | --- |
| Backgrounds and shapes | 34 | bg_panel_robot, bg_control_button, bg_feed_button, bg_sidebar, bg_tab, bg_input, bg_popup, bg_splash |
| Status dots | 4 | dot_teal, dot_amber, dot_red, dot_gray |
| Icons | 74 | Everything named ic_, from ic_home to ic_record_stop; the groups below are drawn from these |
| Navigation and screens | — | ic_home, ic_robot, ic_face_id, ic_mic, ic_cube, ic_lidar, ic_gamepad, ic_settings |
| Movement and pose | — | ic_arrow_up, ic_arrow_down, ic_rotate_left, ic_rotate_right, ic_square, ic_arm, ic_pose_wave, ic_tpose, ic_run, ic_follow |
| Camera and recording | — | ic_camera, ic_videocam, ic_photo, ic_record_dot, ic_record_stop, ic_folder_open, ic_play_white |
| Map and sensing | — | ic_map, ic_place, ic_navigation, ic_layers, ic_crosshair, ic_top_view, ic_scan |
| System and status | — | ic_bluetooth, ic_wifi, ic_battery_h, ic_thermometer, ic_chip, ic_bolt, ic_power, ic_shield, ic_warning |
| Editing and lists | — | ic_add, ic_remove, ic_edit, ic_save, ic_search, ic_refresh, ic_check, ic_chevron_down, ic_chevron_right, ic_dropdown, ic_history, ic_note |
| Camera settings | — | ic_brightness, ic_contrast, ic_saturation, ic_tune, ic_volume_up |
| Legends | 3 | legend_path, legend_target, legend_obstacle |
| Progress and parts | 7 | progress_battery, progress_similarity, progress_splash, progress_loading, seekbar_thumb, switch_thumb, switch_track |

Every icon that appears on a button is placed by the shared helper that sizes and tints it, rather than by a fixed drawable size in the layout. That is why the sidebar could be reduced from a taller button to 32 dp without redrawing anything: the icon follows the size it is given.

## 4.25  Reach and touch

The screen is a tablet held in two hands or set on a stand, and the operator is sometimes watching the robot rather than the screen. The layout is measured for that.

| Rule | Measurement | Why |
| --- | --- | --- |
| The smallest control | 32 dp tall, full panel width, with 8 dp of padding before the icon | A sidebar button: below this the label stops fitting and the target gets hard to hit |
| Ordinary buttons | 40 to 52 dp tall | Comfortable without wasting a panel |
| Stop | In the Move Control group, last, where the thumb falls | It is the control that has to be found without looking |
| Record | 34 dp, top right of the picture, red on white | Far from anything that changes the camera, so it is not pressed by accident |
| Full screen | In the title row, not over the picture | So it does not cover what it is meant to enlarge |
| Shutdown | Last in Quick Actions, and confirmed | The one action that cannot be undone from the tablet |
| The status line | Along the foot, outside the scrolling area | It is where the answer to “did that work” appears, so it must not scroll away |
| The actions sidebar | Outside the scrolling area, scrolling within itself | So a command is reachable wherever the page is scrolled, and even with a panel full screen |

## 4.26  Text that changes length

A label that is one word in English is often three in another language, and Korean, Japanese and Chinese all set at different widths. Where that matters, the layout is built so that the text gives way rather than the control.

| Place | What happens to long text |
| --- | --- |
| Sidebar buttons | One line, clipped with an ellipsis; the icon and the button keep their size |
| Panel titles | One line, clipped; the title row stays 26 dp whatever it says |
| Field labels and values | The label takes what it needs and the value takes the rest, because the rows are weighted rather than fixed |
| Dropdown entries | Wrap in the popup, which is as wide as the field |
| Log lines | Wrap, and the log scrolls |
| Status line | One line, clipped; the most recent thing is always at the left where it is read first |
| Home cards | The title on one line, the description on two |
| Dialog buttons | The dialog grows; nothing is clipped, because a confirmation that cannot be read is worse than an untidy dialog |

# Document 5  Test Case Specification

Ninety-seven test cases, each naming the requirement it verifies. A case passes only if the expected result is observed exactly; anything else is recorded as a failure with what was seen instead.

## 5.1  Test environment

| Item | What is needed |
| --- | --- |
| Tablet | The target device: Android 8.0 or newer, landscape, 1280 × 800 dp or larger, camera, microphone, Bluetooth Low Energy |
| Phone | Any Android 8.0 device, for the layout cases |
| Robot | Powered, advertising the serial service, with a console that shows what it receives |
| Second Bluetooth device | Any named device that does not offer the robot's service, for TC-018 |
| Room | Ordinary lighting and ordinary quiet; one person to be enrolled and recognised; a chair and a bottle for object detection |
| Build | falcon.ai.robot.apk from the build under test, freshly installed with the application's data cleared |

## 5.2  How to run a pass

Install the build and clear the application's data, so that permissions and switch states start from nothing.

Run the static check of Document 3, section 3.17 first; a build that fails it is not tested further.

Work through the groups in order. The link cases come early because most later cases need a connected robot.

Record, for each case, the build identifier, the date, the device, pass or fail, and for a failure what was seen.

Where a case cannot be run — no robot to hand, for instance — record it as blocked rather than passed.

## 5.3  Shell and navigation

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-001 | FR-001 | The application is installed | Open the application; open each of the eight entries in the navigation sidebar in turn | Each entry opens its screen; nothing crashes and nothing is blank |
| TC-002 | FR-002 | Any screen is open | Look at the sidebar | The entry for the open screen is marked and the others are not |
| TC-003 | FR-003 | Any screen is open | Look at the edges of the display; swipe in from the top edge | No system bars are shown; a swipe brings them back temporarily |
| TC-004 | FR-004 | The application is closed | Start the application | A splash screen appears, then the Home screen |
| TC-005 | FR-005 | Any screen is open | Find the connection indicator; tap it | The indicator shows the state; tapping opens the connection dialog |

## 5.4  The robot link

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-010 | FR-010 | Bluetooth is on, permissions granted, a named device is in range | Open Remote Control; press Scan | Devices appear within a few seconds, strongest signal first, each with address and dBm |
| TC-011 | FR-011 | A scan is running | Press Scan; wait without touching anything | The scan stops by itself after about twelve seconds and the button returns to Scan |
| TC-012 | FR-012 | The robot is powered and advertising | Tap the robot in the list | The state goes Connecting, then Connected with the device name; the log records it; the status indicator on every screen shows connected |
| TC-013 | FR-013 | Bluetooth permissions have never been granted | Press Scan | The platform prompt appears; refusing it leaves a line in the log saying the permission is needed, and no scan starts |
| TC-014 | FR-014 | Connected | Press Forward on the Robot Control sidebar; watch the robot's serial console | One line arrives, MOVE FORWARD, terminated by a newline |
| TC-015 | FR-015 | Connected to a robot whose characteristic offers only write-without-response | Press any action button | The command is delivered; the screen reports it as sent to the robot |
| TC-016 | FR-016, NFR-06 | Connected | Press five action buttons in quick succession | All five arrive, in order; no command is lost and the link does not wedge |
| TC-017 | FR-017 | Connected to a robot that sends text back | Have the robot send a line | The line appears in the Remote Control log, prefixed as coming from the robot |
| TC-018 | FR-018 | A Bluetooth device that is not the robot is in range | Connect to that device | The log says the device does not offer the robot service and the link is dropped |
| TC-019 | FR-019 | Connected | Power the robot off; look at the screens; press an action | Every screen shows offline; the action is reported as not sent |
| TC-020 | FR-020 | Connected | Leave Remote Control for another screen and come back | The robot is still connected and commands still arrive |
| TC-021 | FR-021 | Any screen | Open the connection dialog | The dialog states that the address is not a transport and that the robot is driven over Bluetooth from the Remote Control screen |

## 5.5  Robot Control screen

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-030 | FR-030 | Robot Control is open | Read the Robot Status panel | Mode, speed, battery, temperature and position X, Y, Z are shown; disconnected shows a dash rather than a stale value |
| TC-031 | FR-031 | Robot Control is open | Press Wave, then Forward | The 3D figure waves, then walks |
| TC-032 | FR-032 | Camera permission granted; a face and a chair are in view | Switch on Face and Objects | The picture is live; a box follows the face and boxes follow the objects |
| TC-033 | FR-033 | Robot Control is open | Tap the camera chooser on the picture | The other sources are offered and the choice takes effect |
| TC-034 | FR-034 | Robot Control is open | Drag inside the Camera Settings panel down to the frame rate; change it | The panel scrolls under the drag, the frame rate is reachable, and the change is sent |
| TC-035 | FR-035 | Robot Control is open on a tablet | Compare the LiDAR panel with the 3D view above it; press the LiDAR full-screen button | The two are the same width; full screen gives the LiDAR panel the whole page and the button returns it |
| TC-036 | FR-036 | Robot Control is open | Press Load Map in the LiDAR title row; choose a map | The saved maps are listed; choosing one sends SLAM LOAD_MAP with that name and the status line reports it |
| TC-037 | FR-037 | Robot Control is open | Press Start on the Log panel; speak a command; press an action button | Both are written down with the time; the command line says what became of it |
| TC-038 | FR-038 | Robot Control is open | Press any action | The status line along the foot shows it with the time |
| TC-039 | FR-039 | Robot Control is open | Press full screen on Robot Status, then again; repeat for Camera and LiDAR | Each panel fills the page in turn and the layout is restored each time; the sidebar and the status line stay in reach |
| TC-040 | FR-040 | Robot Control is open | Read the sidebar from top to bottom | Every one of the sixteen commands has a button, plus Voice, Go Home and Shutdown |
| TC-041 | FR-041 | Robot Control is open | Press Face, Objects and Voice in turn | Each switches its feature on and the button shows it; pressing again switches it off |
| TC-042 | FR-042 | Connected | Press Forward; wait five seconds; watch the robot | The robot keeps moving; MOVE FORWARD is repeated once a second |
| TC-043 | FR-043 | Forward is latched | Press Forward again; latch it again and press Turn Left; latch it again and press Stop; latch it again and leave the screen | Each of the four ends the movement and STOP is sent (the second takes the latch over first) |
| TC-044 | FR-044, NFR-12 | Three link states in turn: none, demonstration, Bluetooth | Press Forward in each state | The line reads, in order: not sent with the reason; sent to the demonstration link with the warning that nothing reached the robot; sent to the robot |
| TC-045 | FR-045 | Listening is on and connected | Say “forward”, then “stop” | The Forward button lights and the repeat starts as if it had been pressed; Stop clears it |

## 5.6  Camera and recording

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-050 | FR-050 | Camera permission granted | Open Robot Control, then Face Recognition, then Object Detection | The picture appears on each; the camera is not reopened per screen and no screen shows an error about the camera being in use |
| TC-051 | FR-051 | The camera permission has been refused once | Move between four screens that show the picture | The permission prompt does not appear again; each screen shows the artwork instead of the picture |
| TC-052 | FR-052 | Robot Control is open, camera permission granted | Press the red button at the top right of the picture; wait ten seconds; press it again | It becomes a red square while recording and returns to a disc; a message names the saved file |
| TC-053 | FR-053 | A recording has been made | Look in Android/data/com.falcon.robot/files/Movies | A file named VID_<date>_<time>.mp4 exists and plays |
| TC-054 | FR-054 | Listening is on | Record ten seconds; speak a command while recording | The command is recognised during the recording; the file has a picture and no sound track |
| TC-055 | FR-055 | At least two recordings exist | Press the Open button; choose one | They are listed newest first; the chosen one opens in a video player |
| TC-056 | FR-056 | A recording is running | Go to the Face screen, wait ten seconds, come back | The recording is still running and the button still shows a red square; the file covers the whole period |
| TC-057 | FR-057 | A device whose camera cannot bind three use cases | Switch on both models and press record | The recording is refused with a message; the picture and the models keep running |

## 5.7  Face recognition

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-060 | FR-060 | Face Recognition is open, a person is in view | Watch the picture | A box follows the face |
| TC-061 | FR-061 | A person is enrolled | Put that person in view | Name, identifier, similarity, department, position and time are shown |
| TC-062 | FR-062 | A person who is not enrolled is in view | Watch the result panel | The result is unrecognised, and no record is shown |
| TC-063 | FR-063 | Face Recognition is open, a person is in view | Press add; fill in name, identifier, department and position; confirm | The record appears in the database panel and the person is recognised afterwards |
| TC-064 | FR-064 | Several records exist | Type part of a name in the search box; long press a row | The list narrows to the matches; the long press removes the record after confirmation |
| TC-065 | FR-065 | A record exists | Close the application; open it again | The record is still there and still recognised |
| TC-066 | FR-066 | Recognitions have happened | Read the history panel | Each is listed with its time and result |

## 5.8  Object detection

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-070 | FR-070 | Object Detection is open, a chair and a person are in view | Switch detection on | Both are found and labelled |
| TC-071 | FR-071 | Detection is running | Switch boxes, labels and masks in turn | Each disappears and returns as switched |
| TC-072 | FR-072 | Detection and tracking are on | Walk an object across the view | It keeps the same identity and colour while it is in view |
| TC-073 | FR-073 | Detection is running | Set the threshold to its highest value | Only confident detections remain |
| TC-074 | FR-074 | Detection is running | Read the result panel | What is in frame is listed with counts, and an inference time in milliseconds is shown |

## 5.9  Speech recognition

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-080 | FR-080 | Flight mode is on, listening is on | Say “wave” | It is recognised with no network |
| TC-081 | FR-081 | Voice screen is open | Open the Language tab; note the languages; choose Korean and press Apply | English and Korean are offered; the message names the model chosen — tiny-en for English, tiny-kp for Korean |
| TC-082 | FR-082 | Korean has been chosen | Leave the screen and return; close the application and start it again | Korean is still chosen both times |
| TC-083 | FR-083 | Listening is on and connected | Say each of the sixteen commands in turn | Each is recognised, named on screen, and carried out: thirteen go to the robot, three act on the tablet |
| TC-084 | FR-084 | Listening is on | Say “what is the weather like today” | Unknown Command is shown; what was heard is not displayed; nothing is sent |
| TC-085 | FR-085 | Listening is on | Say “wave” | A confirmation sound is heard as soon as it is understood |
| TC-086 | FR-086 | Listening is on, face recognition off | Say “face recognize”; then say it again | Face recognition switches on, then off; the Face button in the sidebar follows |
| TC-087 | FR-087 | Listening is on | Say “follow me”; then press the Follow me button | The switch alternates cleanly: the spoken order starts following, the button stops it; the button never shows the opposite of what was sent |
| TC-088 | FR-088 | Listening is on | Go to another screen and say “wave” | It is still recognised and carried out |
| TC-089 | FR-089 | Voice screen is open | Say a command | Transcript, matched command, confidence and time are shown, and a history line is added |
| TC-090 | FR-090 | Voice screen is open | Tap a row in the command list | It behaves as though the phrase had been spoken |
| TC-091 | FR-091 | Voice screen is open | Switch command control off; say “forward” | It is recognised and reported, and nothing is sent to the robot |
| TC-092 | FR-092 | Voice screen is open | Press the microphone and speak | A live waveform is drawn under Speak Now, about nine tenths of the panel width |

## 5.10  LiDAR SLAM

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-100 | FR-100 | LiDAR screen is open | Open the 3D, top and map tabs in turn | Each view appears |
| TC-101 | FR-101 | Connected | Press Start Mapping; then Stop | The commands are sent and the status panel follows |
| TC-102 | FR-102 | Connected | Press Save Map; then Load Map and choose one; repeat from the Robot Control LiDAR panel | Both screens send the commands and log the result |
| TC-103 | FR-103 | LiDAR screen is open | Change range, resolution, mode and update rate; press Apply | The settings are sent and confirmed in the log |
| TC-104 | FR-104 | LiDAR screen is open | Read the pose and quality panels | X, Y, yaw, a compass, range, points and quality are shown |

## 5.11  Remote Control

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-110 | FR-110 | Remote Control is open | Read the top row | Camera view, robot status with position, and the map are shown |
| TC-111 | FR-111 | Remote Control is open | Read the bottom row | The BLE Connection panel is present with state, Scan, Disconnect and the device list |
| TC-112 | FR-112 | Connected | Press photo, video and audio; open the gallery | Each sends its command and is logged; what has been captured is listed |
| TC-113 | FR-113 | Remote Control is open | Do several things; press Clear | Each is logged with its time; Clear empties the log |

## 5.12  Settings

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-120 | FR-120 | Settings is open | Read the list | Connection, language, camera and system information are offered, each with a summary |
| TC-121 | FR-121 | Settings is open | Choose Korean; look at the screens | The interface is in Korean at once, including the navigation sidebar |
| TC-122 | FR-122 | Settings is open | Change the lens and the analysis frame size; restart the application | The picture changes accordingly and the choices survive the restart |

## 5.13  Background service

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-130 | FR-130 | Face recognition is on | Leave the application with the home button; wait thirty seconds; return | Recognition has kept running and the results continue |
| TC-131 | FR-131 | A feature is on | Open the notification shade; press Stop | A notification is shown while the camera or microphone is held; Stop switches everything off and the notification goes |
| TC-132 | FR-132 | Face and objects are on | Close the application; start it again | Both are still on |
| TC-133 | FR-133 | Everything is switched off | Leave every screen | The service stops; the notification goes |

## 5.14  Non-functional

| ID | Req. | Pre-condition | Steps | Expected result |
| --- | --- | --- | --- | --- |
| TC-140 | NFR-01 | Object detection is running on the target tablet | Read the inference time over a minute | It stays below 150 ms for a 640 × 480 frame |
| TC-141 | NFR-02 | Both models are on | Watch the preview while moving the tablet | The picture stays fluid; no freeze longer than half a second |
| TC-142 | NFR-03 | Listening is on and connected | Say “forward” and time the robot's reaction | The command is acted on within two seconds of the end of the phrase |
| TC-143 | NFR-04 | Forward is latched | Record the traffic for ten seconds | Ten repeats, one a second, each within 200 ms of its slot |
| TC-144 | NFR-05 | Listening on and a recording running | Switch the screen off for a minute; switch it on | Both are still running and the recording covers the period |
| TC-145 | NFR-07 | A model file has been removed from the assets | Switch on the feature that needs it | The application says the model is missing and everything else continues to work |
| TC-146 | NFR-08, NFR-17 | Flight mode is on | Use face, object and speech recognition, and record | Everything works; nothing is sent anywhere; no network error is shown |
| TC-147 | NFR-09 | A recording exists | Look for the file from another application's file browser | It is in the application's own folder and is not readable by others; opening it from the Open button still works |
| TC-148 | NFR-10 | The application is running | Inspect the manifest and the open ports | No internet permission is declared and no socket is listening |
| TC-149 | NFR-11 | Robot Control is open on a tablet with a mouse | Drag inside the Camera Settings panel with the mouse; press buttons with a finger | The panel scrolls under the drag and every control responds to both |
| TC-150 | NFR-13 | The build under test | Run the resource and translation check | The four languages carry the same string names, the same list lengths and the same placeholders; no string is unused |
| TC-151 | NFR-14 | A phone and a tablet | Open Robot Control on each | The tablet lays the panels side by side; the phone stacks them and the page scrolls |
| TC-152 | NFR-15 | The source | Search for the text of a command, for example MOVE FORWARD | It is defined once, in the command table, and referred to from there |
| TC-153 | NFR-16 | A candidate build | Run the static check described in Document 3, section 3.17 | It reports no problems |
| TC-154 | NFR-18 | Several recordings exist | Use the application for a week, filling storage moderately | No recording is deleted by the application |

## 5.15  Traceability

Each requirement of Document 2 names its test case, and each case above names its requirement. The count is: 82 functional requirements and 18 non-functional requirements, covered by 97 cases. A requirement whose case is blocked or failing is not accepted.

## 5.16  Regression set

The shortest set worth running after any change, because these are the paths most things pass through:

| Case | Why it is in the set |
| --- | --- |
| TC-012 | The link is made at all |
| TC-014 | A command reaches the robot in the expected wording |
| TC-016 | The write queue does not wedge |
| TC-042 and TC-043 | The movement latch starts and stops |
| TC-044 | The screen tells the truth about delivery |
| TC-083 | All sixteen commands are recognised and carried out |
| TC-050 and TC-051 | The camera is shared, and the prompt does not repeat |
| TC-130 | Recognition survives leaving the application |
| TC-150 and TC-153 | Resources and translations are sound |

## 5.17  Entry and exit criteria

| Gate | Criteria |
| --- | --- |
| Entry to a test pass | The build installs; the static check of Document 3, section 3.17 reports nothing; the robot is available for the link cases |
| Exit from a test pass | Every mandatory requirement has a passing case; no failure of severity 1 or 2 is open; blocked cases are listed with their reason |

## 5.18  How a failure is classified

| Severity | Meaning | Example |
| --- | --- | --- |
| 1 Critical | The robot can be made to move when it should not, or cannot be stopped | Stop does not release the latch |
| 2 Major | A mandatory requirement fails with no workaround | Commands do not reach a connected robot |
| 3 Moderate | A requirement fails but there is a way round it | Load Map works from the LiDAR screen but not from the panel |
| 4 Minor | Cosmetic or wording | A label is clipped in one language |

## 5.19  Result sheet

One row per case, filled in during the pass. The first rows are shown as an example of the form.

| Case | Build | Date | Device | Result | Note |
| --- | --- | --- | --- | --- | --- |
| TC-001 |  |  |  |  |  |
| TC-002 |  |  |  |  |  |
| TC-003 |  |  |  |  |  |
| … |  |  |  |  |  |
| TC-154 |  |  |  |  |  |

## 5.20  Test data

| Item | Value to use |
| --- | --- |
| Enrolled person | Name: Test Subject 1; identifier: EMP-0001; department: Engineering; position: Technician |
| Objects | A chair, a bottle and a person — three COCO classes that are easy to find |
| Spoken phrases | The sixteen commands, each said plainly once, then “move forward please” and “what is the weather like today” |
| Map names | Whatever the saved-map list offers on the build under test |
| Recording length | Ten seconds for the ordinary cases; one minute for TC-144 |

## 5.21  The static check

Run before anything else, and before a build is offered for test at all. It reads the resources and the sources and refuses a build that has lost something a screen needs. It has been proved against deliberately broken identifiers, so a pass means something.

| ID | What is checked | A failure means |
| --- | --- | --- |
| TC-150 | Every drawable, string, colour, dimension, style and layout a layout file names exists | A screen would fail to inflate, usually as a blank page |
| SC-2 | Every identifier a screen looks up is reachable from that screen's own layout | A null view, and a crash the first time that control is touched |
| SC-3 | No string is left that nothing uses | Dead weight, and worse, something to translate that nobody reads |
| SC-4 | The four languages hold the same string names | A missing translation shows English in the middle of another language |
| SC-5 | Format specifiers match across the four languages | A crash when that string is formatted |
| SC-6 | String arrays are the same length in each language | A dropdown that selects the wrong thing, because the code selects by position |
| SC-7 | Every XML file parses | A build failure at best |
| SC-8 | Braces and parentheses balance in every source file | A build failure, caught faster here |

## 5.22  Command matching: English

These phrases are run against the matcher and must produce the action named. They are not a sample of what an operator might say; they are the boundary cases, including the ones that must produce nothing. A run of this table takes a minute from the Voice screen's command list, or can be run against the matcher directly.

| Said | Action | Line written to the robot |
| --- | --- | --- |
| move forward please | Forward | MOVE FORWARD |
| go forwards | Forward | MOVE FORWARD |
| straight ahead | Forward | MOVE FORWARD |
| back up | Backward | MOVE BACKWARD |
| go backwards now | Backward | MOVE BACKWARD |
| reverse | Backward | MOVE BACKWARD |
| turn right | Turn Right | TURN RIGHT |
| right | Turn Right | TURN RIGHT |
| turn to the left | Turn Left | TURN LEFT |
| arm left | Arm Left | ARM SELECT LEFT_ARM |
| raise the left arm | Arm Left | ARM SELECT LEFT_ARM |
| arm right | Arm Right | ARM SELECT RIGHT_ARM |
| follow me | Follow me | FOLLOW START, then FOLLOW STOP |
| face recognize | Face Recognize | nothing; the tablet acts |
| detect objects | Object Detect | nothing; the tablet acts |
| object detect | Object Detect | nothing; the tablet acts |
| dance | Dance | POSE DANCE |
| stop | Stop | STOP |
| halt | Stop | STOP |
| start recording | Recording | nothing; the tablet records |
| stop recording | Recording | nothing; the tablet stops recording |
| record | Recording | nothing; the tablet records |
| wave | Wave | POSE WAVE |
| hello | Hello | GREET |
| hi there | Hello | GREET |
| t pose | T-Pose | POSE T_POSE |
| tpose | T-Pose | POSE T_POSE |
| no | No | CANCEL |
| cancel | No | CANCEL |
| warm weather | nothing | nothing is sent |
| open the door | nothing | nothing is sent |
| turn the light on | nothing | nothing is sent |
| start following | nothing | nothing is sent — see the note below |

Two rows in that table are the interesting ones. “stop recording” gives Recording and not Stop, because Stop excludes the word recording — without that exclusion, asking the tablet to stop recording would stop the robot instead. And “start following” matches nothing, because the keyword is follow and the matcher compares whole words: the phrase to use is “follow me”. That is current behaviour rather than a defect to be filed, and it is written into the user manual.

## 5.23  Command matching: Korean

Korean is matched as a substring, because the endings glue onto the verb. Each of these must produce the action named, in the North Korean forms the interface uses.

| Said | Action |
| --- | --- |
| 전진 | Forward |
| 앞으로 가 | Forward |
| 뒤로 | Backward |
| 오른쪽으로 | Turn Right |
| 왼쪽 돌아 | Turn Left |
| 왼팔 | Arm Left |
| 오른팔 | Arm Right |
| 멈춰 | Stop |
| 따라와 | Follow me |
| 얼굴 인식 | Face Recognize |
| 록화 | Recording |
| 춤 춰 | Dance |
| 안녕 | Hello |
| 취소 | No |
| 티자세 | T-Pose |
| 손 흔들어 | Wave |

록화 and 녹화 are both accepted for Recording, and 촬영 with them, so that an operator is not caught out by the form the model happens to produce.

## 5.24  Command matching: Chinese and Japanese

The interface offers these two languages, and the matcher understands their phrases whatever the interface is set to, so they are checked even though speech recognition is offered in English and Korean only.

| Said | Language | Action |
| --- | --- | --- |
| 前进 | Chinese | Forward |
| 右转 | Chinese | Turn Right |
| 左臂 | Chinese | Arm Left |
| 停止 | Chinese | Stop |
| 跳舞 | Chinese | Dance |
| 你好 | Chinese | Hello |
| 前進 | Japanese | Forward |
| 後退 | Japanese | Backward |
| 右へ | Japanese | Turn Right |
| 手を振って | Japanese | Wave |
| 録画 | Japanese | Recording |
| ついてきて | Japanese | Follow me |

## 5.25  The movement latch

Seven scripts. Each is a sequence of presses on the Robot Control sidebar, and what must be written to the robot. Run them with the robot's console open; the point of the test is what left the tablet, not what the figure on the screen did.

| No. | Presses | Must be written |
| --- | --- | --- |
| L1 | Forward, then wait four seconds | MOVE FORWARD at once, then again at about one second intervals — five lines in four seconds, give or take one |
| L2 | Forward, then Forward again | MOVE FORWARD, then STOP; the repeat ends and the button is no longer marked |
| L3 | Forward, then Backward | MOVE FORWARD, then MOVE BACKWARD; only Backward is marked, and only it repeats |
| L4 | Forward, then Stop | MOVE FORWARD, then STOP; nothing repeats afterwards |
| L5 | Turn Left, then Turn Right, then Stop | TURN LEFT, TURN RIGHT, STOP, each taking over from the one before |
| L6 | Forward, then say “backward” | MOVE FORWARD, then MOVE BACKWARD; the Backward button becomes the marked one, so the screen and the robot agree about what was ordered |
| L7 | Forward, then leave the screen | MOVE FORWARD, then the repeat stops; nothing further is written, and the robot's own timeout is what stops the machine |

## 5.26  Telling the truth about delivery

The failure that started this: a robot that did not move while the tablet looked perfectly happy. These cases exist to make sure the screen can no longer say a command arrived when it did not.

| State | Press Forward | The status line must say |
| --- | --- | --- |
| Connected over Bluetooth | MOVE FORWARD is written to the robot | That the command was sent over Bluetooth, naming it |
| Connected to the demonstration link | Nothing reaches a robot | That the command went to the demonstration link only |
| Not connected | Nothing is sent | That the command was not sent, because no robot is connected |
| Connected, then the robot is switched off mid-movement | The link drops | Offline on every screen, and the next command reports that it was not sent |
| Connected, but the robot answers to different words | The line is written; the robot does nothing | That it was sent — which is true, and why the vocabulary in Document 1, section 1.19 has to be agreed rather than assumed |

The last row is the one to be careful with in a test report. A command that is correctly written and correctly reported, to a robot that does nothing with it, is a firmware finding, not a tablet finding. The way to tell them apart is the robot's own console: if the line arrived there, the tablet has done its work.

## 5.27  Languages

Run once per interface language. Korean, Japanese and Chinese each take about twenty minutes; the point is not to read every string but to look for the four faults that translation actually produces.

| Check | What to look for |
| --- | --- |
| Nothing English left | An English word in the middle of another language means a string was not translated |
| Nothing clipped | A label cut off by an ellipsis where the English fits; the sidebar and the panel titles are where this shows first |
| Nothing pushed off | A control moved out of its panel by a longer label |
| Dropdowns in order | Every chooser offers the same entries in the same order as in English, because the code selects by position |
| Numbers and units | Percentages, metres and degrees formatted as that language writes them |
| Korean forms | North Korean forms throughout, including the command names |
| The language names | Each language's own name is shown in that language, in every interface language — this is deliberate, not a missing translation |
| Speech is separate | Changing the interface language does not change the language being listened for, and the other way round |

## 5.28  Devices and configurations

The application is built for one tablet, but it must not fall over on another. These are the configurations worth one pass each.

| Configuration | What is being checked | Expected |
| --- | --- | --- |
| The target tablet, landscape | Everything | The full test pass |
| A tablet at 600 to 840 dp | Rows stack, tablet sizes stay | Nothing clipped, nothing unreachable |
| A phone, below 600 dp | The compact dimension set | The page scrolls, the sidebars narrow, every control still reachable |
| Android 8.0 | The oldest platform supported | No notification permission is asked for; scanning asks for location instead of the Bluetooth permissions |
| Android 12 or newer | The newer permission model | Scanning asks for the Bluetooth permissions; notifications are asked for on 13 and above |
| No camera permission | Every screen that shows a picture | Artwork and an explanation; no repeated prompting |
| No microphone permission | The Voice screen | Listening stays off and says why |
| Bluetooth turned off | Remote Control | The state says so; the scan refuses to start |
| Airplane mode | The whole application | Nothing crashes; the link says it is not connected |
| No robot in range | A scan | An empty list after twelve seconds, and a scan that stops itself |

## 5.29  Timing

Measured against the targets of Document 2, section 2.14, on the target tablet, with a stopwatch or the robot's own console timestamps. Record the figure, not merely a pass.

| What is measured | How | Target |
| --- | --- | --- |
| Button to line written | The robot's console timestamp against the press | Under 100 ms |
| Repeat interval while held | Timestamps of successive MOVE lines | About 1000 ms |
| Stop | Press Stop while a direction is held | Under 200 ms |
| Scan to first device | Stopwatch from the press | A few seconds |
| Connect to usable | From the tap to Connected on the screen | Under 5 s |
| Screen change to picture | Open a screen that shows the camera | Under 1 s |
| Face inference | The figure the AI line reports | Under 200 ms |
| Object inference | The same | Under 200 ms |
| Speech to result | From the end of speaking to the transcript | Under 2 s |
| Acknowledgement | Listen for it | At once on a match, before the command is sent |

## 5.30  Robustness

What happens when the application is used badly, or when something is taken away while it is working. None of these may crash, and none may leave the robot moving.

| No. | Do this | Expected |
| --- | --- | --- |
| R1 | Press a direction button ten times quickly | The latch follows the last press; the commands do not pile up |
| R2 | Press Forward and Backward alternately, quickly | Each takes over from the one before; no two directions are marked at once |
| R3 | Turn the robot off while a direction is held | The screen shows offline within a moment; the next command says it was not sent |
| R4 | Walk out of range and back | The link drops and is shown as dropped; reconnecting works without restarting the application |
| R5 | Press Scan repeatedly | One scan runs; the button reads Stop Scan while it does |
| R6 | Connect to a device that is not the robot | It is reported as not offering the service, and disconnected |
| R7 | Start recording and change screens four times | Recording continues; the notification says so; stopping it works from the camera screen |
| R8 | Start recording and leave the application | Recording continues; stopping from the notification closes the file properly |
| R9 | Fill the storage, then record | The recording refuses and says why |
| R10 | Switch face, objects and listening all on at once | All three run; the frame rate drops but nothing stops |
| R11 | Change the recognition language while listening | Listening stops, the new model loads with its name shown, listening resumes |
| R12 | Change the interface language with a recording running | The screens redraw in the new language; the recording is not interrupted |
| R13 | Rotate the tablet | The layout holds; nothing is lost |
| R14 | Say a command while the robot is disconnected | It is recognised and acknowledged, and reported as not sent |
| R15 | Say two commands quickly | Both are recognised, in order |
| R16 | Speak nonsense | Unknown Command; nothing is sent; what was heard is not offered as a command |
| R17 | Press Shutdown by accident | A confirmation, which can be refused |
| R18 | Leave the application running for an hour with everything switched on | Still working; the link still up or honestly reported as down; the tablet not hot enough to throttle |

## 5.31  The field pass

The part that cannot be done at a desk. It needs the robot, a clear floor of about four metres, and two people: one on the tablet, one beside the robot with a hand near its power.

| Step | What is done | What is being proved |
| --- | --- | --- |
| 1 | Power the robot, scan, connect | The link, in the room it will be used in |
| 2 | Press Stop before anything else | That the control which has to work, works, and that the robot answers to it from rest |
| 3 | Press Forward, let it run two seconds, press Stop | The latch, the repeat and the stop, in that order |
| 4 | Repeat with Backward, Turn Left, Turn Right | The other three directions |
| 5 | Press each of Wave, Hello, T-Pose, Dance | The poses |
| 6 | Press Arm Left, then Arm Right | Arm selection |
| 7 | Say each of the sixteen commands in turn, from two metres away | Recognition in the real acoustics of the room, and that every spoken order does what its button does |
| 8 | Say Forward, then say Stop | That a spoken stop reaches the robot as quickly as a pressed one |
| 9 | Walk out of range while moving | That the robot stops on its own, which is the robot's obligation and not the tablet's |
| 10 | Reconnect and drive again | Recovery without restarting anything |
| 11 | Record thirty seconds of video, then play it back | Recording, in the field |
| 12 | Read the operation log from the start | That the log is a usable record of what was sent and what came back |

Step 2 is not a formality. If Stop does not work at the beginning, the rest of the pass is done with a machine that cannot be stopped from the tablet, and should not be done at all.

## 5.32  Cases that need the robot

So that a pass can be planned around what is available, and so that a case which cannot be run is recorded as blocked rather than quietly passed.

| Group | Needs a robot | Can be run without one |
| --- | --- | --- |
| Shell and navigation | None | All |
| Link | Scanning, connecting, writing, notifications, dropping | The permission cases, and the case for a device that is not the robot |
| Robot Control | That a command arrives | Every screen behaviour, the latch marking, the delivery reporting on the demonstration link and with nothing connected |
| Camera and recording | None | All |
| Face and objects | None | All |
| Speech | That the command arrives | Recognition, matching, acknowledgement, switch behaviour |
| LiDAR | Live scanning and mapping | The views, the controls, the saved map |
| Remote Control | The robot camera and its recording | The connection panel with any Bluetooth device, and the log |
| Settings and service | None | All |

Roughly two thirds of the ninety-seven cases can be run with no robot present. That is deliberate: it is what lets a build be checked the day it is made, rather than the day the robot is free.

## 5.33  What to run after a particular change

A full pass is ninety-seven cases and the better part of a day. Most changes do not warrant one. This is what a change of each kind actually puts at risk.

| If this changed | Run these | Because |
| --- | --- | --- |
| A command string | TC-014, TC-083, and the field pass of section 5.31 | The wording is the interface, and nothing else checks it |
| VoiceCommands | Sections 5.22 to 5.24 in full, and TC-083 | One rule can shadow another: an exclusion removed in one place changes what a different phrase matches |
| The latch | Section 5.25, all seven scripts | Latching is where a robot gets left moving |
| BleLink | TC-010 to TC-021, and R3 to R6 | The write queue and the connection sequence are where the link wedges |
| The camera binding in the service | TC-050 to TC-057, and R7 and R8 | Two use cases bound wrongly gives a black picture or a refused recording |
| A layout | The static check, then that screen on a tablet and on a phone | An identifier moved out of reach is a crash the first time that control is touched |
| A string, in any language | The static check, and section 5.27 for that language | Parity and format specifiers |
| A model | The recognition cases for that model, and the timing of section 5.29 | Accuracy and speed both move |
| Anything at all | The regression set of section 5.16 | It is nine cases and it catches most of what breaks |

## 5.34  Behaviours that are not defects

Each of these has been raised as a fault at least once. They are current, intended behaviour; a report that records one of them as a failure is describing the design.

| What is seen | Why it is so |
| --- | --- |
| The figure on the screen moves although the robot does not | The figure moves on recognition, before and independently of any send. Only the line along the foot reports delivery |
| Saying “start following” does nothing | The keyword is follow, matched as a whole word; the phrase is “follow me” |
| Saying “stop recording” does not stop the robot | Stop excludes the word recording on purpose, so that the camera can be stopped without stopping the machine |
| The camera permission prompt does not come back | It is asked once, application-wide; the opposite was a reported fault |
| Battery and temperature look plausible with no robot connected | They are simulated until the firmware sends telemetry, and are marked as simulated in section 2.20 of Document 2 |
| A command reported as sent produced no movement | The tablet reports what it wrote, not what the robot did. The robot console settles it |
| The recordings have no sound | Recording is silent so that listening can continue |
| Each language name is shown in its own language, in every interface language | Deliberate: a reader looking for their own language should find it written the way they write it |
| Three features on at once slows the picture | Three models share one camera and one processor; the frame rate is what gives way |

# Document 6  User Manual

This manual is for the person who operates the robot from the tablet. It assumes no knowledge of the software; it does assume the robot is switched on and within a few metres.

## 6.1  What you have

The tablet application AI Robot Control does three jobs. It drives the robot. It recognises faces, objects and spoken commands, all on the tablet itself. And it records what the camera sees. Nothing it does needs a network, and nothing it sees leaves the tablet.

| Screen | What it is for |
| --- | --- |
| Home | A card for each screen, and the tablet's own status |
| Robot Control | The screen to work from: the robot, the camera, the LiDAR map, the log and every action |
| Face Recognition | Enrolling people and recognising them |
| Voice Recognition | Listening, the command list and speech settings |
| Object Detection | What the camera can see, with boxes and labels |
| LiDAR SLAM | The point cloud, the map, mapping and navigation |
| Remote Control | Connecting to the robot over Bluetooth, and the robot's own camera |
| Settings | Connection, language, camera, system information |

## 6.2  First run

Switch the tablet's Bluetooth on.

Start AI Robot Control. The Home screen appears.

Open Robot Control. The tablet asks for the camera the first time; allow it, or the camera panel shows artwork instead of a picture. It will not ask again on every screen.

When you first switch listening on, the tablet asks for the microphone. On Android 13 and newer it also asks to show a notification, which is what lets recognition keep running while you are on another screen.

Connect the robot, as described next. Until you do, buttons and spoken commands are recognised but reach nothing — and the screen says so.

## 6.3  Connecting the robot

Open Remote Control.

Find the BLE Connection panel at the bottom left. If it says Bluetooth is off, switch Bluetooth on and come back.

Press Scan. Devices appear over the next few seconds, the closest first, each with its address and signal strength. The scan stops by itself after about twelve seconds.

Tap the robot. The panel shows Connecting, then Connected with the robot's name, and the log records it.

The link belongs to the application, not to that screen: go anywhere you like and the robot stays connected. Press Disconnect, or switch the robot off, to end it.

If the panel says the device does not offer the robot service, you have connected to something else — a headset or a watch. Disconnect and pick another entry.

## 6.4  The Robot Control screen

### 6.4.1  What is where

| Area | What it shows |
| --- | --- |
| Top left | The robot as a 3D figure, with mode, speed, battery, temperature and position |
| Top right | The live camera, with faces and objects marked on it |
| Bottom left | The LiDAR map |
| Bottom middle | Camera settings |
| Bottom right | The log |
| Right-hand strip | Every action the robot answers to |
| Along the foot | The last thing that happened, with the time |

### 6.4.2  Driving

Press Forward and the robot keeps going: the command is repeated every second until something stops it. To stop, press the lit button again, press Stop, press another direction, or leave the screen. There is no speed control on this screen.

### 6.4.3  Reading the line along the foot

This is the line to watch, because the 3D figure acts out every order whether or not anything reached the robot.

| What it says | What it means | What to do |
| --- | --- | --- |
| Sent to the robot: MOVE FORWARD | It was delivered over Bluetooth | Nothing |
| … the link is the demo one, so nothing reached the robot | You are connected to the address in the connection dialog, which is not a robot | Connect over Bluetooth from Remote Control |
| … not sent: no robot connected | There is no link at all | Connect over Bluetooth from Remote Control |
| … done on the tablet | Face, object or recording — the tablet did it itself | Nothing |

### 6.4.4  Recording what the camera sees

The red button at the top right of the picture starts recording; it becomes a red square while it runs. Press it again to stop, and a message names the file.

Recording has no sound, so that the robot can go on listening while it records.

The button beside it lists what you have recorded, newest first; tap one to play it.

A recording keeps running while you use other screens. It stops if you change the camera settings, which is announced.

### 6.4.5  The log

Press Start at the top of the Log panel and the screen writes down every recognition and every command, each with the time. This is the quickest way to see what the robot is being told. Press Stop to leave the lines in view without adding more.

## 6.5  Speaking to the robot

Press Voice in the right-hand strip to start listening. Say one of the commands below. You will hear a short confirmation as soon as the tablet understands, the button for that command lights, and the line along the foot says where it went.

| Say | The robot | Say | The robot |
| --- | --- | --- | --- |
| Forward | moves ahead until stopped | Arm Left | selects its left arm |
| Backward | moves back until stopped | Arm Right | selects its right arm |
| Turn Left | turns left until stopped | Wave | waves |
| Turn Right | turns right until stopped | Hello | greets you |
| Stop | stops | T-Pose | holds the T-pose |
| Follow me | follows you; say it again to stop | Dance | dances |
| No | cancels what it was asked for | Face Recognize | (tablet) face recognition on or off |
| Recording | (tablet) recording on or off | Object Detect | (tablet) object detection on or off |

If you say anything else, the screen shows Unknown Command and nothing is sent. It does not print what it thought it heard, because a wrong guess is worse than no guess.

### 6.5.1  Choosing the language you speak

Open Voice Recognition and find Advanced Settings at the bottom.

On the Language tab, choose English or Korean as the Recognition Language.

Press Apply. A message names the model being used — tiny-en for English, tiny-kp for Korean.

The choice is remembered, on this screen and everywhere else, until you change it.

The language you speak and the language the interface is written in are separate. The interface language is under Settings.

## 6.6  Face recognition

Open Face Recognition and switch detection on. A box follows each face.

To enrol someone: have them face the camera, press the add button in the Face Database panel, and fill in name, identifier, department and position.

From then on, that person is named in the result panel with a similarity figure.

Search the database by typing part of a name. Press and hold a row to remove it.

Everything stays on this tablet: the register is a file in the application's own storage.

## 6.7  Object detection

Open Object Detection and switch it on. Boxes, labels and masks can each be switched, and the confidence threshold decides how sure the tablet must be before it draws anything. The result panel lists what is in view with counts, and how long each frame took.

## 6.8  LiDAR and maps

The LiDAR SLAM screen shows the point cloud in 3D, from the top, or as a 2D map.

Start Mapping tells the robot to build a map; Stop ends it.

Save Map stores what has been built. Load Map offers what has been stored, from this screen or from the LiDAR panel on Robot Control.

Range, resolution, mapping mode and update rate are under Settings on that screen; press Apply to send them.

## 6.9  Settings

| Setting | What it does |
| --- | --- |
| Connection | The address link and its state. The robot itself is connected over Bluetooth from Remote Control |
| Interface Language | English, Korean, Japanese or Chinese; it changes at once |
| Camera | Which lens, and the frame size the models work on. A larger frame finds smaller things and is slower |
| System Information | Version and build details |

## 6.10  If something does not work

| What you see | Why | What to do |
| --- | --- | --- |
| The 3D figure moves but the robot does not | The command reached nothing: no link, or the demonstration link | Read the line along the foot; connect over Bluetooth from Remote Control |
| The camera panel shows artwork, not a picture | The camera permission was refused | Android Settings → Apps → AI Robot Control → Permissions → Camera |
| Scan finds nothing | Bluetooth is off, the permission was refused, or the robot is not advertising | Switch Bluetooth on; allow the permission when asked; power-cycle the robot |
| The robot connects and immediately disconnects | That device does not offer the robot's serial service | Connect to the right device; if it is the right one, its firmware identifiers differ from the ones the application expects |
| Listening does nothing | The microphone permission was refused, or command control is switched off | Allow the microphone; switch command control on in Voice Recognition |
| Unknown Command every time | The recognition language does not match the language being spoken | Set the Recognition Language and press Apply |
| Recording is refused | The camera cannot run the picture, the models and a recording at once | Switch one of the models off and record again |
| Speech says a model is missing | The model file is not in this build | Install a build with the model, or choose the other language |

## 6.11  Questions

| Question | Answer |
| --- | --- |
| Does anything I record or recognise leave the tablet? | No. There is no network code in the application and no internet permission in it. |
| Where are my recordings? | In the application's own folder, Android/data/com.falcon.robot/files/Movies. The Open button is the easy way to them. |
| Can I use the robot while recognition is running? | Yes. Both run at once; recognition even keeps running while you are on another screen. |
| Why does the confirmation sound like a tone rather than a voice? | Because the file supplied with the build is a placeholder. Replacing assets/ok.wav changes what you hear. |
| Why is there no speed control? | Speed is set from the Remote Control screen. The Robot Control sidebar is for directions and actions. |
| What happens if the robot goes out of range while it is moving? | The link drops, every screen shows offline, and the next command reports that it was not sent. The robot stops when its own controller stops hearing from the tablet. |

## 6.12  Quick reference

| To do this | Do this |
| --- | --- |
| Connect the robot | Remote Control → Scan → tap the robot |
| Drive | Robot Control → Forward, Backward, Turn Left, Turn Right; press again or Stop to end |
| Speak to the robot | Robot Control → Voice, then say one of the sixteen commands |
| Record the camera | Robot Control → the red button on the picture |
| Watch a recording | Robot Control → the button beside the red one |
| See what is being sent | Robot Control → Log panel → Start |
| Enrol a face | Face Recognition → add in the database panel |
| Load a map | Robot Control → LiDAR panel → the folder icon |
| Change the spoken language | Voice Recognition → Advanced Settings → Language → Apply |
| Change the interface language | Settings → Interface Language |
| Stop everything | The notification shade → Stop |

## 6.13  Notes for whoever maintains the tablet

| Task | How |
| --- | --- |
| Change the confirmation sound | Replace app/src/main/assets/ok.wav with a mono WAV and rebuild. With the file absent, the tablet speaks the word instead |
| Remove a spoken language | Delete its entry from the recognition language list in the string resources; the model can then be deleted from the assets |
| Make the package smaller | Delete the speech models that are not offered; the repository readme says what each is worth |
| Point at different robot firmware | Change the three identifiers in BleLink, and the command strings in the command table, and nothing else |
| Collect the recordings | Android/data/com.falcon.robot/files/Movies over USB, or through the Open button one at a time |
| Start again from nothing | Android Settings → Apps → AI Robot Control → Storage → Clear data. This removes the face register, the switch states and the language choices, and not the recordings |

## 6.14  Putting the application on a tablet

Someone will have done this for you. If you are doing it yourself, this is all of it.

| Step | What to do |
| --- | --- |
| 1 | Copy falcon.ai.robot.apk to the tablet, or install it over a cable |
| 2 | Open it. Android will ask whether to allow an application from this source; say yes |
| 3 | Open the application. It will ask for the camera when it first wants a picture, for the microphone when you first switch listening on, and for Bluetooth the first time you scan. Allow all three |
| 4 | On Android 13 and newer it will also ask to show notifications. Allow that too: the notification is how you stop the camera and the microphone when the application is not in front of you |
| 5 | Open Settings and set the interface language, if English is not what you want |
| 6 | Open Remote Control, press Scan, and connect to the robot |

Installing a newer version over the top keeps everything: the faces you enrolled, your languages, your camera choices and your recordings. Removing the application takes all of it with it, and there is no copy anywhere else.

## 6.15  A first session, start to finish

If you have never used it, do this once. It takes about ten minutes and it puts you through everything you will use daily.

Switch the robot on and give it a moment to start advertising.

Open the application. You will see Home: a card for each screen, and the tablet's own status along the bottom.

Open Remote Control from the sidebar. Find the Bluetooth Connection panel and press Scan.

Wait. Devices appear as they are found, the strongest signal first. Tap the robot.

Watch the state: Connecting, then Connected with the robot's name. Every screen now shows the robot as connected.

Open Robot Control. This is where you will spend your time.

Find Stop in the Move Control group and press it. Nothing will happen, because the robot is standing still — but the line along the foot should say the command was sent. That is your check that the link works.

Press Forward. The robot moves and keeps moving: the button stays on, and the command is repeated once a second until you stop it.

Press Stop. The robot stops.

Press Voice in the AI group. The tablet starts listening.

Say “forward”. You should hear the acknowledgement, see the command appear along the foot, and see the Forward button come on by itself.

Say “stop”.

Press the red button at the top right of the camera picture. It becomes a red square and the line along the foot says Recording.

Press it again, then press the Open icon beside it and play what you recorded.

Press Voice again to stop listening, and put the tablet down. The application keeps whatever you left switched on, and tells you so with a notification.

## 6.16  The Actions sidebar, button by button

Nineteen buttons down the right of the Robot Control screen, in five groups. Every one of the sixteen spoken commands has a button here, so anything you can say you can also press.

| Group | Button | What it does |
| --- | --- | --- |
| AI | Face | Look for faces in the picture, and name the ones you have enrolled |
| AI | Objects | Look for things in the picture and label them |
| AI | Voice | Listen. Press it again to stop listening |
| Move Control | Forward | Drive forward and keep going until you stop it |
| Move Control | Backward | Drive backwards, the same way |
| Move Control | Turn Left | Turn on the spot, and keep turning |
| Move Control | Turn Right | The same, the other way |
| Move Control | Stop | Stop. This is the one to reach for without looking |
| Arm Control | Left Arm | Choose the left arm for whatever comes next |
| Arm Control | Right Arm | Choose the right arm |
| Preset Pose | Wave | Wave |
| Preset Pose | Hello | Greet whoever is in front of it |
| Preset Pose | T-Pose | Stand with both arms out; useful for checking the arms move |
| Preset Pose | Dance | Dance |
| Quick Actions | Follow Me | Start following you. Press it again to stop |
| Quick Actions | Recording | Start recording what the tablet's camera sees; press again to stop |
| Quick Actions | No | Cancel whatever was asked for |
| Quick Actions | Go Home | Send the robot back to its base |
| Quick Actions | Shutdown | Power the robot down. You will be asked to confirm |

Four of them stay on once pressed: Face, Objects, Voice and Follow Me, and Recording makes five. The button shows you which way it is set, and saying the command aloud moves the same switch, so the button and your voice can never disagree.

## 6.17  What to say, in English

You do not have to say the exact words. The tablet looks for the important word, so “move forward please” works as well as “forward”. The left column is what it listens for.

| Say | Or | What happens |
| --- | --- | --- |
| Forward | forwards, ahead, straight | The robot drives forward |
| Backward | back, backwards, reverse | It drives backwards |
| Turn Right | right | It turns right |
| Turn Left | left | It turns left |
| Stop | halt, freeze | It stops |
| Arm Left | left arm | The left arm is selected |
| Arm Right | right arm | The right arm is selected |
| Wave | waving | It waves |
| Hello | hi, hey | It greets |
| T-Pose | t pose, tpose | It stands with both arms out |
| Dance | dancing | It dances |
| No | nope, cancel | Whatever was asked for is cancelled |
| Follow me | — | It starts following you; say it again to stop |
| Face Recognize | face | The tablet starts looking for faces |
| Object Detect | objects, detect, target | The tablet starts labelling what it sees |
| Recording | record | The tablet starts recording; say it again to stop |

Three things are worth knowing. Say “follow me” rather than “start following” — the second one is not recognised. Saying “stop recording” stops the recording and does not stop the robot; to stop the robot, say “stop” on its own. And “left arm” chooses an arm while “turn left” turns the robot, so the word arm is what keeps those apart.

## 6.18  What to say, in Korean

Set the language you speak on the Voice Recognition screen, under Advanced settings, or in Settings. It is a separate setting from the language the screens are written in, so you can read English and speak Korean, or the other way round.

| Say | Or | What happens |
| --- | --- | --- |
| 전진 | 앞으로, 직진 | The robot drives forward |
| 후진 | 뒤로, 물러 | It drives backwards |
| 오른쪽 | 우회전, 우측 | It turns right |
| 왼쪽 | 좌회전, 좌측 | It turns left |
| 정지 | 멈춰, 스톱, 그만 | It stops |
| 왼팔 | 왼쪽 팔, 왼손 | The left arm is selected |
| 오른팔 | 오른쪽 팔, 오른손 | The right arm is selected |
| 손 흔들 | 인사 | It waves |
| 안녕 | 반갑 | It greets |
| 티자세 | 티 자세 | It stands with both arms out |
| 춤 | 댄스 | It dances |
| 취소 | 아니, 그만두 | Whatever was asked for is cancelled |
| 따라와 | 날 따라, 추종 | It starts following; say it again to stop |
| 얼굴 | — | The tablet starts looking for faces |
| 물체 | 대상, 객체 | The tablet starts labelling what it sees |
| 록화 | 녹화, 촬영 | The tablet starts recording; say it again to stop |

Endings do not matter. 왼쪽, 왼쪽으로 and 왼쪽 돌아 all turn the robot left, because the tablet looks for the part of the word that does not change.

## 6.19  Recording, and watching what you recorded

| To do this | Do this |
| --- | --- |
| Start recording | Press the red circle at the top right of the camera picture, or say “recording” |
| See that it is recording | The red circle becomes a red square, and the line along the foot says Recording |
| Keep recording while you work | Change screens freely. Recording carries on, and the notification says so |
| Stop | Press the red square, or say “recording” again |
| Watch one back | Press the Open icon beside the record button. The newest is at the top. Tap one to play it |
| Find the files | They are in the application's own Movies folder, named with the date and time |
| Free up room | Delete them from the tablet's file manager; the application never deletes them for you |

The recordings have no sound. That is deliberate: it leaves the microphone free, so the robot still answers you while you are recording.

## 6.20  Maps

| To do this | Do this |
| --- | --- |
| See the map | It is in the LiDAR panel on Robot Control, and on the LiDAR SLAM screen in full |
| Open the LiDAR screen | Tap the LiDAR panel's title, or use the sidebar |
| Start mapping | Start Mapping, on the LiDAR screen |
| Keep a map | Save Map |
| Open a map you kept | Load Map, either on the LiDAR screen or from the corner of the LiDAR panel on Robot Control |
| Look around the map | The zoom buttons, and Fit to bring the whole map back |
| See where the robot thinks it is | The pose panel: X, Y, the heading and a compass |

## 6.21  Settings, row by row

| Row | What it does |
| --- | --- |
| Connection | Shows the address and whether you are connected, and opens the connection dialog. Connecting to a robot is normally done from Remote Control, over Bluetooth; this row is the older address connection, which is used for demonstrations |
| Interface language | English, Korean, Japanese or Chinese. The screens redraw at once. This does not change the language you speak to it in |
| Camera | Which camera to use, and the frame size the recognition works on. A smaller frame is faster and less accurate |
| System information | Which build this is. Worth reading out when reporting a fault |

## 6.22  What the messages mean

The line along the foot of the Robot Control screen, and the log beside it, tell you what actually happened. These are the ones worth recognising.

| If it says | It means | Do this |
| --- | --- | --- |
| Sent over Bluetooth | The command was written to the robot | Nothing. If the robot did not move, the fault is on the robot's side |
| Sent to the demonstration link only | You are connected to the address link, not to a robot. Nothing reached any machine | Connect over Bluetooth from Remote Control |
| Not sent — no robot connected | Nothing was sent | Connect first |
| Connected · <name> | The Bluetooth link is up and the robot can answer | Nothing |
| Connecting to <name> | The link is being made | Wait a few seconds |
| Scanning | Looking for devices | Wait; it stops by itself after twelve seconds |
| Not connected | There is no link | Scan and connect |
| Bluetooth is off | The tablet's radio is switched off | Switch Bluetooth on in the tablet's own settings |
| This device does not offer the robot service | You connected to something that is not the robot | Scan again and choose the robot |
| Camera permission is needed | The picture cannot be shown | Allow the camera in the tablet's application settings |
| Microphone permission is needed | It cannot listen | Allow the microphone the same way |
| Unknown Command | It heard you but matched nothing | Say one of the words in section 6.17 or 6.18 |
| Loading <model> | A recognition model is being made ready | Wait a moment; it is normal the first time |
| Speech engine not built | The optional extra engine is not in this build | Nothing; the ordinary models are being used |
| Recording | The tablet's camera is recording | Press the red square to stop |
| Recording could not start | Usually there is no room on the tablet | Delete some recordings |
| Unrecognised | A face was seen but is not one you enrolled | Enrol them from the Face Recognition screen |

## 6.23  The robot is not moving

Work down this list. It is in the order that finds the fault fastest.

| Ask | If no | If yes |
| --- | --- | --- |
| Does the top of the screen say connected? | Connect from Remote Control | Go on |
| Does it say Connected · with a name, rather than just connected? | You are on the demonstration link; connect over Bluetooth | Go on |
| When you press Forward, does the line along the foot say it was sent over Bluetooth? | That is a tablet-side fault; note exactly what it says and report it | Go on |
| Is the robot itself powered, and out of any fault state of its own? | Fix that first | Go on |
| Does the figure on the screen move while the robot does not? | — | The figure moves because the tablet understood you, not because the robot did anything. It proves recognition, not movement |
| Does the robot's own console show the command arriving? | It is a radio problem: range, or the robot's Bluetooth | The command arrived and the robot did nothing with it — that is a robot-side fault, and the wording it expects should be checked against Document 1, section 1.19 |

The one to remember is the fifth. The figure on the screen moves as soon as the tablet recognises a command, whether or not anything was sent and whether or not a robot is listening. It is never evidence that the robot received anything. The line along the foot is.

## 6.24  Looking after the tablet

| Matter | What to do |
| --- | --- |
| Battery | Recognition and the camera use a good deal of power. On a long session, keep it on charge |
| Heat | With faces, objects and listening all on at once the tablet gets warm and the picture may slow down. Switch off whichever of the three you are not using |
| Storage | Recordings are the only thing that grows. Nothing deletes them but you |
| The faces you enrolled | They live on this tablet and nowhere else. A tablet that is wiped or replaced starts with an empty register |
| Cleaning | Be careful of the camera lens; a smudged lens looks exactly like a bad recognition model |
| Updating | Installing a newer build over the top keeps everything |
| Reporting a fault | The build number from Settings, what you did, what the line along the foot said, and whether the robot's console showed anything |

## 6.25  Working safely

The tablet drives a machine that moves. Very little of this is about the software.

Know where Stop is before you press anything else. It is the last button in the Move Control group, and it is also the word “stop”.

A direction button keeps going. It is not a nudge: it repeats until you stop it, or until the link drops.

If the link drops while the robot is moving, the robot should stop by itself. Do not rely on that alone — stay where you can reach the robot's own power.

Keep the floor clear in front of the robot before you press Forward, and behind it before you press Backward: it cannot see behind itself.

Do not drive it out of sight. The tablet shows you what it was told, not what it hit.

Shutdown cannot be undone from the tablet. You will be asked to confirm; read the question.

When you have finished, stop the robot, then switch off listening and the camera — or press stop on the notification, which does all of it at once.

## 6.26  Things people ask

| Question | Answer |
| --- | --- |
| Can I use it without a robot? | Yes. Every screen works, recognition works, and recording works. Commands will tell you plainly that no robot received them |
| Why does it need the internet? | It does not. Recognition runs on the tablet, and there is no network connection in the application at all |
| Can I read the screens in English and speak Korean? | Yes. They are two separate settings |
| Why is there no sound on my recordings? | So that the microphone stays free and the robot still answers you while you record |
| Why did it hear me but do nothing? | Either it matched no command — it will say Unknown Command — or it matched one and could not send it. The line along the foot says which |
| Why does the camera prompt not come back after I refused it? | It is asked once on purpose. Allow it in the tablet's application settings when you want it |
| Can two people use two tablets on one robot? | Not over this link. One tablet holds the Bluetooth connection at a time |
| Where did my faces go after the tablet was replaced? | They were on the old tablet. There is no copy |
| How far away can I be? | As far as Bluetooth reaches in that room, which is usually a good deal less than it is in an empty corridor |
| It says the command was sent, but nothing happened. | Then the tablet did its part. Section 6.23 tells you how to find out which side the fault is on |

## 6.27  Enrolling someone, step by step

The tablet can only name people it has been shown. Enrolling takes about a minute a person, and it is worth doing in the light the robot will actually work in.

Open Face Recognition and make sure detection is on. A box should follow each face in the picture.

Have the person stand about a metre from the camera, facing it, with nothing over their face.

Press the add button in the Face Database panel.

Hold still while it collects. It takes several samples and averages them, which is why it is worth a few seconds of stillness.

Fill in the name, the identifier, the department and the position. The name is what appears when they are recognised.

Save. The person appears in the register with their photograph.

Check it: step out of the picture, then back in. The result panel should give their name and a similarity figure.

| If | Then |
| --- | --- |
| Nobody is boxed at all | Detection is off, or the camera permission was refused. The message in the panel says which |
| They are boxed but not named | They are not enrolled, or the threshold is set too high. The threshold is in the settings panel on that screen |
| The wrong name comes up | Two enrolments are too close. Remove one — press and hold the row — and enrol it again in better light |
| The similarity figure is low but right | Enrol them a second time in the light they are usually seen in |
| Someone leaves | Press and hold their row in the register to remove them. It is removed from the tablet and there is no copy elsewhere |

## 6.28  Object detection in use

Switch it on from the Object Detection screen, or with the Objects button in the AI group, or by saying “object detect”. Boxes and labels appear over whatever it knows: people, furniture, vehicles, everyday things.

| Control | What it is for |
| --- | --- |
| Detection | The whole thing on or off |
| Boxes | Draw the outlines |
| Labels | Name what was found, with how sure it is |
| Masks | Shade the shape as well as the box; heavier, and slower |
| Tracking | Keep the same number on a thing as it moves, rather than renumbering it each frame |
| Confidence threshold | How sure it must be before it draws anything. Raise it if the picture fills with doubtful boxes; lower it if things are being missed |

The result panel lists what is in view with counts, and the time each frame took. If that time climbs, switch off whichever of face recognition, object detection and listening you are not using: the three of them share one camera and one processor.

## 6.29  The Remote Control screen

This is where the link is made, and where the robot's own camera and recording are worked. Everything to do with connecting lives here.

| Panel | What you do with it |
| --- | --- |
| Bluetooth Connection | Scan, pick the robot, watch it connect, and disconnect when you are done |
| Camera View | The robot's own cameras, front, rear, left and right, with a photograph button and full screen |
| Robot Status | Mode, speed, battery, temperature, how it is connected, where it thinks it is, and which way it faces |
| Map View | The map in 2D or 3D, with zoom and a button to find the robot |
| Camera and Recording | Ask the robot for a photograph, a video or an audio recording, and see what has been captured |
| Operation Log | Every command sent and everything the robot sent back, with times. This is the record to read when something did not work |

If the Devices Found list stays empty: check the robot is powered and advertising, check Bluetooth is on in the tablet's own settings, and stand closer. The scan stops by itself after twelve seconds, so press Scan again rather than waiting.

## 6.30  Driving with a gamepad

If a gamepad is attached to the tablet, it drives the robot from any screen. The tablet says so when the pad is connected.

| Control | What it does |
| --- | --- |
| Left stick | Drive: the robot moves in the direction you push, and stops when the stick centres |
| Right stick | Turn |
| D-pad up | Forward |
| D-pad down | Backward |
| D-pad left | Turn left |
| D-pad right | Turn right |
| B | Stop |
| A | Go home |
| X | Wave |
| Y | Dance |

The sticks and the buttons send the same commands as the screen, so a pad is a convenience rather than a separate mode. Stop is on B, and it works from any screen.

## 6.31  Two minutes before a demonstration

Worth doing every time, in this order. It takes two minutes and it catches the things that are embarrassing in front of people.

Charge the tablet, or have it on charge.

Power the robot and give it time to advertise.

Connect, and watch for Connected with the robot's name — not merely a connected indicator.

Press Stop, and check the line along the foot says the command was sent over Bluetooth.

Press Forward, then Stop. Watch the robot, not the screen.

Switch listening on and say one command. Listen for the acknowledgement.

Set the interface language to whatever your audience reads.

Delete old recordings if you are going to record.

Decide where you will stand: within Bluetooth range, and where you can reach the robot's own power.

## 6.32  Words used in this manual

| Word | What it means here |
| --- | --- |
| The link | The Bluetooth connection between this tablet and the robot |
| Connected | The link is up and the robot can answer. The name of the robot is shown beside it |
| The demonstration link | An older address connection, used to show the screens working with no robot present. It reaches no machine |
| Latched | A direction button that stays on: the robot keeps going until you stop it |
| A switch | A button that stays on until pressed again: Face, Objects, Voice, Follow Me and Recording |
| The sidebar | Two different things: the narrow strip down the left that moves between screens, and the Actions strip down the right of Robot Control |
| The line along the foot | The status line at the bottom of Robot Control, which says what just happened |
| The log | A panel that keeps those lines with their times |
| Enrol | Teach the tablet a person's face so that it can name them |
| Unknown Command | It heard you and matched nothing you can ask for |

## 6.33  Getting it to hear you better

Most trouble with spoken commands is the room rather than the tablet. Try these in order; the first two fix nearly everything.

Stand closer. Two metres in a quiet room is comfortable; across a workshop is not.

Say the command on its own. “Forward” works better than a sentence with forward somewhere in it.

Check which language it is listening for. Speaking Korean to a tablet set to English will produce nothing, or nonsense.

Switch noise suppression on, in Advanced settings on the Voice Recognition screen.

Raise the sensitivity if it is not noticing that you have spoken, and lower it if it keeps waking up on other people.

Give it a moment after a screen change: if the model is still loading, the line says so.

Switch off face recognition and object detection while you test. Three things at once slows everything, including this.

| Panel on the Voice screen | What it is for |
| --- | --- |
| Listening | The microphone button, the live waveform, and whether it is listening |
| Recognition result | What it heard, in which language, how sure it was, and when |
| Matched action | Which of the sixteen it took that to be, and what it does. A tap on Execute runs it |
| Voice commands | Every command with an example phrase. Tapping one runs it exactly as though you had said it — the quickest way to check the robot without speaking |
| Voice control | Command control, continuous listening, and noise suppression |
| History | What was said, when, and what came of it. Useful after a session that did not go well |
| Advanced settings | Three tabs: the language and how it is detected, the sensitivity and timeout, and the audio settings. Press Apply |

The command list is worth knowing about for another reason: it is the one place that shows you every phrase the tablet will answer to, in the language you are speaking.

## 6.34  Where to go for what

An index by the thing you want to do rather than by the screen it lives on.

| To do this | Go here |
| --- | --- |
| Connect to the robot | Remote Control, Bluetooth Connection panel |
| Disconnect | The same panel, Disconnect |
| See whether you are connected | The indicator at the top of any screen |
| Drive the robot | Robot Control, Move Control group |
| Stop the robot | Robot Control, Stop — or say “stop”, or press B on a gamepad |
| Make it wave, greet, pose or dance | Robot Control, Preset Pose group |
| Choose an arm | Robot Control, Arm Control group |
| Send it home | Robot Control, Quick Actions, Go Home |
| Have it follow you | Robot Control, Quick Actions, Follow Me |
| Switch listening on | Robot Control, AI group, Voice |
| See what it heard | Voice Recognition, Recognition result |
| Change the language you speak | Voice Recognition, Advanced settings — or Settings |
| Change the language you read | Settings, Interface language |
| Record what the camera sees | The red button at the top right of the camera picture |
| Watch a recording | The Open icon beside it |
| Enrol a face | Face Recognition, Face Database panel, add |
| Remove a face | The same panel: press and hold the row |
| Label what the camera sees | Object Detection, or the Objects button in the AI group |
| Build or load a map | LiDAR SLAM, Control panel — or Load Map in the corner of the LiDAR panel on Robot Control |
| Change the camera or the frame size | Settings, Camera |
| Find the build number | Settings, System information |
| See what was sent and what came back | Remote Control, Operation Log — or the Log panel on Robot Control |
| Stop everything at once | The notification: press stop |

# Appendix A  Source map

Where each part of this specification lives in the repository.

| Area | Path |
| --- | --- |
| Screens | app/src/main/java/com/falcon/robot/*Activity.java |
| Shared screen behaviour | app/src/main/java/com/falcon/robot/BaseActivity.java |
| Background service | app/src/main/java/com/falcon/robot/RobotService.java |
| Link state | app/src/main/java/com/falcon/robot/RobotSession.java |
| Bluetooth client | app/src/main/java/com/falcon/robot/ble/BleLink.java |
| Spoken commands | app/src/main/java/com/falcon/robot/voice/VoiceCommands.java |
| Speech engines | app/src/main/java/com/falcon/robot/voice/ |
| Face recognition | app/src/main/java/com/falcon/robot/face/ |
| Object detection | app/src/main/java/com/falcon/robot/detect/ |
| Custom views | app/src/main/java/com/falcon/robot/widget/ |
| Screen layouts | app/src/main/res/layout/ |
| Strings and translations | app/src/main/res/values{,-ko,-ja,-zh}/strings.xml |
| Styles, colours, dimensions | app/src/main/res/values/ |
| Models and sounds | app/src/main/assets/ |
| Model tooling | tools/train_commands.py, tools/train_moonshine.py |
| Build | app/build.gradle, app/src/main/AndroidManifest.xml |

# Appendix B  Building the application

What someone needs in order to produce the package this specification describes.

| Step | Detail |
| --- | --- |
| Tools | Android Studio or the command-line SDK, with platform 34 and CMake 3.22.1 for the optional native engine |
| Clone | The repository, with app/, tools/ and the readme |
| Models | app/src/main/assets must hold mobilefacenet.tflite, yolo26n.tflite, ok.wav and the moonshine folders. The readme says where each comes from and which are committed |
| Optional engine | For the whisper fallback, clone whisper.cpp into app/src/main/cpp/whisper.cpp; without it the application says the engine was not built and uses the other models |
| Build | gradlew assembleDebug, which writes falcon.ai.robot.apk |
| Install | adb install -r on the tablet, or copy the package to it |
| Before offering a build for test | Run the static check of Document 3, section 3.17 |

# Appendix C  Screen layouts

| Layout | Screen it belongs to |
| --- | --- |
| activity_shell.xml | The frame every screen is drawn into, with the navigation sidebar |
| activity_splash.xml | The opening screen |
| activity_main.xml | Home |
| activity_robot_control.xml | Robot Control |
| activity_remote_control.xml | Remote Control |
| activity_face_recognition.xml | Face Recognition |
| activity_voice_recognition.xml | Voice Recognition |
| activity_object_detection.xml | Object Detection |
| activity_lidar_slam.xml | LiDAR SLAM |
| activity_settings.xml | Settings |
| view_page_header.xml, view_page_header_rich.xml | Page headers |
| view_system_status.xml | The tablet's own status row on Home |
| dialog_connection.xml | The connection dialog |
| dialog_camera_settings.xml | Camera settings from Settings |
| dialog_add_face.xml | Enrolling a face |
| item_home_card.xml, item_quick_status.xml | Home rows |
| item_settings_row.xml, item_setting_dropdown_row.xml | Settings rows |
| item_face_db_row.xml, item_face_history_row.xml | Face rows |
| item_voice_history_row.xml, item_voice_toggle_row.xml | Voice rows |
| item_slam_status_row.xml | LiDAR status rows |
| item_spinner.xml, item_spinner_dropdown.xml | Dropdown appearance |

# Appendix D  Glossary

| Term | Meaning |
| --- | --- |
| BLE | Bluetooth Low Energy: the radio link to the robot |
| GATT | The protocol BLE uses for services and characteristics |
| NUS | Nordic UART Service: a serial line over BLE, the shape the robot's service takes |
| MTU | Maximum transmission unit: how many bytes one BLE write carries |
| CameraX | The Android library that opens the camera and delivers frames |
| Use case | One job the camera does at a time: preview, analysis or video |
| dp | Density-independent pixel: the unit layouts are measured in |
| COCO | The eighty everyday classes the object model was trained on |
| Embedding | The vector that stands for a face, compared to recognise it |
| Keyword spotting | Recognising a fixed set of words without transcribing speech |
| Foreground service | A background component Android keeps alive, with a notification |
| Latch | A direction button that stays on, its command repeated every second |
| Demonstration link | The address connection that exercises the screens and reaches no robot |